LABEL CHECKLIST

PC Code:									Date:

Chemical Name:								Reg. No.:

Minimum Information Needed from Labels for Conducting Human Health Risk Assessments, Ecological Risk Assessments, and/or Drinking Water Exposure Assessments

Yes	No

1.  The label uses enforceable language, such as, required instead of recommended whenever possible.

Yes	No

2.  The label indicates that the product is intended for use by ‘homeowners’.

Yes	No

3.  The label indicates that the product is intended for use by commercial/licensed/professional applicators.

Comment(s):

A. Labels for Products with Applications to Terrestrial Environments:

Yes	No

1.  The label clearly states the formulation type (e.g., soluble concentrate, emulsifiable concentrate, water soluble granule, powder, wettable powder, granular, pellets)

Yes	No

2.  The application rate is provided in pounds a.i./acre (if this is not possible, then we need enough information to convert the rate to lb a.i./acre; see examples below); for multi-a.i. products, ensure that the maximum rate (in lb a.i./acre) is provided for each a.i.

    - Examples:

      - If the application rate is provided in lb a.i./1,000 ft row, provide minimum row spacing.  For things like container-grown ornamentals or per plant/tree applications, maximum containers/plants/trees per acre.

      - For bait stations specify how many bait stations can be placed in an acre  (or specified area that can be converted to an acre rate)

      - For ant mound treatments, specify how many mounds per acre (or specified area) can be treated

      - For spot treatments, specify a lb a.i./acre limit

Comment(s) (Question 1 – 2):

Yes	No

3.  The single maximum application rate allowed (e.g., maximum lbs a.i./acre) is provided on the label.

Yes	No

4.  The maximum annual and seasonal application rate [per a.i., not per product; in other words, how much of the a.i., accounting for all the formulations that could be used, can be applied to an acre per year and per season (if applicable)?] is provided on the label.

Yes	No

5.  The number of applications allowed per year (NOT season) is provided on the label.

Yes	No

6.  The application rates are consistent on the label (e.g., when an application rate is provided in ‘lb a.i,/acre’ and fl oz of product per acre, make sure the calculations are correct so that the application rates are consistent).

Yes	No

7.  The minimum reapplication interval in days (for those pesticides that can be applied more than once per year) is provided on the label.

Yes	No

8.  The lb a.i. per each gallon of product is provided on the label.

Comment(s) (Question 3 – 8):

Yes	No

9.  The application method (e.g., handheld or backpack ground sprayer, ground boom sprayer, aerial sprayer, banded, chemigation, airblast) is provided on the label.

Yes	No

10.  For incorporated pesticides, the minimum depth of incorporation is specified on the label.

Yes	No

11.  If applicable, are geographic restrictions clear on the label (e.g., the label does not use general geographic terms like ‘not for use in the Southeastern United States’)?

Comment(s) (Questions 9 – 11):

12.  For seed treatments, is the following information provided on the label:

Yes	No

a. Number of seeds (treated seeds, if incapsulated) per lb

Yes	No

b. Number of treated seeds per acre or lb of treated seed per acre

Yes	No

c. Amount of a.i. per treated seed or cwt of seed

Yes	No

d. Minimum planting depth

Comment(s):

Yes	No

13.  Are the use sites clear and specific on the label, for example:

    - For turf, identify what type of turf (e.g., residential lawns; golf courses – tees, greens, fairways; athletic fields; sod farms).

    - For ornamentals, identify application site (e.g., greenhouse, residential, commercial landscaping, container-grown, field-grown).

    - For things like rights-of-way, identify which types (e.g., roadways, power lines, railroads, etc.).

    - For forestry uses, specify the site (e.g., Christmas tree, commercial pulp/paper or lumber plantation, BLM land).  For applications to federal lands (National forest, National park, BLM lands, etc.) specify who has authority to apply the pesticide.

    - Overly broad terms, such as, ‘non-crop areas’ and ‘agricultural field’ are not used.

Comment(s):

Yes	No

14.  Is spray drift language on the label consistent with the recent FR notice including:

Yes	No

- Is the minimum ASABE Drop Size Distribution specified on the label?

Yes	No

- For aerial applications, is the maximum windspeed and maximum release height specified on the label?

Comment(s):

Yes	No

15.  Is the pre-harvest interval (PHI) provided on the label?

Yes	No

-  If ‘No’, is the description of growth stage(s) for application provided?

Yes	No

16.  Are there rotational crop restrictions stipulated on the label?

Yes	No

17.  Is information on whether or not adjuvants and/or surfactants could be used with the product(s) provided on the label?

Comment(s):

Yes	No

18.  Is all of the above information provided for each use/use site?

Comment(s):

B. Labels for Products with Applications Directly to Water:

Yes	No

1.  Is all of the information identified above for terrestrial uses also specified on the label?

Yes	No

2.  Is the maximum target concentration in water specified on the label (specify if there is a requirement to test the water body for stratification to calculate the application rate)?

Yes	No

3.  Is the length of time the target concentration in water should be maintained specified on the label?

Yes	No

4.  Is the specific application method(s) specified on the label (e.g., applied by hose under water surface)?

Yes	No

5.  If applicable (e.g., for applications to rice), is the period of time the water is to be retained after treatment before being released into receiving waters specified on the label?

Yes	No

6.  Is all of the above information provided for each use/use site?

Comment(s) (Questions 1 – 5):

C. Labels for Products with Indoor Applications (Industrial/Commercial Scale):

Yes	No

1.  Does the label specify the waste water disposal requirements?

Yes	No

2.  Does the label specify the application rate (e.g., number of application spots per unit area)?

Yes	No

3.  Does the label specify the number of applications allowed per year?

Yes	No

4.  Does the label provide adequate preparation instructions (e.g., cover/remove food before use)?

Comment(s):

USEFUL INFORMATION INCLUDES:

- For granular uses, in addition to all of the information listed above, the weight of one granule and the amount of a.i. per granule would be useful.

Comment(s):