Abandoned and inactive mines (AIMS) database

Version

2

#### Region 3 and 10

# DATA DICTIONARY

# DATA DICTIONARY

USDA Forest Service

Southwestern Region

517 Gold Avenue, SW

Albuquerque, New Mexico 87102-0084

Phone 505.842.3829

Revised December 1999

#### Introduction

Data Item and Terminology Definitions:

These definitions are written for the person collecting basic inventory data to describe what information to record on the inventory form.  It also describes the entry format for the database designer and data entry technician, identifies the corresponding name(s)of the data field(s) in other databases, and identifies potential terminology and/or data type conflicts.

Definition components:

Description:   A plain English explanation of a term or data field. Ideally, it will not use the term to describe the term.

Required/Optional: Only data fields, which have a corresponding, field in the old FS Region 3, FS Region 1, and Bureau of Mines (BOM) databases, will be considered eligible to be "REQUIRED" fields.  All others will be "OPTIONAL".  This is for the purpose of meshing the databases at some time in the future.  It DOES NOT mean "OPTIONAL" fields may be left blank on the inventory form.

##### Source:  The source where the description has been taken from.

Comments/Corrections should be sent to the database manager.

#### GENERAL DEFINITION

MINE SITE:

- Description: A location where minerals (gold, copper, lead, uranium, etc.) and common materials (sand gravel, pumice, limestone, rock, etc.) have been extracted from the earth.  The site may contain buildings, equipment, mill facilities, or foundations.  Other features may include exploration pits or drill holes, small adits and/or shafts with dump and waste rock, open trenches, large open pits and quarries with large waste piles.

Source [Dennis Inman, Region 3, Minerals Staff, 6/21/96]

- MINE SITE NUMBER:

- The unique site numbers contain Region, Forest, District, and four-digit unique site number.

a. REGION CODE:

Description:  A two-digit code representing the Forest Service Region.

Required/Optional: REQUIRED

Source: [R3 GIS Data Dictionary, Road Features, page 20]

b. FOREST CODE:

Description: A two-digit code, which combined with the Region code, uniquely identifies a National Forest.

Required/Optional: REQUIRED

Source: [R3 GIS Data Dictionary, Road Features, page 15]

c. DISTRICT CODE:

Description:  A two-digit code, which with a Region and Forest uniquely identifies a Forest Service Ranger District.

Required/Optional: REQUIRED

Source:  [R3 GIS Data Dictionary, Road Features, page 14]

d. UNIQUE NUMBER:

Description: An identifying four-digit number the data collector assigns starting with 0001, 0002, etc.  Each number is used once on each forest.

Required/Optional: REQUIRED

Source: Decision made by Kathleen Adam, Dennis Inman, Penny Leuhring, and Steve Hall, 6/12/96

2. HISTORICAL NAME:

Description: Any long term previously used name/names for the site.

Required/Optional: REQUIRED

Source: BOM [MILS Guide]

3. OTHER NAME/REFERENCE:

Description: Any recorded name that is found in the records.

Required/Optional: OPTIONAL

Source: BOM [MILS Guide]

4. DATE SURVEY:

Description: The date the survey was taken in the field.

Required/Optional: Required

Source: BOM [MILS Guide]

5. MILS NUMBER:

Description: A 10-character identification system adopted by the US Bureau of Mines known as "MILS" (Mineral Industry Location System).

MILS record number format:

Valid Values

| 1 2 3 | 4 5 6 | 7 8 9 10 |
| --- | --- | --- |
| X X X | X X X | X X X X |

Position 1:3: State/Nation (AZ Code = 004)

Position 4:6: County/Political Subdivision

Position 7:10: Deposit reference, sequence #

###### If no MILS record exists for this location, this field will be left blank

Required/Optional: OPTIONAL

Source: [Arizona State Land Department letter titled Arizona Mine Locations-Data Base Design, dated 10/17/95]

6. FEDPLAN ID:

Description: The Regional Office assigns The Federal Facility Identification number when a project is proposed.  Each Region, National Forest and Research Station will have a unique FFID number.

Required/Optional: Optional

Source: FEDPLAN Update Instructions

7. ID:

Description: Unique number that Region 10 set-up when they visited sites.

Required/Optional: Optional

Source: Region 10

8. STATE CODE:

Description: A unique two character alphanumeric abbreviation for one of the first order political divisions of the United States

Required/Optional: REQUIRED

Valid Value

| State | State Name |
| --- | --- |
| AZ | Arizona |
| NM | New Mexico |
| OK | Oklahoma |
| TX | Texas |
| AK | Alaska |

Source:  [R3 GIS Data dictionary from FIPS PUB 5:1 State and State Equivalents of the United States]

9. COUNTY CODE:

Description: An entity (also a geographic feature), which represents the largest territorial division for local government within a state.

Required/Optional: REQUIRED

Valid Values

| County Code | County Name |  | State |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 001 | Apache |  | Arizona |  |  |  |
| 003 | Cochise |  |  |  |  |  |
| 005 | Coconino |  |  |  |  |  |
| 007 | Gila |  |  |  |  |  |
| 009 | Graham |  |  |  |  |  |
| 011 | Greenlee |  |  |  |  |  |
| 013 | Maricopa |  |  |  |  |  |
| 015 | Mohave |  |  |  |  |  |
| 017 | Navajo |  |  |  |  |  |
| 019 | Pima |  |  |  |  |  |
| 021 | Pinal |  |  |  |  |  |
| 023 | Santa Cruz |  |  |  |  |  |
| 025 | Yavapai |  |  |  |  |  |
| County Code |  | County Name | County Code |  | County Name | State |
| 001 |  | Bernallillo | 033 |  | Mora | New Mexico |
| 003 |  | Catron | 035 |  | Otero |  |
| 005 |  | Chaves | 039 |  | Rio Arriba |  |
| 006 |  | Cibola | 043 |  | Sandoval |  |
| 007 |  | Colfax | 047 |  | San Miguel |  |
| 015 |  | Eddy | 049 |  | Santa Fe |  |
| 017 |  | Grant | 051 |  | Sierra |  |
| 021 |  | Harding | 053 |  | Socorro |  |
| 023 |  | Hildago | 055 |  | Taos |  |
| 027 |  | Lincoln | 057 |  | Torrenece |  |
| 028 |  | Los Alamos | 059 |  | Union |  |
| 031 |  | McKinley | 061 |  | Valencia |  |
| 025 |  | Cimmarron |  |  |  |  |
| County Code |  | County Name | State |  |  |  |
| 129 |  | Roger Mills | Oklahoma |  |  |  |
| County Code |  | County Name | State |  |  |  |
| 111 |  | Dallas | Texas |  |  |  |
| 179 |  | Gray |  |  |  |  |
| 211 |  | Hemphill |  |  |  |  |
| County Code |  | County Name | State |  |  |  |
|  |  |  | Alaska |  |  |  |

Source: [R3 GIS Data dictionary from FIPS PUB 6:2 Counties and County Equivalents of the States of the United States and the District of Columbia]

10. ELEVATION:

Description: Vertical distance from a datum, usually mean sea level, to a point of an object on the earth’s' surface.

Required/Optional: REQUIRED

Source: [Common usage and R3 GIS Data Dictionary, Topography, page (5)]

11. QUAD NAME:

Description: The name of the 7.5-minute quadrangle largest scale map on which the property is located.

Required/Optional: REQUIRED

Source: [BOM Hazard Evaluation Handbook, page 15, 5/94]

12. PRINCIPAL MERIDIAN:

Description: Line extending north and south on the surface of the earth between the two poles, and runs along the astronomical meridian.  The principal meridian is the line from which the survey township boundaries along the parallels are initiated, and along which the township, section, quarter, and section corners are established.

Required/Optional: REQUIRED

Source:  [R3 GIS Data Dictionary, Land Survey Theme, page (7).  Definitions of Surveying and Associated Terms prepared by a joint standard committee of the American Congress on Surveying and Mapping and the American Society of Civil Engineers. Source for codes: Lands LOS User's Guide]

13. TOWNSHIP:

Description: A public land surveying unit.  An approximately square area about six miles on a side with boundaries conforming to meridians and parallels within established limits.  It is subdivided into 36 sections, some of which are designed to take up the convergence of the Meridians township boundary lines and accumulated measurement errors.  The word township is used in conjunction with the appropriate range to indicate the coordinates of a particular township with references to the initial point.

Required/Optional: REQUIRED

Source: [R3 GIS Data Dictionary, Land Survey Theme, page 15.  Definitions of Surveying and Associated Terms prepared by a joint standard committee of the American Congress on Surveying and Mapping and the American Society of Civil Engineers.]

14. RANGE:

Description: Any series of contiguous townships situated north and south of each other; also, sections similarly situated within a township.  Ranges of townships are numbered consecutively east and west from a principal meridian: thus "range 3 east" indicates the third range or row of townships to the east from a principal meridian.  The word "range" is used in conjunction with the appropriate township with reference to the initial point; thus "township 14 north, range 3 east" indicates the particular township which is the 14th township north of the base-line an the 3rd township east of the principal meridian.

Required/Optional: REQUIRED

Source: [R3 GIS Data Dictionary, Land Survey Theme, page 10.  Definitions of Surveying and Associated Terms prepared by a joint standard committee of the American Congress on Surveying and Mapping and the American Society of Civil Engineers.]

15. SECTION:

Description: The unit of subdivision of a township; normally a quadrangle 1 mile square, with boundaries conforming to the meridians and parallels within established limits, and containing 640 acres as nearly as possible.

Required/Optional: REQUIRED

Source: [R3 GIS Data Dictionary, Land Survey Theme, page 10.  Definitions of Surveying and Associated Terms prepared by a joint standard committee of the American Congress on Surveying and Mapping and the American Society of Civil Engineers.]

16. ALQ:

Description:  Aliquot: A fractional part of a Section.

Required/Optional: OPTIONAL

Source: [Webster's Ninth New Collegiate Dictionary, 1990]

17. & 18. COORDINATES:

Description: Location information on the mine site, recorded as either Latitude/Longitude or UTM: North/UTM: East. Latitude is the angular distance measured in degrees north or south of the equator, recorded in degrees, minutes and seconds.  Longitude is the arc or portion of the earth’s equator intersected between the meridian of a given place and the prime meridian, recorded in degrees, minutes and seconds. Universal Transverse Macerator (UTM) is one of sixty zones dividing the globe each spanning 6-degree of longitude.  Each zone has its own principal meridian. UTM: North (also YUTM) is the coordinate (in meters) of the UTM zone, and UTM-East (XUTM) is the coordinate (in meters) of the UTM zone.

Required/Optional: REQUIRED

Source: [AZ State Land Department letter, AZ Mine Locations: Data Base Design, 10/17/95]

19. ZONE:

Description: Sixty north - south zones are used.  Each one covers six degrees of longitude with an overlap of 30 minutes with the zones on either side

Required/Optional: REQUIRED

Sources: [Map Use] by Phillips C. Muekrcke

20. FEATURE GPS:

Description:  Indicate feature number associated with Global Positioning System location.

Required/Optional: OPTIONAL

Source: [Webster's Ninth New Collegiate Dictionary, 1990]

21. COMMENTS:

Description: An observation or remark expressing an opinion.

Required/Optional: OPTIONAL

Source: [Webster's Ninth New Collegiate Dictionary, 1990]

22. COMMODITIES:

Description: A mineral resource with sufficient value to be individually produced and traded on the open market.

Required/Optional: OPTIONAL

Source: [U.S. Bureau of Mines Mineral Commodity Summaries]

23. SKETCH:

Description:  Rough draft or outline of the site (when available).

Required/Optional: Optional

Source: [Webster's Ninth New Collegiate Dictionary, 1990]

24. LAND OWNERSHIP:

Description: The classification of federal administration, private, local, or state holdings on the surface estate.

Required/Optional: REQUIRED

### Valid Values

| Ownership Code | Ownership Name |
| --- | --- |
| BLM- | Bureau of Land Management |
| FS | Forest Service |
| IND | Indian Reservation |
| MIL | Military |
| ST | State |
| NP | National Parks |
| PRI | Private |
| UN | Unknown |

Source: [R3 GIS Data dictionary has a definition in the Ownership and Jurisdiction section, page 37 for a more complicated term, both surface and subsurface ownership involved]

25. PRIVATE ONLY:

Description:  See 21.  "Land Ownership" and/or 22.  "Claim Ownership".

Required/Optional: OPTIONAL

Valid Values

| Visually apparent | Likely to be significant | Unlikely or minimal |
| --- | --- | --- |

Source: [Webster's Ninth New Collegiate Dictionary, 1990]

26. MINING DISTRICT:

Description: An area or region characterized by the occurrence of specific mineral suites or the nature of mineral deposits.

Required/Optional: OPTIONAL

Source: [BOM, Field Safety Guide, 1993]

27. DESIGNATED WILDERNESS:

Description: The land units included in the National Wilderness Preservation System that were authorized and designated by the Wilderness Act of 1964 or subsequent Acts of Congress.

Required/Optional: OPTIONAL

Valid Values

| Yes | No | Unknown |
| --- | --- | --- |

Source: [The Wilderness Act Handbook, published by the Wilderness Society, Washington, DC, 1984]

28. MINE STATUS:

Description:  Mine status codes.

Required/Optional: REQUIRED

## Valid Values

| AB | Abandoned |
| --- | --- |
| AC | Active |
| AB or IN | Abandoned or Inactive |
| IN | Inactive |
| UN | Unknown |

Source:  [Arizona Mine Locations 10/17/95]

29. SITE VISIT:

Description:  To officially inspect the place, scene, or point of something.

Required/Optional: OPTIONAL

Source: [Webster's Ninth New Collegiate Dictionary, 1990]

30. RESIDENCE:

Description:  The distance to a building used solely as a housekeeping unit with cooking, living, sanitary, and sleeping facilities.

Required/Optional: OPTIONAL

Source: [FSH 7309.11]

31. OCCUPIED:

Description:  Resided in as an owner or tenant.

Required/Optional: OPTIONAL

Source: [Webster's Ninth New Collegiate Dictionary, 1990]

32. ROAD:

Description: A way for purposes of travel.

Required/Optional: OPTIONAL

Source: [FSH 7709.11]

33. TRAIL:

Description: A blazoned path or beaten track, as through woods or wilderness.

Required/Optional: Optional

Source: [R3 GIS Data Dictionary, Trails, page (2)]

34. RECREATION:

Description: When some activities like camping, day use, or motorcycle uses are easily identified at the site.  Also described in terms of high, moderate, or low activity levels.

Required/Optional: OPTIONAL

Source: [Dennis Inman (with communication to recreation, Botts), 6/21/97]

35. USAGE:

Description:  The action, amount, or mode of using.

Required/Optional: OPTIONAL

Valid Values

| High | Moderate | Low |
| --- | --- | --- |

Source: [Webster's Ninth New Collegiate Dictionary, 1990]

36. TYPE:

Description:  A particular kind, class, or group.  For example campsite, trailhead, picnic grounds etc.

Required/Optional: OPTIONAL

Source: [Webster's Ninth New Collegiate Dictionary, 1990]

37. NONE:

Description: No data on field form.

Required/Optional: OPTIONAL

Source: [Webster's Ninth New Collegiate Dictionary, 1990]

38. T&E:

Description: Any species in danger of extinction throughout all or a significant portion of its range.  This does not include species of the class Insect determined by the Secretary to be a pest whose protection under the provisions of the Endangered Species Act would be an overwhelming and overriding risk to humans. Species that is likely to become an endangered species within the near future throughout all or a significant portion of its range, and that the appropriate Secretary has designated as a threatened species.  (Some states also have declared certain species as threatened through their regulations or statutes)

Required/Optional: OPTIONAL

Source: [FSM 2670.5:8] and [FSM 2670.5:21

39. FISHERIES:

Description: A water body where fish are used for commodity & non-community purposes.

Required/Optional: OPTIONAL

Source: [Jim Cooper, 06/26/96 with Webster's Ninth Collegiate Dictionary, 1990]

40. WETLANDS:

Description: Those areas inundated by surface or ground water with a frequency sufficient to support, and under normal circumstances do or would support, prevalence of vegetation or aquatic life that requires saturated or seasonally saturated soil conditions for growth and reproduction.  Wetlands generally include swamps, marshes, bogs, and similar areas such as sloughs, potholes, wet meadows, river overflows, mud flats, and natural ponds.

Required/Optional: OPTIONAL

Source: [FSM 2527.05]

41. NONE:

Description: No data on field form.

Required/Optional: OPTIONAL

Source:  [Webster's Ninth New Collegiate Dictionary, 1990]

42. OTHER:

Description: Does not fit one of the above definitions.

Required/Optional: Optional

Source: Laura M. Shepherd 5/26/98

43. GUANO:

Description: A substance composed chiefly of the excrement of bats.

Required/Optional: Optional

Source: [Webster's Ninth New Collegiate Dictionary, 1990]

44. INSECTS REMAINS:

Description: Insect remains located in the feature or on the site.

Required/Optional: Optional

Source: Laura M. Shepherd and Kathleen Adam 5/26/98

45. BAT SIGHTING:

Description: Evidence of bats sighted in area.

Required/Optional: Optional

Source: Laura M. Shepherd and Kathleen Adam 5/26/98

46. NO:

Description: There is no evidence of bats.

Required/Optional: Optional

Source: Laura M. Shepherd and Kathleen Adam 5/26/98

47. BAT OTHER:

Description: Does not fit one of the above definitions.

Required/Optional: Optional

Source: Laura M. Shepherd 5/26/98

48. BASE FLOODPLAIN:

Description: The lowland and relatively flat areas joining inland and coastal water including debris cones and flood-prone areas of offshore islands, including at a minimum that area subject to a 1 percent (100:year recurrences) or greater chance of flooding in any given year.

Required/Optional: OPTIONAL

Valid Values

| Yes | No | Unknown |
| --- | --- | --- |

Source: [FSM 2527.05]

49. MILL SITES TAILING:

Description: A mineral treatment plant in which crushing, grinding, and further processing of ore is conducted to produce a product.

Required/Optional: OPTIONAL

Valid Values

| Yes | No | Unknown |
| --- | --- | --- |

Source:  [BOM Hazard Evaluation Handbook, page 78, 5/94]

50. SIZE OF SITE:

Description: The contiguous area that includes all of the disturbed or impacted areas at the site.  Measure in acres.

Required/Optional: OPTIONAL

Source: Dennis Inman 6/21/96

51. SPRINGS:

Description:  Where the aquifer (or water table) intersects the surface of the earth.

Required/Optional: OPTIONAL

Valid Values

| Several | Numerous | One | None |
| --- | --- | --- | --- |

Source: [Hydrology]

52. RECEIVING STREAM NAME:

Description: A natural stream that conveys water.

Required/Optional: OPTIONAL

Source: [Resource Conservation Glossary, 3rd Edition, 1982]

53. FLOWING INTO:

Description: Stream into which receiving stream flows

Required/Optional: OPTIONAL

Source:  [Webster's Ninth New Collegiate Dictionary, 1990]

54. SURFACE WATER INTAKES:

Description: Surface water being used for drinking water. Includes spring box, infiltration galleries, shallow wells, or direct use.

Required/Optional: Optional

Valid Values

| Yes | No | Unknown |
| --- | --- | --- |

Source: [Roberta M. Baca 5/26/98]

55. PUBLIC WATER SYSTEM/MUNICIPAL:

Description: A system for the provision of piped water to the public for human consumption provided such a system has 15 service connections or regularly serves at least 25 people.

Required/Optional: OPTIONAL

Valid Values

| Yes | No | Unknown |
| --- | --- | --- |

Source: [42 USC 300f, Safe Drinking Water Act]

56. DOMESTIC WELL W/IN 4-MILE RADIUS:

Description: A source of water from a hole drilled into the earth that provides water for human consumption.

Required/Optional: OPTIONAL

Valid Values

| Yes | No | Unknown |
| --- | --- | --- |

Source: [Penny Leuhring, 6/25/96]

57. GROUNDWATER DEPTH:

Description:  The perpendicular measurement downward from ground surface to groundwater.

Required/Optional: OPTIONAL

Source: [Webster's Ninth New Collegiate Dictionary, 1990]

58. GROUNDWATER:

Description:  Water below the ground surface in a zone of saturation.

Required/Optional: OPTIONAL

Valid Values

| Shaft/Pit/Hole | Well | Wetland |
| --- | --- | --- |

Source: [The Hazardous Waste Q & A, Revised Edition, 1992]

59. EVIDENCE OF HAZMAT IMPACT:

Description: Hazmat impact indicated by water discoloration stressed vegetation, pH less than 4 or stained soil.

Required/Optional: OPTIONAL

Valid Values

| Yes | No | Unknown |
| --- | --- | --- |

Source: Kathleen S. Adam 5/26/98

60. DEAD OR STRESSED VEGETATION:

Description: Altered or changes in vegetation from surrounding live vegetation.

Required/Optional: OPTIONAL

Source: Kathleen S. Adam 5/26/98

61. SURFACE WATER DISCOLORATION:

Description: Alter or change the hue or color from upstream of the site.

Required/Optional: OPTIONAL

Source:  [Webster's Ninth New Collegiate Dictionary, 1990]

62. pH VALUE LESS THAN 4:

Description: Measure of the acid or base strength of a water solution.  Those numbers less than four are acidic.

Required/Optional: OPTIONAL

Valid Values

| Yes | No | Unknown |
| --- | --- | --- |

Source: [Principles of Hazardous Material Management]

63. STAINED SOIL:

Description: Alteration or change in the hue or color of soil from that in surrounding areas.

Required/Optional: OPTIONAL

Source:  [Webster's Ninth New Collegiate Dictionary, 1990]

64. OTHER:

Description: Does not fit one of the above definitions.

Required/Optional: OPTIONAL

Source: [Webster's Ninth New Collegiate Dictionary, 1990]

65. POTENTIAL HAZMAT ON SITE:

Description: An area causing a hazardous material impact that does not fit one of the above definitions.

Required/Optional: OPTIONAL

Valid Values

| Yes | No | Unknown |
| --- | --- | --- |

Source: [A-8 Attachments A&B to OSM Directive AML-1]

66. EXPLOSIVE:

Description: An explosive agent or substance, such as dynamite, present or potentially present on mine site.

Required/Optional: OPTIONAL

Source: [[Random House College Dictionary, 1984]

67. PETROLEUM:

Description: Products such as lubricating oil, hydraulic fluid, brake fluid, & grease, are essential to the operation of mine & mill equipment.

Required/Optional: OPTIONAL

Source: [BOM Hazard Evaluation Handbook, page 15, 5/94]

68. CHEMICAL/TRANSFORMER:

Description: Chemical: A substance produced by or used in a chemical process.  Transformer:  Device employing the principle of mutual induction to convert variations of current in a primary circuit into variation of voltage and current in a secondary circuit.

Required/Optional: OPTIONAL

Source: [Webster's Ninth New Collegiate Dictionary, 1990; Random House College Dictionary, 1984.]

69. CYANIDE/LEACHING OPERATION:

Description: Potential or observed cyanide or other leaching operations of site.

Required/Optional: OPTIONAL

Source: [Webster's Ninth New Collegiate Dictionary, 1990]

70. MINING EQUIPMENT:

Description: Equipment can include, but is not limited to, crushers, grinders, engines, boilers, flotation cells, screens, vats, trams, ore cars, conveyors, and bins.

Required/Optional: OPTIONAL

Source: [BOM Hazard Evaluation Handbook, page 16, 5/94]

71. TRASH DUMP:

Description: An accumulation of refuse and discarded materials.

Required/Optional: OPTIONAL

Source: [Webster's Ninth New Collegiate Dictionary, 1990]

72. OTHER MATERIAL:

Description: Does not fit one of the above definitions.

Required/Optional: OPTIONAL

Source: [Webster's Ninth New Collegiate Dictionary, 1990]

73. MINE/MILL WASTE SAMPLE:

Description:  A small part of the ore deposit, resulting from excavation for the extraction of ore or other economic minerals and/or a mineral treatment plant in which crushing, grinding, and further processing of ore is conducted to produce a product that is too low in grade to be of economic value at the time, selected for inspection or analysis.

Required/Optional: OPTIONAL

Source: [Webster's Ninth New Collegiate Dictionary, 1990]

74. SOIL SAMPLES:

Description:  A small part of the soil selected for inspection or analysis.

Required/Optional: OPTIONAL

Source: [Webster's Ninth New Collegiate Dictionary, 1990]

75. HAZARDOUS WASTES/MATERIAL:

Description:  A hazardous waste as defined in RCRA Section 261.3 of 40 CFR.  Hazardous material: A substance or material, including a hazardous substance, which has been determined by the Secretary of Transportation to be capable of posing an unreasonable risk to health, safety, and property when transported in commerce, and which has been so designated.

Required/Optional: OPTIONAL

Source: [Principles of Hazardous Materials Management, 1988]

FEATURE DEFINITION

76, 89., & 100. FEATURE NUMBER:

Description: Unique number for each feature at the site.

Required/Optional: REQUIRED

Source: Laura M. Shepherd 5/27/98

77., 90., & 109. FEATURE TYPES:

ADIT:

Description: A horizontal or nearly horizontal passage driven in rock from the surface for the working or dewatering of a mine.

Required/Optional: OPTIONAL

Source:  [BOM Hazard Evaluation Handbook, page 73, 5/94 Draft]

SHAFT:

Description: An excavation of limited area compared to its depth, made for access to underground mine workings.  Used for hoisting and lowering men and materials, or for ventilating underground workings.

Required/Optional: OPTIONAL

Source: [BOM Hazard Evaluation Handbook, page 81, 5/94 Draft]

STOPE:

Description: An underground excavation from which ore has been removed. Can be open to the surface, and, as such potentially dangerous.

Required/Optional: OPTIONAL

Source: [BOM Hazard Evaluation Handbook, page 81, 5/94 Draft]

GLORY HOLE:

Description: A funnel-shaped excavation the bottom of which is connected to a raise driven from an underground haulage level.

Required/Optional: OPTIONAL

Source: [BOM Hazard Evaluation Handbook, page 81, 5/94 Draft]

OPEN PIT:

Description: A form of operation designed to extract minerals that lie near the surface.

Required/Optional: OPTIONAL

Source: [BOM Hazard Evaluation Handbook, page 81, 5/94 Draft]

ADIT/WINZE:

Description:  An adit, which contains one or more winzes.  A winze is a vertical or inclined shaft driven downward from a drift or adit into a body of ore.  Can be filled with water and very dangerous.

Required/Optional: OPTIONAL

Source: [Random House College Dictionary, 1984.]

OTHER OPENING:

Description: Open slots or trenches, pits, or overhangs.

Required/Optional: OPTIONAL

Source: BOM [Field Safety Guide,] 1993

PROSPECT HOLE:

Description: Small (usually less then 5 m deep) pit excavated to explore for minerals

Required/Optional: OPTIONAL

Source: BOM [Field Safety Guide,] 1993

SUBSIDENCE:

Description: Area where the ground has sunk creating a depression or hole

Required/Optional: OPTIONAL

Source: BOM [Field Safety Guide,] 1993

MILLSITE:

Description: A mineral treatment plant in which crushing, grinding, and further processing of ore is conducted to produce a product.

Required/Optional: OPTIONAL

Source: BOM [Field Safety Guide,] 1993

DUMP:

Description: A pile or heap of waste rock material or other non-ore refuses near a mine.

Required/Optional: OPTIONAL

Source: [BOM Hazard Evaluation Handbook, page 76, 5/94]

TAILINGS:

Description: The refuse material resulting from washing, concentration, or treatment of ground ore.

Required/Optional: OPTIONAL

Source: [BOM Hazard Evaluation Handbook, page 82, 5/94]

IMPOUNDMENT:

Description:  An area, often contained by a dam, used as a water supply for mining or milling operation, or as a settling pond for tailings slurries.

Required/Optional: OPTIONAL

Source: [BOM Hazard Evaluation Handbook, page 77, 5/94]

OTHER:

Description: Does fit one of the above definitions..

Required/Optional: OPTIONAL

Source: [Laura M. Shepherd 5/26/98]]

78. CONDITIONS:

INTACT:

Description: This describes the condition of the opening: Does it appear stable or solid or has it undergone some failure?  This information is useful for designing a closure.

Required/Optional: OPTIONAL

Source: Dennis Inman [BOM, Field Safety Guided] 1993

PARTIALLY COLLAPSED:

Description: This is an indication of the condition of a shaft, adit, or high wall.  It can indicate that future failure is likely.

Required/Optional: OPTIONAL

Source: Dennis Inman [BOM, Field Safety Guide] 1993

FULLY COLLAPSED:

Description: This indicates that the opening no longer poses a threat.  Nature has taken care of the potential problem.

Required/Optional: OPTIONAL

Source: Dennis Inman [BOM, Field Safety Guide] 1993

STABLE:

Description: Not changing or fluctuating; unvarying; permanently enduring

Required/Optional: OPTIONAL

Source: [Webster’s Ninth New Collegiate Dictionary, 1990]

UNSTABLE:

Description: Lacking steadiness; apt to move, sways, or fall.

Required/Optional: OPTIONAL

Source: [Webster’s Ninth New Collegiate Dictionary, 1990]

N:

Description: N equal’s feature not found at site or on map/sketch.

Required/Optional: OPTIONAL

Source: [Webster's Ninth New Collegiate Dictionary, 1990]

79. HEIGHT:

Description: Height of the opening.

Required/Optional: OPTIONAL

Source: [Forest\Work\Dbase.wq1

80. & 92 WIDTH:

Description: Width of the opening of a feature.

Required/Optional: OPTIONAL

Source: [Forest\Work\Dbase.wq]

81. DEPTH:

Description: Depths need to be recorded when they pose a threat to humans and animals.  At a minimum, depths should be recorded for any shaft more than 6 feet with near vertical sides or adits that can be entered.  If entrapment can be an outcome of entering the opening, it should be recorded.

Required/Optional: OPTIONAL

Source: [Dennis Inman (BOM, Field Safety Guide), 1993]

82. & 94. UNITS:

Description: Record which units were used; English or metric.

Required/Optional: OPTIONAL

Valid Values

| English | Metric | Unknown |
| --- | --- | --- |

Source: [Forest\Work\Dbase.wq]

83. & 99. WATER:

Description: The release of any water from a man made opening is discharge and needs to be recorded.  If a natural spring is on the site, but has nothing to do with the past activity, it can be recorded but it is of no immediate concern.  We need to know volume, color, smell, and any other note worthy comments.

Required/Optional: OPTIONAL

Valid Values

| W=Water draining from the feature | S=Standing water in the feature | N=No water | SP=Water seeping from side of feature |
| --- | --- | --- | --- |

Source: [Dennis Inman, 6/21/96]

84. 104., & 125 LATITUDE GPS:

Description: Latitude is the angular distance measured in degrees north or sought of the equator, recorded in degrees, minutes and seconds. If feature was GPS

Required/Optional: OPTIONAL

Source: [AZ State Land Department letter, AZ Mine Locations: Data Base Design, 10/17/95]

85. 105. & 126. LONGITUDE GPS:

Description: Longitude is the arc or portion of the earth’s equator intersected between the meridian of a given place and the prime meridian, recorded in degrees, minutes and seconds.  If feature was GPS.

Required/Optional: OPTIONAL

Source: [AZ State Land Department letter, AZ Mine Locations: Data Base Design, 10/17/95]

86. & 101. OWNERSHIP:

Description: The classification of federal administration, private, local, or state holding on both the surface estate. Required/Optional: OPTIONAL

### Valid Values

| Ownership Code | Ownership Name |
| --- | --- |
| BLM | Bureau of Land Management |
| FS | Forest Service |
| IND | Indian Reservation |
| MIL | Military |
| ST | State |
| NP | National Parks |
| PRI | Private |
| UN | Unknown |

Source: [R3 GIS Data dictionary has a definition in the Ownership and Jurisdiction section, page 37 for a more complicated term, both surface and subsurface ownership involved]

87. 106. & 118. NOTES:

Description: Comments on individual features.

Required/Optional: OPTIONAL

Source: [Forest\Work\Dbase.wq]

88. 107. & 119. PHOTO:

Description: A photo is available of the record a specific feature or condition.

Required/Optional: OPTIONAL

Valid Values

| Yes | No | Unknown |
| --- | --- | --- |

Source: [BOM Hazard Evaluation Handbook, page 56, 5/94 Draft]

91. LENGTH:

Description: Length of the opening.

Required/Optional: OPTIONAL

Source: [Forest\Work\Dbase.wq1]

93. VOLUME:

Description: The amount of a substance occupying a particular three-dimensional figure as measured in cubic units.

Required/Optional: OPTIONAL

Source: [Webster's Ninth New Collegiate Dictionary, 1990]

95. SIZE OF MATERIAL:

Description: Fine is powdery or silky; gravel is larger than sand up to about 3 inches in diameter or throwing rocks; cobbles are brick size; boulders are over 12 inches in diameter & lobbies.

Required/Optional: OPTIONAL

Valid Values

| F=Fine | S=Sand | G=Gravely | B=Boulders | C=Cobbles |
| --- | --- | --- | --- | --- |

Source: [BOM Hazard Evaluation Handbook, page 76, 5/94]

96. WATER EROSION:

Description:  The wearing away of the land surface by running water.

Required/Optional: OPTIONAL

Valid Values

| Gully: The erosion process whereby water accumulates in narrow channels, & over short periods, removes the soil from this narrow area to considerable depths, ranging from 1:2 feet to 75:100 feet. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Sheet: The removal of a thin, fairly uniform layer of soil from the land surface by runoff water. |  |  |  |  |
| Rill: An erosion process in which numerous small channels only several inches deep are formed; occurs mainly on recently cultivated soils and/or recent cuts and fills |  |  |  |  |
| M= Wind/Water | Seep= Intermittent | Pond= Seasonal | Breach= Breached | N=None |

Source: [Resource Conservation Glossary, 3rd Edition, 1982]

97. WIND EROSION:

Description:  The wearing away of the land surface by running water, wind, and etc..

Required/Optional: OPTIONAL

Valid Values

| High=Fine, | Mod=Moderate, | Low=Little if any fines |
| --- | --- | --- |

Source: [Resource Conservation Glossary, 3rd Edition, 1982

98. VEGETATION:

Description:  Plant life or total plant cover (as of an area).

Required/Optional: OPTIONAL

Valid Values

| Dense=Groundcover > 75% | Barren= Groundcover 25% – 75% | Moderate=Groundcover <25% | Sparse=Barren |
| --- | --- | --- | --- |

Source: [Webster's Ninth New Collegiate Dictionary, 1990]

100. DUMP IN DRAINAGE:

Description: A dump located in a drainage or adjacent to a drainage

Required/Optional: OPTIONAL

Valid Values

| Yes | No | Unknown |
| --- | --- | --- |

Source: Gretchen Barkmann 5/20/98

102. INDICATORS:

Description: A substance used to show visually the condition of a solution with respect to the presence of a particular material.

Required/Optional: OPTIONAL

Valid Values

| No=None | Veg=Vegetation | Stain=Stain | Salt=Salt | Sulfa=Sulfifides | Turb=Discolored |
| --- | --- | --- | --- | --- | --- |

Source: [Webster's Ninth New Collegiate Dictionary, 1990]

103. STABILITY:

Description: Resistance to chemical change or to physical disintegratration

Required/Optional: OPTIONAL

Valid Values

| Unstable | Potentially Unstable | Stable |
| --- | --- | --- |

Source: [Webster's Ninth New Collegiate Dictionary, 1990]

110. SAMPLE NUMBER:

Description: Indicate feature number associated with water information.

Required/Optional: OPTIONAL

Source: Laura M. Shepherd 5/26/98

111. FLOW:

Description: Record flow in gallons per minute, cubic feet per second, or as a seep (no volume amount).

Required/Optional: OPTIONAL

Source: Dennis Inman 6/26/96

112. CONDUCTIVITY:

Description: The resistance to electrical flow in water is measured in mhos.  The maximum is 1,500 mhos for potable water.  There is concern any time this value is exceeded.

Required/Optional: OPTIONAL

Source: Dennis Inman [reference to NM State E.D. Surface Water Quality Standard]

113. METHOD:

Description: How the flow rate was measured.

Required/Optional: OPTIONAL

Valid Values

| E=Estimate | T=bobbler/stopwatch/x-section | W=weir | D=catchment | F=Flow meter |
| --- | --- | --- | --- | --- |

Source: Region 1 Form

114. pH

Description: A measure of acidity; the hydrogen ion activity in a solution expressed as the negative log (base 10) of the hydrogen ion concentration on a scale of 0:14 with <4 triggering further investigation.

Required/Optional: OPTIONAL

Source: [R3 GIS Data dictionary {Water pH}, Water Theme, page 56 from the American Fisheries Society glossary]

115. LOCATION:

Description: Location of sample location in relation to the feature.

Required/Optional: OPTIONAL

Valid Values

| A=immediately adjacent to adit/shaft | B=below/tailing | C=immediately above confluence with receiving stream |  | SW=standing water in/on feature. |
| --- | --- | --- | --- | --- |
| RU=Receiving stream upstream of feature | RD=Receiving stream downstream of feature |  |  |  |

Source: Region 1 Form

116. INDICATORS:

Description: Field evidence, which indicates potential toxicity.

Required/Optional: OPTIONAL

Valid Values

| N=none | A=absence of Benthic organism | W= opaque water | P=yellow or red precipitate | S= suspended solids | D=salt deposit |
| --- | --- | --- | --- | --- | --- |

Source: Region 1 Form

117. TOXICITY:

Description: Field evidence, which indicates potential toxicity.

Required/Optional: OPTIONAL

Valid Values

| N=None | A=Absence of benthic organisms | W=Opaque Water | P=Yellow or red precipitate | S=Suspended soils | D=Salt Deposits |
| --- | --- | --- | --- | --- | --- |

Source: [Kathleen S. Adam, 1998]

#### REGION 10 DEFINITIONS

- GENERAL INFORMATION

PRODUCER TYPE:

Description:  Placer mine, prospect, glory hole, mine, occurrence, etc.

Required/Optional: OPTIONAL

Source: [Kathleen S. Adam, 1998]

CURRENT:

Description:  Ownership of subsurface claim at this exact point in time.

Required/Optional: OPTIONAL

Source: [Kathleen S. Adam, 1998]

ACCESS:

Description:  Ability to enter the site, i.e. 2-wheel drive, 4-wheel drives, etc.

Required/Optional: OPTIONAL

Source: [Webster's Ninth New Collegiate Dictionary, 1990]

TOWN:

Description:  A compactly settled area usually larger than a village but smaller than a city.

Required/Optional: OPTIONAL

Source: [Webster's Ninth New Collegiate Dictionary, 1990]

RESIDENCE:

Description:  A building used as a home.

Required/Optional: OPTIONAL

Source: [Webster's Ninth New Collegiate Dictionary, 1990]

WETLAND/BOG:

Description: Wetland: Those areas inundated by surface or ground water with a frequency sufficient to support, and under normal circumstances do or would support, prevalence of vegetation or aquatic life that requires saturated or seasonally saturated soil conditions for growth and reproduction.  Wetlands generally include swamps, marshes, bogs, and similar areas such as sloughs, potholes, wet meadows, river overflows, mud flats, and natural ponds. Bog:  Wet, spongy ground especially a poorly drained, usually acid area rich in plant residues, frequently surrounding a body of open water, and having a characteristic flora (as of sedges, heaths, and sphagnum).

Required/Optional: OPTIONAL

Source: [FSM 2527.05, Bog: Webster’s Ninth New Collegiate Dictionary, 1990]

- LOCAL UNDISTURBED VEGETATION

BARREN OR SPARSELY:

Description:  Producing no vegetation or vegetation of few and scattered elements.

Required/Optional: OPTIONAL

Source: [Webster's Ninth New Collegiate Dictionary, 1990]

WEEDS/GRASSES:

Description:  Weed: A plant that is not valued where it is growing or that tends to overgrow or choke out more desirable plants.  Grass:  Any of a large family (Gramineae) of monocotyledonous, mostly herbaceous plants with jointed stems, slender sheathing leaves, and flowers borne in spikel ets of bracts.

Required/Optional: OPTIONAL

Source: [Webster's Ninth New Collegiate Dictionary, 1990]

BRUSH:

Description:  Land covered with scrub vegetation.

Required/Optional: OPTIONAL

Source: [Webster's Ninth New Collegiate Dictionary, 1990]

RIPARIAN/MARSH:

Description:  Riparian: Related to or living or located on the bank of a natural watercourse (as a river) or sometimes of a lake or a tidewater.  Marsh:  A tract of soft, wet land usually characterized by monocotyledons (as grasses or cattails).

Required/Optional: OPTIONAL

Source: [Webster's Ninth New Collegiate Dictionary, 1990]

DECIDUOUS TREES:

Description:  Trees that shed seasonally or at a certain stage of development in the life cycle.

Required/Optional: OPTIONAL

Source: [Webster's Ninth New Collegiate Dictionary, 1990]

PINE/SPRUCE/FIR:

Description:  Pine: Any of a genus of coniferous evergreen trees, which have slender elongated needles.  Spruce:  Any of a genus of evergreen trees of the pine family with a conical head of dense foliage and soft lightwood.  Fir:  Any or a genus of north temperate evergreen trees of the pine family that have flattish leaves, smooth circular leaf scars, and erect cones.

Required/Optional: OPTIONAL

Source: [Webster's Ninth New Collegiate Dictionary, 1990]

- SITE/LOCAL TERRAIN

ROLLING OR FLAT:

Description:  Rolling: To have an undulating contour.  Flat: To have a level surface of land.

Required/Optional: OPTIONAL

Source: [Webster's Ninth New Collegiate Dictionary, 1990]

FOOTHILLS:

Description:  A hill at the foot of higher hills.

Required/Optional: OPTIONAL

Source: [Webster's Ninth New Collegiate Dictionary, 1990]

MESA:

Description:  An isolated, relatively flat-topped, natural elevation usually more extensive than a butte and less extensive than a plateau.

Required/Optional: OPTIONAL

Source: [Webster's Ninth New Collegiate Dictionary, 1990]

MOUNTAINS:

Description:  A landmass that projects conspicuously above its surroundings and is higher than a hill.

Required/Optional: OPTIONAL

Source: [Webster's Ninth New Collegiate Dictionary, 1990]

STEEP/NARROW CANYON:

Description:  Narrow Canyon: A deep valley of slender width and often with a stream flowing through it.  Steep Canyon: A canyon whose sides make a large angle with the plane of the horizon.

Required/Optional: OPTIONAL

Source: [Webster's Ninth New Collegiate Dictionary, 1990]

- ACID INDICATOR

ARSENOPYRITE:

Description:  Iron-arsenic sulfide, FeAsS.

Required/Optional: OPTIONAL

Source: [BOM AML Inventory and Hazard Evaluation Handbook, Sept. 1993]

CHALCOPYRITE:

Description:  Copper-iron sulfide, CuFeS2.

Required/Optional: OPTIONAL

Source: [BOM AML Inventory and Hazard Evaluation Handbook, Sept. 1993]

GALENA:

Description:  Lead sulfide, PbS.

Required/Optional: OPTIONAL

Source: [BOM AML Inventory and Hazard Evaluation Handbook, Sept. 1993]

IRON OXIDE:

Description:  Iron (and possibly other metallic ions) in combination with oxygen.

Required/Optional: OPTIONAL

Source: [BOM AML Inventory and Hazard Evaluation Handbook, Sept. 1993]

LIMONITE:

Description:  Iron hydroxide, HFeO2.

Required/Optional: OPTIONAL

Source: [BOM AML Inventory and Hazard Evaluation Handbook, Sept. 1993]

MARCASITE:

Description:  Iron sulfide, FeS2.

Required/Optional: OPTIONAL

Source: [BOM AML Inventory and Hazard Evaluation Handbook, Sept. 1993]

PYRITE:

Description:  Iron sulfide, FeS2.

Required/Optional: OPTIONAL

Source: [BOM AML Inventory and Hazard Evaluation Handbook, Sept. 1993]

PYRRHOTITE:

Description:  Iron sulfide, Fe1-xS.

Required/Optional: OPTIONAL

Source: [BOM AML Inventory and Hazard Evaluation Handbook, Sept. 1993]

SPHALERITE:

Description:  Zinc-iron sulfide, (Zn, Fe) S.

Required/Optional: OPTIONAL

Source: [BOM AML Inventory and Hazard Evaluation Handbook, Sept. 1993]

OTHER SULFIDE:

Description: Any other sulfide not defined above.

Required/Optional: OPTIONAL

Source: [Kathleen S. Adam, 1998]

- HAZARDS

PHYSICAL HAZARDS:

Description: Defined by presence of outside vertical openings (OVO) and underground workings/vertical openings (UW/VO) and mill facility (MF) and cabins/structures (C/S), and/or access/visitation (A/V).

Required/Optional: REQUIRED

Source: [R10 Abandoned and Inactive (AIM) Inventory, 1995/1996 Results]

PETROLEUM HAZARDS:

Description: Defined by the presence of quantity of petroleum product (Q) and condition of containment (CoC) and proximity of potential target (PPT).

Required/Optional: REQUIRED

Source: [R10 Abandoned and Inactive (AMI) Inventory, 1995/1996 Results]

CHEMICAL HAZARDS:

Description: Defined by the presence of downstream expedience (DE) and flow rate (FR) and proximity of potential target (PPT).

Required/Optional: REQUIRED

Source: BOM [MILS Guide]

- OPERATIONAL HISTORY

Dates of Significant Mining Activity

STARTING DATE:

Description: Beginning date of significant mining activity.

Required/Optional: OPTIONAL

Source: [Kathleen S. Adam, 1998]

ENDING DATE:

Description: Concluding date of all significant mining activity.

Required/Optional: OPTIONAL

Source: [Kathleen S. Adam, 1998]

- MINE PRODUCTION

COMMODITY:

Description: A mineral resource with sufficient value to be individually produced and traded on the open market.

Required/Optional: OPTIONAL

Source: [U.S. Bureau of Mines Mineral Commodity Summaries]

PRODUCTION:

Description: Amount of commodity removed from the site.

Required/Optional: OPTIONAL

Source: [Kathleen S. Adam, 1998]

- MILL PROCESS

YEARS THAT MILL OPERATED:

Description: When the Mill was operated.

Required/Optional:  OPTIONAL

Source:  [Kathleen S. Adam, 1998]

AMALGAMATION:

Description: The process by which mercury is alloyed with some other metal to produce an amalgam.  Used at one time for the extraction of gold and silver from pulverized ores.

Required/Optional: OPTIONAL

Source: BOM [Abandoned Mine Land Inventory and Hazard Evaluation Handbook, 1993]

ARRASTRE; ARRASTRA:

Description: A circular rock-lined pit in which broken ore is pulverized by stones attached to a pillar and dragged around the pit.  The arrester was in common use in the western and southwest United States in the 18th and 19th centuries.

Required/Optional: OPTIONAL

Source: BOM [Abandoned Mine Land Inventory and Hazard Evaluation Handbook, 1993]

LEACH:

Description:  (1) The removal in solution of the more soluble minerals by percolating waters.  (2) Extracting a soluble metallic compound from an ore by selectively dissolving it in a suitable solvent, such as water, sulfuric acid, hydrochloric acid, cyanide, etc.

Required/Optional: OPTIONAL

Source: BOM [Abandoned Mine Land Inventory and Hazard Evaluation Handbook, 1993]

JIG PLANT:

Description: A recovery plant that uses mineral jigs to recover mineral values.  A mineral jig is a machine in which the feed is stratified in water by means of a pulsating motion and from which the stratified products are separately removed, the pulsating motion usually being obtained by alternate upward and downward currents of the water.

Required/Optional: OPTIONAL

Source: BOM [Abandoned Mine Land Inventory and Hazard Evaluation Handbook, 1993]

CYANIDATION:

Description: Recovery process involving the use of cyanide compounds to dissolve metal values.

Required/Optional: OPTIONAL

Source: BOM [Abandoned Mine Land Inventory and Hazard Evaluation Handbook, 1993]

FLOTATION:

Description: The method of mineral separation in which a froth created in water by a variety of reagents floats some finely crushed minerals whereas other minerals sink.

Required/Optional: OPTIONAL

Source: BOM [Abandoned Mine Land Inventory and Hazard Evaluation Handbook, 1993]

RETORT:

Description: A device used to distill off and recover mercury from amalgam.

Required/Optional: OPTIONAL

Source: BOM [Abandoned Mine Land Inventory and Hazard Evaluation Handbook, 1993]

GRAVITY:

Description: A process in which heavy metals or minerals are separated from waste by the action of agitation and gravity on materials suspended in a liquid, usually water.

Required/Optional: OPTIONAL

Source: BOM [Abandoned Mine Land Inventory and Hazard Evaluation Handbook, 1993]

HEAP LEACH:

Description: A recovery process in which prepared ore is stacked in heaps on impervious pads and a solvent percolated through the heap to dissolve selected metal values.

Required/Optional: OPTIONAL

Source: BOM [Abandoned Mine Land Inventory and Hazard Evaluation Handbook, 1993]

NO MILL:

Description: The absence of a mineral treatment plant in which crushing, grinding, and further processing of ore is conducted to produce a product.

Required/Optional: OPTIONAL

Source: BOM [Abandoned Mine Land Inventory and Hazard Evaluation Handbook, 1993]

STAMP:

Description: An apparatus (also the building containing the apparatus) in which rock is crushed by a stamp battery.  A stamp battery consists essentially of a crushing member (composed of a stem, head, and shoe), which is dropped on a die, the ore being crushed in water between shoe and die.  An important crushing machines in the past but now becoming obsolete.

Required/Optional: OPTIONAL

Source: BOM [Abandoned Mine Land Inventory and Hazard Evaluation Handbook, 1993]

CIP (Carbon-in-Pulp):

Description: A recovery process in which slurry of gold ore, free carbon particles, and cyanide is mixed together.  Cyanide dissolves gold from the ore and the gold are simultaneously adsorbed onto the carbon.  The carbon is separated from the slurry and the gold recovered.

Required/Optional: OPTIONAL

Source: BOM [Abandoned Mine Land Inventory and Hazard Evaluation Handbook, 1993]

CRUSHER ONLY:

Description: A machine for crushing rock or other materials.  Among the various types of crushers are the ball mill, gyrator crusher, Hadsel mill, hammer mill, jaw crusher, rod mill, rolls, stamp mill, and tube mill.

Required/Optional: OPTIONAL

Source: BOM [Abandoned Mine Land Inventory and Hazard Evaluation Handbook, 1993]

UNKNOWN:

Description: A mill process that does not fit any of the above definitions.

Required/Optional: OPTIONAL

Source: BOM [Abandoned Mine Land Inventory and Hazard Evaluation Handbook, 1993]

- MILL PRODUCTION

COMMODITY:

Description: Mineral product produced at mine or mill (e.g. gold, silver, and copper).

Required/Optional: OPTIONAL

Source: BOM [Abandoned Mine Land Inventory and Hazard Evaluation Handbook, 1993]

PRODUCTION:

Description: Total quantity of mineral extracted from a commodity.

Required/Optional: OPTIONAL

Source: [Webster's Ninth Collegiate Dictionary, 1990]

- MISCELLANEOUS

PIPE:

Description: A long tube or hollow body for conducting a liquid, gas, or finely divided solid or for structural purposes.

Required/Optional: OPTIONAL

Source:  [Webster's Ninth Collegiate Dictionary, 1990]

POLE:

Description: A long, slender, usually cylindrical object (as a length of wood).

Required/Optional: OPTIONAL

Source:  [Webster's Ninth Collegiate Dictionary, 1990]

SCRAP METAL:

Description: Fragments of discarded or leftover metal.

Required/Optional: OPTIONAL

Source: [Webster's Ninth Collegiate Dictionary, 1990]

OVERHEAD WIRE:

Description: Metal in the form of a usually very flexible thread or slender rod operating, lying, or coming from above.

Required/Optional: OPTIONAL

Source:  [Webster's Ninth Collegiate Dictionary, 1990]

OVERHEAD CABLE:

Description: A wire rope or metal chain of great tensile strength operating, lying, or coming from above.

Required/Optional: OPTIONAL

Source: [Webster's Ninth Collegiate Dictionary, 1990]

HEADFRAME:

Description: Vertical steel or timber frame at the top of a shaft, which carries the sheave or pulley for the hoist.

Required/Optional: OPTIONAL

Source: BOM [Abandoned Mine Land Inventory and Hazard Evaluation Handbook, 1993]

WOODEN STRUCTURE:

Description:  A structure made of wood to support, shelter, or enclose persons, animals, or property of any kind.

Required/Optional: OPTIONAL

Source: [FSH 7309.11]

TOWER:

Description: A building or structure typically higher than its diameter and high relative to its surroundings that may stand apart, or be attached to a larger structure, and that may be fully walled in or of skeleton framework.

Required/Optional: OPTIONAL

Source:  [Webster's Ninth Collegiate Dictionary, 1990]

POWER SUBSTATION:

Description: A subsidiary station in which electric current is transformed.

Required/Optional: OPTIONAL

Source:  [Webster's Ninth Collegiate Dictionary, 1990]

ANTENNAE:

Description: A usually metallic device (as a rod or wire) for radiating or receiving radio waves.

Required/Optional: OPTIONAL

Source:  [Webster's Ninth Collegiate Dictionary, 1990]

TRESTLE:

Description:  A braced framework of timbers, piles, or steelwork serving as support.

Required/Optional: OPTIONAL

Source: BOM [Abandoned Mine Land Inventory and Hazard Evaluation Handbook, 1993]

POWERLINES:

Description:  A source or means of supplying energy via a system of wires

Required/Optional: OPTIONAL

Source: [Webster's Ninth Collegiate Dictionary, 1991]

TRANSFORMERS:

Description: A device employing the principle of mutual induction to convert variations of current in a primary circuit into variations of voltage and current in a secondary circuit

Required/Optional: OPTIONAL

Source: [Webster's Ninth Collegiate Dictionary, 1991]

TRAMWAY:

Description:  An overhead cable or tails for trams

Required/Optional: OPTIONAL

Source: [Webster's Ninth Collegiate Dictionary, 1991]

FLUMES:

Description:  An inclined channel for conveying water.

Required/Optional: OPTIONAL

Source:  [Webster's Ninth Collegiate Dictionary, 1991]

TRAM BUCKET:

Description:  A box like wagon running on a tramway or railway.

Required/Optional: OPTIONAL

Source:  [Webster's Ninth Collegiate Dictionary, 1991]

FENCES:

Description:  A barrier intended to prevent escape or intrusion or to mark a boundary; esp.: such a barrier made of posts and wire or boards.

Required/Optional: OPTIONAL

Source: [Webster's Ninth Collegiate Dictionary, 1991]

MACHINERY:

Description:   Machines in general or as a functioning unit.

Required/Optional: OPTIONAL

Source: [Webster's Ninth Collegiate Dictionary, 1991]

#### BIBLIOGRAPHY

A-8 Attachments A & B to OSM Directive AML-1

Arizona Mine Locations

Arizona State Land Department, Letter titled Arizona Mine Locations: Data Base Design dated October 17, 1995

BOM Hazard Evaluation Handbook,

BOM Field Safety Guide and MILS Guide

FEDPLAN Update Instructions

Forest Service Handbook (FSH) 7709.11, Transportation Engineering Handbook

Forest Service Manual (FSM) 2100, Environmental Management

FSH 7309.11, Buildings and Related Facilities Handbook

FSM 2500, Watershed and Air Management

FSM 2600, Wildlife, Fish, and Sensitive Plant Habitat Management

FSM 2800, Minerals and Geology

FSM 7700, Transportation System

Hazardous Waste Q & A, The, Revised Edition, 1992

Hydrology for Engineers, Linsley, Kohler, Paulhus, McGraw Hill, 1982.

Infrastructure Version 1.5 Training manual

Map Use by Phillips C. Muekrcke

Park Service Form

Principles of Hazardous Material Management, Roger D.Griffin, 1988, Lewis Publisher

Random House College Dictionary, 1984

Region 1, Abandoned Mine Lands Survey  (AMLS) Form

Region 3, Dams Data Dictionary

Region 3, Graphic Information System (GIS) Data Dictionary

Region 3, Original AMLS Form

Resource Conservation Glossary, 3rd Edition, 1982

Title 42, United States Code, Section 300f, Safe Drinking Water Act (SDWA)

U.S. Bureau of Mines Commodity Summaries

U.S. Department of the Interior, Bureau of Mines, Abandoned Mine Land Inventory and Hazard Evaluation Handbook, May 1994

Webster's Ninth New Collegiate Dictionary, 1990

Webster's Tenth New Collegiate Dictionary, 1995

The Wilderness Act Handbook