Proposal for Integrating the "Dose-Response Analysis" Software into BMDS

1.  “Dose-Response Analysis” software

This software was written in VB (version 4) and it has one main file “Doseprg.bas” and four visual components: Dose1.frm, Dose2.frm, Dose3.frm, and Dose4.frm.

The input data are in a text file (.NRD) and the format is as the following:

3                          number of input variables

68                         number of observations

Conc mg/m3

Minutes

BW grams

Exposed

Dead

952                     ------------

15                                   | one record

200                                  |

10     1                ------------

……

The output data are also in a text file called “DoseResp.Log” and it has the following format:

Filename = ClF5many.nrd

04-12-2006    1:02:20 PM

Conc mg/m3     Minutes    BW grams     Exposed        Dead

952.00       15.00      200.00         10.          1.

1278.00       15.00      200.00         10.          4.

0.00       15.00      200.00         10.          6.

1631.00       15.00      200.00         10.          7.

1767.00       15.00      200.00         10.          9.

2028.00       15.00      200.00         10.          6.

2349.00       15.00      200.00         10.          9.

653.00       30.00      200.00         10.          0.

886.00       30.00      200.00         10.          0.

1006.00       30.00      200.00         10.          3.

1033.00       30.00      200.00         10.          6.

1267.00       30.00      200.00         10.          9.

1359.00       30.00      200.00         10.         10.

435.00       60.00      200.00         10.          0.

544.00       60.00      200.00         10.          1.

……

Selection of trials from number  1  through  68

Transformation of variables

Conc mg/m3     is transformed logaritmically!

Minutes        is transformed logaritmically!

BW grams       is transformed logaritmically!

Probit model used without background response correction!

Variable  1   =  Conc mg/m3

Variable  2   =  Minutes

Variable  3   =  BW grams

Variable  4   =  combination of Conc mg/m3 and Minutes

Chi-Square          =  114.86

Degrees of Freedom  =  63

B 0  = -1.198E+01  Student t  for B 0  = -2.1

B 1  = 2.233E+00   Student t  for B 1  = 2.71

B 2  = 1.524E+00   Student t  for B 2  = .96

B 3  = -2.334E-01  Student t  for B 3  = -6.74

B 4  = -8.252E-02  Student t  for B 4  = -.35

variance  B 0  0  = 3.246E+01

covariance  B 0  1  = -4.678E+00

covariance  B 0  2  = -8.864E+00

covariance  B 0  3  = 2.574E-02

covariance  B 0  4  = 1.278E+00

variance  B 1  1  = 6.792E-01

covariance  B 1  2  = 1.293E+00

covariance  B 1  3  = -3.231E-03

covariance  B 1  4  = -1.881E-01

variance  B 2  2  = 2.539E+00

covariance  B 2  3  = -1.439E-03

covariance  B 2  4  = -3.726E-01

variance  B 3  3  = 1.201E-03

covariance  B 3  4  = -2.416E-04

variance  B 4  4  = 5.532E-02

When the graphics is to display, it also generates a text data file (.Asc) like this:

1.000     0.000     0.000     0.000

2.000     0.000     0.000     0.000

3.000     0.000     0.000     0.000

4.000     0.000     0.000     0.000

5.000     0.000     0.000     0.000

6.000     0.000     0.000     0.000

7.000     0.000     0.000     0.000

8.000     0.001     0.001     0.001

9.000     0.001     0.001     0.001

10.000     0.001     0.001     0.001

11.000     0.001     0.001     0.001

12.000     0.001     0.001     0.001

13.000     0.001     0.001     0.001

14.000     0.001     0.001     0.001

15.000     0.001     0.001     0.001

……

Also an image file (.Wmf) is created while the graphics is displayed.

2. About the VB/Basic to C translators

A few open source VB/Basic to C code translators have been investigated and none of them would translate the whole “Dose-Response Analysis” software into C and make it executable.

The converters examined are:

a) b2cse -- which only converts Visual Basic automation code to Visual C++ (http://www.microsoft.com/downloads/search.asp),

b) VB2C – which only converts individual .frm files into C/gtk style code, and

c) BCX – that inputs a “BCX BASIC” source code file and outputs a “C” source code file.

BCX works fine to translate the Doseprg.bas file into a whole .c file but it doesn’t work on the 4 .frm files. The translated Doseprg.c file is meaningful except it leaves those GUI components only variable names in the main file.

3. Some considerations on the integration of software

Based on the investigation, the input data file can be easily generated through BMDS. The output file needs to transform into gnu plot format before it can be viewed in BMDS. The dynamic selection of parameters on the Ten Berge model makes it hard to use the existing software components without big changes.

The good news is that, if we decide to totally convert the VB code into C code, it doesn’t seem to be overwhelming. Because, firstly, the main file has been translated into meaningful C code; secondly, the visual components (.frm files) are mostly just GUI widgets to catch the values; and lastly, all 5 source files are less than 1000 lines of code.

4. Estimates of the software integration

The target here is to generate an executable that runs at command line, taking various combinations of input data/parameter values, and outputs the analysis of the variance data plus the graphics data that can be displayed with GNU plot tool.

- Figure out the logic relationships among all the VB functions (16hr);

- Redirect the takings of input data/parameter values into the software (40hr);

- Redirect the generation of the analysis result (20hr);

- Test by comparing the input/output from both Doseprg.exe and the reengineered executable and progressively correct errors (32hr);

- Reformat the input data/parameter values file if necessary (TBD);

- Reformat the output data into standard BMDS format and GNU plot format (20hr);

- Repeat step 4) to make sure the reengineered executable works correctly (24hr).

- Total hours estimated:  152 + TBD