# Draft Summary of THE ANALYSIS OF THE Emissions

# REPORTED IN THE 1999 nei fOR sTAGE I AND STAGE II OPERATIONS

# AT GASOLINE SERVICE STATIONS

### September 2002

Prepared by:

Pacific Environmental Services, Inc.

5001 South Miami Boulevard, Ste. 300

PO Box 12077

Research Triangle Park, NC 27709

### Prepared for:

### US Environmental Protection Agency and the

### Emission Inventory Improvement Program

Office of Air Quality Planning and Standards

Emission Factor and Inventories Group

RTP, NC

Table of Contents

##### Title											   Page

1.0	Introduction	1

2.0	REVIEW OF THE 1999 AREA & POINT SOURCE NEI	2

2.1	Review of Source Classification Code(s) (SCC)	2

2.2	Review of Gasoline Throughput	2

2.3	Review of VOC Emission Factors	3

2.4	Review of VOC Emissions	4

- Methodology FOR CALCULATING INDEPENDENT

EMISSION ESTIMATES	7

3.1	General	7

3.2	Gasoline Throughput Data	7

3.3	County Allocation of Gasoline Throughput	7

3.3.1	County Population	7

3.3.2	Vehicle Miles Traveled (VMT)	8

3.3.3	Service Station Sales	8

3.4	VOC Emission Factors/Methodology	9

3.4.1	Stage I Loading Operations	9

3.4.2	Stage II Loading Operations	11

3.4.3	UST Breathing & Emptying Operations	11

3.5	Regulatory Review	12

3.5.1	Stage I	12

3.5.2	Stage II	13

3.6	Rule Penetration	14

3.7	Hazardous Air Pollutant (HAP) Profiles	15

3.8	Calculations 	15

3.9	Results Compared to NEI	16

4.0	CONCLUSIONS & RECOMMENDATIONS	17

### List of Tables

Table											   Page

Table 1.	List of SCC’s Used in Analyzing the 1999 Point and Area

Source Inventory for Service Stations	18

Table 2.	999 DOE Reported Gasoline Throughput and

State-Reported Gasoline Throughputs 	19

Table 3.	VOC Emission Factors Listed in the 1999 Area and Point Source

NEI for Gasoline Service Stations	21

Table 4.	Gasoline Throughput and Emissions Reported in the 1999 NEI	23

Table 5.	Nationwide Service Station Consumption Distribution	24

Table 6.	HAP Vapor Profile for Various Gasoline Types	24

###### APPENDIX

Table											   Page

Table A-1.	Gasoline Throughput Reported in the Area Source NEI by State and SCC	26

Table A-2.	Gasoline Throughput Reported in the Point Source NEI by State and SCC	27

Table A-3.	Emissions Reported in the Area Source NEI by State and SCC	28

Table A-4.	Emissions Reported in the Point Source NEI by State and SCC	30

Table B-1.	State Stage I and Stage II Control Levels	31

Table C-1.	Average UST Temperatures Used for Stage I Calculations	45

Table D-1.	Counties Modeled with Federal Reformulated Gasoline	46

Table D-2.	Counties Modeled with Oxygenated Gasoline	49

Table E-1.	State-Level Comparison of PES and NEI VOC Emission Estimates	50

## Draft Summary of the Analysis of the Emissions Reported in the

## 1999 NEI for Stage I and II Operations at Gasoline Service Stations (Draft v2.0)

## PES Project I561.004

###### 1.0	INTRODUCTION

The draft 1999 National Emission Inventory (NEI) (draft v2.0) contains area and point source emission estimates for evaporative emissions resulting from the loading of gasoline at service stations.  The emission estimates in the NEI are a mixture of data supplied by EPA as well as State and/or local air pollution control agencies.  As a result, it is probable that the service station emission estimates in the 1999 NEI are based on inconsistent methodologies and assumptions.

This project had a two-fold goal: to analyze the emissions estimates in the 1999 NEI from service station operations, and to develop a methodology for estimating service station emissions on a State and county level for the entire U.S.  State- and county-level emissions comparisons were developed to show the differences between emissions listed in the 1999 NEI and emission estimates developed for this project.

This document summarizes PES’ analysis of the 1999 Area and Point Source NEI for service stations that dispense gasoline.  The analysis consisted of reviewing and summarizing the reported gasoline throughputs, emission factors, and emissions by State and Source Classification Code (SCC).  The reported data was reviewed for completeness and accuracy using the quality assurance (QA) checks described in this document.  Based on the results of the QA checks, PES summarized its conclusion and provided recommendations for improving the quality of the data.

Also discussed is PES’ approach for developing a consistent methodology for estimating VOC and HAP emissions from service station operations.  A comparison of the emissions calculated by PES and those reported in the NEI are also provided.  This effort included obtaining 1999 gasoline consumption data, allocating state-level gasoline consumption to the county level, reviewing State and local air regulations for control requirements, and developing county-level emission estimates resulting from gasoline loading and unloading operations at service stations.

For this effort, PES calculated emissions occurring from gasoline transfers into the service station’s underground storage tanks (i.e., Stage I emissions) and emissions occurring from automobile refueling (i.e., Stage II emissions).

Note:  “Stage II” refers to vehicle refueling operations at service stations (includes both displacement losses occurring during refueling as well as spillage losses).

###### 2.0	REVIEW OF THE 1999 AREA & POINT SOURCE NEI

2.1	Review of the Source Classification Code(s) (SCC)

PES reviewed SCCs associated with Stage I and Stage II operations at gasoline service stations as well as underground breathing and emptying losses at gasoline service stations.  The 28 SCC’s that PES reviewed are summarized in Table 1.

PES did not consider SCCs that were not specifically associated with gasoline operations at service stations.  For example, PES did not consider the SCC’s 40400401 through 40400406 because they were listed as “non-refinery” sources and were not specific to service stations.  Similarly, PES did not consider SCC’s like 250107000 and 2501070050 because they were associated with non-gasoline products (i.e., diesel).  Note that the Point Source NEI has SCCs that distinguish between “retail ” and “fleet” refueling operations.  Both types of operations were included in the analysis of the Point Source NEI.

Note:  “Retail” refueling operations refer to private vehicle refueling at retail service stations.  “Fleet" refueling refers to refueling conducted by an owner of a group of vehicles (e.g., a rental car or leasing company).

## 2.2	Review of Gasoline Throughput

PES reviewed the gasoline throughput data provided in the NEI.  The gasoline throughput listed in the NEI is summarized in Table 2.  Note that gasoline throughput was available for only eight States in the Area Source NEI and only 15 States in the Point Source NEI.  When reviewing the gasoline throughput data, PES conducted three quality assurance (QA) checks:

- Are the total reported throughputs reasonable when compared to another data source such as the Department of Energy?

- Are the same gasoline throughputs listed in the NEI’s for Stage I, Stage II, and UST breathing and emptying?

- Were the reported units for gasoline throughput reasonable/consistent?

To conduct QA Check #1, PES compared the gasoline throughputs in the NEI to the 1999 throughputs reported by the U.S. Department of Energy (DOE) on their Website at (http://www.eia.doe.gov/emeu/states/_states.html).  The DOE throughputs are summarized in Table 2 for easy comparison to the throughputs listed in the NEI.  As stated earlier, the NEI did not list gasoline throughput for most states.  For States that listed gasoline throughput, some were comparable to the DOE values (e.g., Missouri and Maryland [Area Source NEI]) and others were significantly different to the DOE values (e.g., Maine and Delaware [Area Source NEI]).

To conduct QA Check #2, PES compared the NEI gasoline throughputs for Stage I, Stage II, and underground storage tank breathing & emptying (B&E) losses.  PES assumed that throughputs associated with each type of operation should be the same for a given state (i.e., the amount of gasoline unloaded into a service station’s UST should be approximately equal to the amount of gasoline loaded into the vehicle from the UST).  Similarly, PES assumed that the amount of gasoline contributing to UST B&E emissions should be equal to the amount of gasoline loaded into the UST from Stage I processes.  For example as shown in Table 2, Missouri lists the same gasoline throughput (3,073,876,000 gallons) for its Stage I, Stage II, and UST operations in the Area Source NEI.  Similarly, Maryland listed approximately the same throughput amounts for its Stage I, Stage II, and UST operations in the Area Source NEI.  However, note the large difference in gasoline throughput for Colorado’s Stage I and II operations in the Point Source Inventory.  Similarly, note the large difference in gasoline throughput amounts for South Carolina’s Stage I, Stage II, and UST operations.  Also note that Massachusetts and Maine reported Stage I gasoline throughputs but no Stage II gasoline throughputs.  More detailed tables showing reported gasoline throughputs are located in Appendix A (Tables A-1 and A-2).

- To conduct QA Check #3, PES reviewed the NEI for gasoline throughput units.  Similar to gasoline throughput, few throughput units were listed in the NEI.  For those were listed, most were reported in units of 1,000 gallons (“E3GAL”) or gallons (“GAL”).  However, in the Area Source NEI, throughput units listed for South Carolina varied widely for the same SCC (e.g., Thousand Gallons, Thousand Tons, Million Cubic Feet, Million Gallons, Gallons, and Pounds).

Note:  Throughput discrepancies do not necessarily indicate that the emissions values are incorrect, since many of the listed emissions are apparently not based on the listed throughputs.

2.3	Review of VOC Emission Factors

PES queried the NEI to for VOC emission factors listed for the various service station operations.  Similar to throughput, the field for VOC emission factors did not contain a value.   When reviewing the emission factors, PES conducted two quality assurance (QA) checks:

- Are the reported emission factors reasonable?

- Were the reported emission factor units reasonable and consistent?

To conduct QA Check # 4, PES queried and summarized the emission factors reported in the NEI in Table 3.  Note that Table 3 is arranged by type of loading or process to allow for easy comparison between similar emission factors in the Area and Point Source NEI.

One method used to QA the reported emission factors was to compare emission factors for similar processes.  For example SCC 2501060053 (Area Source NEI) and SCCs 40600306 and 40600706 (Point Source NEI) represent Stage I balanced/submerged fill operations.  Note that the VOC emission factor listed in the Area Source NEI for Stage I balanced/submerged fill operations 0.3 lbs/103 gallons loaded.  However, the Point Source NEI lists emission factors for balanced/submerged fill as high as 2,000 lbs/103 gallons loaded.  Such a large difference in the value of the emission factors is not reasonable.  PES suspects that these extremely high emission factors are reporting errors.

Another method used to QA the NEI data was to compare the listed emission factors to the EPA average factors (see Table 5.2-7 of EPA’s AP-42 Manual) which are also listed in Table 3.  Generally, the emission factors listed in the Area Source NEI appear reasonable when compared to the average EPA factors (Some variability is expected due to differences in gasoline RVP and temperature).  However, the emission factors listed in the Point Source NEI varies by several orders of magnitude when compared to the EPA average factors (e.g., 0.01 – 2000 lbs/ 103 gallons for SCC 40600306) and appear to be in error.  For example in the Point Source NEI, spillage emission factors for SCC 40600402 range from 0.01 – 2000 lbs/103 gallons loaded.  Assuming gasoline weighs 5.6 lbs/gallon (Table 7.1-2, AP-42, September 1997), a spillage loss of 2,000 lbs/103 gallons loaded is approximately equivalent to 357 gallons/103 gallons loaded (approximately 1/3 liquid loss due to spillage.)  Note that the EPA average factor for spillage is 0.7 lbs/103 gallons loaded.

To conduct QA Check #5, PES queried the NEI data for emission factor units.  There were few emission factor units listed in the NEI.  Table 3 summarizes the VOC emission factor units listed in the NEI.  Although most of the emission factors were listed in units of “lbs/ 103 gallons”, there were erroneous emission factor units such as “103 gal/ton” and “Each/ton”.

2.4	Review of VOC Emissions

PES reviewed the NEI to determine the VOC emissions listed for the various service station operations.  Table 4 provides a summary of the VOC emissions listed in the NEI.  Tables A-3 and A-4 provide a more detailed summary of the emission reported in the NEI by State and SCC.  As shown in Table 4, the combined VOC emissions from the NEI are 814,791 tons.  The majority (98.4%) of the emissions are listed in the Area Source NEI.  Only 1.65% of the total emissions were listed in the Point Source NEI.  Because most, if not all, service stations are considered area sources, it appears reasonable that the majority of the emissions were listed in the Area Source NEI.  Also note that in the Point Source NEI, only 53.79 tons of VOC emissions were listed for “fleet operations” representing less than 0.1% of the emissions in the Point Source NEI.

When reviewing the emissions listed in the NEI, PES conducted four quality assurance (QA) checks:

- For a given state, are emissions listed in the Area and/or Point Source NEI?

- Are Stage I, Stage II, and UST B&E emissions listed for each state?

- Were emissions associated with SCC’s for nonspecific processes (e.g., “Stage I Total”) or with SCC’s for specific processes (e.g.,  “controlled” or “uncontrolled”)?

- Did states with Stage I and Stage II controls list emissions with the appropriate SCC?  For example, do States’ with Stage II controls list emissions using a SCC for controlled Stage II?

For QA Check #6, PES queried the NEI database and noted that there were no emission records for Alaska, Hawaii and Oregon in the Area or Point Source NEI.

PES conducted QA Check #7 using the Tables A-3 (Area Source NEI) and A-4 (Point Source NEI).  PES assumed that Stage I, Stage II, and UST breathing and emptying emissions should appear in the NEI for all states.  For example in the Area Source NEI, several states (e.g., Alabama, Arizona, Iowa, and Kansas) listed Stage I - Total, Stage II – Total, and UST B&E emissions.  Similarly, Delaware and Pennsylvania listed Stage I (controlled), Stage II, and UST B&E emissions.  However, California listed Stage II and UST B&E emissions but did not list Stage I emissions.  New York listed Stage I evaporative emissions but did not report any Stage II evaporative emissions or UST B&E emissions.  Similar observations were made in the Point Source NEI.

Note:  Theoretically a State would not have any breathing and emptying emissions from UST’s if they required Stage II controls on every service station because Stage II controls effectively maintains a saturated vapor space in the UST preventing additional evaporation of gasoline during nonloading periods.  However, Stage II is typically implemented at stations exceeding some throughput threshold (e.g., 10,000 gal/month); therefore, stations operating below the threshold would presumably not use Stage II, and would have UST breathing and emptying emissions.

PES made the following assumptions when conducting QA Check #8 (refer to Tables A-3 and A-4):

- Emissions associated with the Area Source SCC “250106000” (Service Station/Total: All Gasoline/All Processes) should include all Stage I, Stage II, and UST B&E emissions.  PES also assumed that emissions listed with this SCC would not be listed under any other Area Source SCC listed in Table 1.  (Note that this was not the case for Georgia, Missouri, and Ohio).

- Emissions associated with the Area Source SCC “2501060050” (Service Station/Stage 1: Total) should include emissions associated with submerged fill, splash fill, and submerged/balanced filling operations.  PES assumed that if 2501060050 was used to list emissions for a State, then emissions would not be listed using 2501060051, 2501060052, and 2501060053.  (Note that Florida, Kentucky, Louisiana, Michigan, and West Virginia did not pass this QA check).

- Emissions associated with the Area Source SCC “2501060100” (Service Station/Stage 2: Total) should include emissions associated with uncontrolled and controlled Stage II processes as well as spillage processes. PES also assumed that if a State listed emissions with this SCC, that it would not list emissions using 2501060101, 2501060102, and 2501060103.  (Note that Louisiana, Maine, Virginia, and West Virginia did not pass this QA check).

- UST B&E emissions should either be listed using Area Source SCC “250160200” (Underground Tank: Total) or 2501060201 (Underground Tank: Breathing and Emptying), but not both.  (Note that Alabama, Georgia and Michigan did not pass this QA check).

- Emissions associated with the Point Source SCC “40600305” (Stage I/Unloading) should include emissions associated with splash filling, submerged filling (controlled and uncontrolled), and UST B&E.  PES assumed that if a state listed emissions with this SCC that it should not report emissions using 40600301, 4060302, and 40600306.  (Note that Colorado did not pass this QA check).

- Emissions associated with the Point Source SCC “40600399” (Stage I/Not Classified) should include emissions associated with all splash filling, submerged filling (controlled and uncontrolled) and UST B&E.  PES assumed that if a state listed emissions with this SCC that it should not report emissions using 40600301, 40600302, 40600305, 40600306, and 40600307.  (Note that Colorado, Kentucky, Louisiana, and Ohio did not pass this QA check.)

- Emissions associated with the Point Source SCC “40600499” (Stage II/Not Classified) should include emissions associated with Stage II displacement (controlled and uncontrolled) and liquid losses.  PES assumed that if a state listed emissions with this SCC that it should not report emissions using 40600401, 40600402, and 40600403.  (Note that California, Louisiana, Pennsylvania, and Washington did not pass this QA check.)

PES conducted QA Check #9 using the Tables A-3 (Area NEI) and A-4 (Point NEI) as well as Table B-1, which summarize those counties where Stage I and Stage II controls are required.  For example in the Area Source NEI, Delaware listed emissions associated only with Stage I submerged loading operations (SCC 2501060051); however, the regulatory analysis indicated that vapor balancing is also required in Delaware; therefore, Delaware’s Stage I emissions should be associated with balanced submerged loading (SCC 2501060053) instead of submerged.  Similarly, Delaware listed only Stage II uncontrolled emissions (SCC 2501060101).  However Delaware requires Stage II controls statewide so there should be some emissions associated with controlled Stage II operations (SCC 2501060102).  Note that this QA check could not be applied to those states, which opted to report emissions using SCC’s for “Stage I Total” and “Stage II Total.”

###### METHODOLOGY FOR CALCULATING INDEPENDENT

###### EMISSIONS ESTIMATES

3.1	General

Provided below is a discussion PES’ approach for developing a consistent methodology for estimating VOC and HAP emissions from Stage I and Stage II service station operations for comparison to values reported in the NEI’s.  This effort included obtaining 1999 gasoline consumption data, allocating state-level gasoline consumption to the county level, reviewing state and local air regulations for Stage I and Stage II requirements, and developing county-specific emission estimates occurring from gasoline loading and unloading operations at service stations.  The methodology used to calculate emissions from the following operations are discussed later in this document:

- Uncontrolled Stage I (UST filling) loading operations (e.g., splash fill)

- Controlled Stage I loading operations (e.g., submerged fill and/or vapor balancing)

- Uncontrolled Stage II vehicle refueling operations (including spillage emissions)

- Controlled Stage II refueling operations (including spillage emissions)

- UST breathing and emptying operations.

3.2	Gasoline Throughput Data

For this effort, PES used 1999 state gasoline sales information provided on the U.S. Department of Energy’s (DOE) Website (http://www.eia.doe.gov/emeu/states/_states.html).  DOE provided monthly sales estimates (“1,000 gallon per day”) for regular, mid-grade, and premium gasoline.  To calculate monthly throughput estimates for each state, PES multiplied the daily estimates for each gasoline type for each month by the number of days in the month.  The annual DOE gasoline throughputs are summarized in Table 2 by state.  Note that DOE estimates that approximately 131 billion gallons of gasoline were dispensed nationwide in 1999.

Note:  PES used the monthly gasoline throughputs to calculate monthly emissions for each State in order to consider the effects of seasonal changes in temperature and RVP.

###### 3.3	County Allocation of Gasoline Throughput

PES considered several options for allocating the State-level gasoline sales data to the county level.  Such options included county population, vehicle miles traveled, and service station sales.  Each of these options is discussed below.

    - County Population

One methodology considered for allocating State-level gasoline sales to the county level was based on county population.  PES obtained 1999 county population estimates from the U.S. Bureau of Census (http://eire.census.gov/popest/archives/county/co_99_1.php).  Using this methodology, a county comprising 5% of its State’s population would consume 5% of the State’s gasoline consumption.  Although a simple methodology, this methodology may overestimate gasoline consumption in areas that have high populations that use mass transit.  It may also not account for more rural areas where lower populations may travel greater distances and consume more gasoline per capita.  Due to these limitations, county population was not selected as the general methodology for allocating State-level gasoline consumption to the county level.

3.3.2	Vehicle Miles Traveled (VMT)

Another methodology considered for allocating State-level gasoline sales to the county level was based on VMT.  The Highway Performance Monitoring System (HPMS) was investigated as a source of county-level VMT.  The Federal Highway Administration’s Department of Transportation produces HPMS.  It contains data (including VMT) on various types of public roads in the US.  HPMS contains more detailed data on large arterial connector roads and area-wide summary information for urbanized, small urban, and rural areas.  PES contacted the Office of Highway Policy Information (OHPI) to determine if county- level VMT data were available.  OHPI indicated that HPMS contains detailed VMT data for only major highways; however, total county-level VMT was not available in HPMS.  VMT can be easily calculated for 7 of the 12 highway types by multiplying the length of roadway and annual average daily traffic (AADT) provided in HPMS.  However for the other 5 highway types, HPMS does not provide AADT, therefore VMT cannot be calculated.

PES also reviewed the document entitled “Documentation for the Draft 1999 National Emissions Inventory for Criteria Air Pollutants – Onroad Sources”, dated October 2001.  The document describes the methodology used for developing criteria emission estimates for onroad vehicles (e.g., cars, trucks, motorcycles, etc.) using the MOBILE model.  The methodology required that VMT be estimated for each county.  For that study, county-level VMT was calculated based on interstate highway mileage for rural interstates.  County-level VMT for other roadway types was based on county population.  The VMT in MOBILE was consequently not used to allocate gasoline consumption because VMT for almost half of the roadway types was allocated to the county level based on population.

Note:  Delaware, Maryland, and Washington provided PES county-level VMT.  PES used the VMT data provided by these States for allocating their State-Level gasoline consumption to the county level.

3.3.3	Service Station Sales

Another methodology considered for allocating State-level gasoline sales to the county level was based on service station sales.  The U.S Census Bureau publishes a national Economic Census every 5 years.  The last Economic Census was published for calendar year 1997 and provides detailed statistics for various sectors such as the Retail Trade Sector, which includes sales data from gasoline service stations.  The Economic Census data for the gasoline service station subsector was available at the county level.  This data was queried using the Census Bureau’s American FactFinder at http://factfinder.census.gov.  Using the American FactFinder, PES queried and downloaded 1997 county- level sales data for the gasoline service station subsector.  It is important to note that the sales data included not only gasoline sales but also revenue from the sales of other fuels (e.g., diesel fuel and gasohol), sales from convenience store items, sales from automotive repair services, etc.  PES contacted an analyst at American FactFinder to determine if the Economic Census data could be queried for only gasoline sales data by county.  According to the analyst, sales data for only gasoline was available at Metropolitan Statistical Areas (MSA) level, but not at the county level.

It is also important to note that 1997 Census of Retail Trade did not disclose service station sales data for approximately 6 percent of the counties in the U.S.  These counties had only a few service stations and the Census Bureau withheld their sales information to avoid disclosing sales data about individual service stations or companies.  However, in most cases, complete sales data for each State was available at the State level, therefore PES subtracted the sum of the county-level sales data from the State-level sales data to determine the sales not reported for those few counties.  The calculated undisclosed sales were allocated to the counties according to their population.  This methodology was selected to allocate the State-level gasoline sales to the county level for all States except Delaware, Maryland, and Washington.

Note:  The general consensus from the Area Source EIIP committee was that allocating gasoline consumption based on VMT was the preferred methodology.  However county-level data was not readily available for all States.  Delaware, Maryland, and Washington provided PES with county-level VMT data.  PES allocated State-level gasoline consumption using VMT and service station sales (see Section 3.3.3).  PES compared the two methodologies and found that they provided similar results. As a result, service station sales was used to allocate the State-level gasoline consumption to the county level.

3.4	VOC Emission Factors/Methodology

3.4.1	Stage I Loading Operations

PES calculated uncontrolled loading loss emissions resulting from Stage I operations using the methodology outlined in Section 5.2 (dated January 1995) of EPA’s AP-42.  Specifically, PES used the following equation:

LL= 12.46×SPMT

where:

LL	=	Loading loss (uncontrolled), pounds per 1000 gallons (lb/103 gal) of liquid loaded

S	=	A saturation factor (see Table below)

P	=	True Vapor pressure of liquid loaded, pounds per square inch absolute (psia) (calculated using Figure 7.1-14b of AP-42 (dated September 1997))

M	=	Molecular weight of vapors, pounds per pound-mole (lb/lb-mole)

T	=	Temperature of bulk liquid loaded, R (F + 460)

The assumptions and or values used for the variables in the above equation are discussed below.

Saturation Factor (S) - As discussed in EPA’s AP-42, the saturation factor, S, represents the expelled vapors fractional approach to saturation and accounts for the variations observed in emission rates from the different unloading and loading methods.  Section 5.2 of AP-42 provides suggested saturation factors for the various cargo tank loading methods.  The following saturation factors are listed in EPA’s AP-42 and were used by PES to calculate loading loss emissions:

| Mode of Operation | S Factor |
| --- | --- |
| Submerged loading with no vapor balance | 0.6 |
| Submerged loading with vapor balance | 1.00 |
| Splash loading no vapor balance | 1.45 |

Note:	There is not an SCC for splash loading with vapor balance.  Consequently, PES assumed that all splash loading operations were uncontrolled (i.e., not equipped with vapor balancing).  Also, PES assumed that where vapor balancing was required that submerged fill was also present (S=1.00).

True Vapor Pressure (P) - PES used the formula in Figure 7.1-14b (dated September 1997) of AP-42 to calculate the true vapor pressure of the gasoline.  The formula is as follows:

P=exp0.7553−413.0T+459.6S0.5log10(RVP)−1.854−1,042T+459.6S0.5

where:

P	=	Stock true vapor pressure, in pounds per square inch absolute.

T	=	Stock temperature, in degrees Fahrenheit (assumed the same as bulk liquid temperature described below).

RVP	=	Reid vapor pressure, in pounds per square inch (Note: RVP is a standard industry measure of fuel volatility and represents the vapor pressure of the fuel at 100F [Source:  Section 3.2.1.2, Gasoline Distribution Industry (Stage I) – Background Information for Proposed Standards,  EPA-453/R-94-002a, January 1994].  PES used the same monthly RVP values which were used to calculate the 1999 onroad emission estimates using the MOBILE 6 model.)

S	=	Slope of the ASTM distillation curve at 10 percent evaporated, in degrees Fahrenheit per percent.  PES assumed that S = 3.0 for gasoline per Figure 7.1-14a of AP-42)

Molecular Weight (M) - For molecular weight, PES referred to Table 7.1-2 of AP-42.  As shown in the table, molecular weight of gasoline varies from 62-68 lb/lb-mole depending on the RVP of the gasoline.  For the purposes of estimating molecular weight, PES used the same RVP values used to calculate 1999 onroad emission estimates using the MOBILE 6 model.

Bulk Liquid Temperature (T) - PES searched for recent data documenting bulk liquid temperatures in underground storage tanks.  Texas provided temperature data for a few cities.  Radian Corporation conducted a more comprehensive study in 1976 for the American Petroleum Institute.  The report from that study entitled “Summary and Analysis of Data from Gasoline Temperature Survey conducted at Service Station by the American Petroleum Institute” provided gasoline fuel temperature data collected from 56 service stations across the U.S. during all seasons of the year.  The data was arranged by six arbitrarily defined regions and is summarized in Table C-1.  PES used this data as the basis for its Stage I calculations.  Note in Table C-1 that there were some gaps in the temperature data (e.g., Region 4’s January data).  In such cases, PES added reasonable temperature values to fill in the gaps.  There were also a few instances where the API data did not appear reasonable (e.g., Region 2’s, April data).  As a result, PES adjusted some of the data as shown in Table C-1.

###### Stage I Control Efficiency - Stage I loading losses can be controlled using a vapor balance system (i.e., a hose that allows gasoline vapors from the UST to be displaced back to the cargo tank during UST loading operations.).  Most State regulations require that the vapor balancing system be 90-95% efficient in controlling this vapor transfer process.  To account for controlled Stage I loading, PES multiplied the uncontrolled loading losses calculated using the equation above by the overall reduction efficiency term:

1 − Control Efficiency100

where,

Control Efficiency is the efficiency specified in the respective State regulation (typically 90 or 95%).

3.4.2	Stage II Loading Operations

PES’ subcontractor, PECHAN, used the MOBILE 6 model to calculate refueling emission factors which represented refueling emissions occurring from both displacement and spillage.  The general methodology used to calculate Stage II emissions using MOBILE is described below.

- Approximately 160 MOBILE input files were developed to represent controlled and uncontrolled refueling scenarios for all counties in the U.S.  Corresponding output files were generated which provided monthly emission factors (grams VOC/mile) as well as fuel economy (miles per gallon) and VMT mix for 14 different gasoline vehicle types (e.g., LDGV, LDGT, and HDGV).  The same monthly temperature and RVP data was included in the input files as was used to generate the 1999 onroad emission estimates.

- For each vehicle type, the monthly emission factor was multiplied by the fuel economy to convert the emission factor to “grams VOC/ gallon”.

grams/mile  x  mile/gallon  =  grams VOC /gallon

- The VMT mix for the 14 vehicle types was used to calculate a single weighted monthly emission factor  (grams VOC/gallon)

- The weighted emission factor was converted to lbs/1000 gallons

grams VOC/gallon  x  lb/453.59 grams  x  1000/1000 = lbs VOC/1000 gallons

- The spillage emission factor (0.68 lbs VOC/1000 gallons) was subtracted from the weighted emission factor to calculate the controlled or uncontrolled Stage II displacement emission factor.

Note:  MOBILE assumes a constant spillage factor of approximately 0.31 grams VOC/gallon which is equivalent to 0.68 lbs VOC/1000 gallons

- The monthly Stage II displacement and spillage emission factors were multiplied by the monthly county throughput (1000gallons) to estimate VOC emissions.

Emission factor (lbs VOC/1000 gallons)  x  Throughput (1000 gallons)

Note:  For the States with both uncontrolled and controlled Stage II operations, PES used controlled and uncontrolled emission factors and throughputs to calculate emissions.

3.4.3	UST Breathing & Emptying Operations

To estimate UST B&E emissions, PES evaluated two options.  One option was to use the

factor of 1.0 lb/1000 gal loaded found in Section 5.2 of AP-42.  This factor was used during the development of the Stage I Gasoline Distribution NESHAP (EPA-450/3-94-002a) to calculate baseline emissions.  This factor was also used to estimate baseline emissions in EPA’s Stage II Technical Guidance document (EPA-450/3-91-022a).  Both documents state that the original source for the factor was a November 1963 Journal of the Air Pollution Control Association article about a study by the Los Angeles Air Pollution Control District (LAAPCD).  The documents also cite other studies, which attempted to estimate breathing losses.  The various studies were conducted in the 1970’s and 1980’s and listed UST B&E factors ranging from 0.21 to 2.3 lbs/1000 gal loaded.

The other option was to assume that USTs have no breathing losses, since the insulation of the earth limits the diurnal temperature changes. This assumption is consistent with Section 7.1 of AP-42, Organic Liquid Storage Tanks (see p. 7.1-11) and with emission results from the TANKS software.

Note:  There is an inconsistency in AP-42 in the handling of breathing and emptying emissions.  Section 5.2 of AP-42 lists the factor of 1.0 lb/1000 gallons loaded while Section 7.2 (and TANKS) assumes that breathing and emptying emissions from USTs are zero.  In addition, there is an apparent inconsistency in terminology regarding the types of losses from storage tanks.  For example, Table 5.2-7 in AP-42 refers to a “breathing and emptying” factor.  However the text in Section 5.2.2.2 of AP-42 refers only to “breathing” emissions.  Section 7.1.3.1 of AP-42 and TANKS refers to “standing storage losses” and “working losses” when discussing losses from fixed roof tanks (USTs are treated as an underground fixed roof tank).  Standing storage losses (Section 7.1.3.1 of AP-42) and breathing and emptying losses (Section 5.2.2.2 of AP-42) both refer to the tank losses attributable to daily changes in temperature and barometric pressure.

For this analysis, PES opted to assume that UST B&E emissions are 1.0 lb/1000 gallons loaded.  Using the AP-42 emission factor and the 1999 DOE throughput, nationwide breathing and emptying losses are approximately 66,500 tons of VOC compared to breathing losses reported in the Area Source NEI (47,000 tons of VOC) and in the Point Source NEI (1,300 tons of VOC).

3.5	Regulatory Review

PES reviewed State and local air regulations to identify those states which have Stage I and/or Stage II control requirements.  To assist in its regulatory search, PES used an commercially available online product called ENFLEX (http://www.enflex.com).  ENFLEX’ databases are updated at least monthly so PES had access to the most current air regulations for each State.  The results of the regulatory review are summarized below.

3.5.1	Stage I

PES specifically searched for Stage I regulations to determine control level requirements (e.g., submerged loading and/or vapor balancing requirements), the date such regulations began, and the area and type of facility (e.g., minimum throughput size) requiring Stage I controls.  The results of the Stage I regulation searches are summarized by county in Table B-1.

After researching the regulations, PES concluded that nine states do not have Stage I regulations (i.e., Alaska, Idaho, Iowa, Kansas, Mississippi, Nebraska, South Carolina, South Dakota, and Wyoming).  As a result, PES will assume that these all counties in these states load gasoline using only the “splash fill” method and will use the saturation factor (S) of 1.45 in the calculation of emissions.

Most of the remaining States have statewide Stage I regulations that require submerged loading and vapor balancing.  PES assumed an S factor of 1.0 for these states when calculating Stage I emissions.  A few states required submerged loading but not vapor balancing (i.e., Arizona, Michigan, Minnesota, Montana, Nevada, North Dakota, and parts of Virginia.  PES assumed an S factor of 0.6 for submerged loading operations, which are not vapor balanced.

PES assumed that facilities which required submerged loading or vapor balancing (but not both) would opt for installing submerged loading only (S=0.6).  Arizona and Hawaii were the only States, which provided the facility an option of Stage I controls.

Parts of California and Maryland specifically required vapor balancing but not submerged loading.  In this situation, PES assumed that facilities have both submerged fill and vapor balance (S=1.0).

Regarding the control efficiency of Stage I, states either did not specify a control efficiency, specified 90% control, or specified 95% control.  If a State did not specify the control efficiency for Stage I, PES assumed a control efficiency of 90%.  Otherwise, PES used the control efficiency cited in the regulations (e.g., 90% or 95%).

Also note that for most states, the Stage I controls only apply to facilities that either exceed a throughput limit (e.g., 10,000 gallons per month) or have tanks storing gasoline which exceed a size limit (e.g., 250 gallons).  The impact of such provisions on the emission calculations is discussed in the section entitled “Rule Penetration.”

3.5.2	Stage II

Section 182(b) of the Clean Air Amendment (CAAA) of 1990 requires the installation of Stage II vapor recovery systems in areas that are designated as moderate, serious, severe, and extreme for ozone nonattainment.  As a result, Stage II controls are less prevalent than Stage I controls.  PES specifically searched Stage II regulations for data input needed to run the MOBILE model.  The following information was gathered from the analysis of the Stage II regulations:

- Counties requiring Stage II controls (i.e., vapor balancing between the UST and vehicle fuel tank),

- Calendar year in which Stage II began,

- Number of years for phase-in of Stage II controls,

- Percent efficiency for light duty gasoline vehicles (LDGV) and light duty gasoline trucks (LDGT), and

- Percent efficiency for heavy duty gasoline vehicles (HDGV).

Note:  LDGV represents passenger cars; LDGT represents pickup trucks, vans and other small trucks that have a gross vehicle weight of 0-8500 lbs; and HDGV represents all vehicles with a gross vehicle weight greater than 8,500 lbs, powered by gasoline.

Table B-1 summarizes the results of PES’ regulatory search.  Most states require 95% control for Stage II.  State regulations do not distinguish between control efficiencies for LDGV, LDGT, and HDGV. For the purposes of this effort, PES assumed the same Stage II control efficiency for HDGV as is assumed for LDGV.

As shown in Table B-1, 23 states do not have Stage II requirements (i.e., Alabama, Alaska, Arkansas, Colorado, Hawaii, Idaho, Iowa, Kansas, Michigan, Minnesota, Mississippi, Montana, Nebraska, Nevada, New Mexico, North Carolina, North Dakota, Oklahoma, South Carolina, South Dakota, Utah, West Virginia, and Wyoming).  California, Connecticut, Delaware, Massachusetts, New Jersey, and Vermont require Stage II statewide.  Washington D.C. also has Stage II requirements.  The remaining states require Stage II in only counties or towns designated as ozone nonattainment areas or VOC control areas.

Similar to the Stage I regulations, most states’ Stage II controls apply to facilities that exceed a throughput limit (e.g., 10,000 gallons per month).  Therefore even if Stage II is required for the entire state, existing service stations which do not exceed the throughput threshold are exempt from Stage II requirements.  PES’ methodology for estimating the impact of such provisions on the emission calculations is discussed in the next section entitled “Rule Penetration.”

###### 3.6	Rule Penetration

Rule penetration can be described as the percent of sources covered by a rule, or in this case, the percent of gasoline throughput covered by a rule.  As stated earlier, most State’s Stage I regulations apply to facilities exceeding a specified throughput or tank size threshold.  For example in Alabama, submerged loading and vapor balancing only apply to facilities in the counties listed whose monthly throughput exceed 4,000s gallons per month and for tanks greater than 3,000 gallons.  Stage II regulations have similar throughput and/or tank size applicability thresholds.  For example, Delaware requires Stage II statewide but only at facilities whose gasoline throughput exceeds 10,000 gallons per month.  As a result, it is reasonable to assume that facilities who do not exceed throughput limit do not have Stage II controls.

To account for the effects of the throughput thresholds, PES located information regarding the size (or consumption) distribution of retail service stations (see Table 5).  If a State has a 10,000 gallon per month threshold, PES estimates that approximately 91.3% of the throughput is affected by the regulation.  One would expect that less throughput would be affected by a States’ with a higher applicability threshold.  For example, Texas’ Stage II regulations only apply to facilities whose throughput is greater than 100,000 gallons per month.  Using Table 5, the amount of gasoline throughput affected by Texas’ Stage II regulations is 18.8 percent.

Note:  PES did not have information regarding tank size distribution; therefore the rule penetration for regulations with tank size thresholds (e.g., greater than 250 gallons) was ignored (i.e., PES assumed 100% rule penetration).

To estimate the portion of fuel that was loaded in a controlled versus uncontrolled scenario, PES multiplied the total gasoline throughput by the percent of throughput expected to be controlled (see Table 5).  For example, Delaware has a 1999 throughput of 438,621,000 gallons (according to DOE) and requires Stage II statewide at facilities with throughputs exceeding 10,000 gallons per month.  According to Table 5, PES estimated that 91.3% of Delaware’s annual throughput is dispensed from stations that have Stage II controls).  PES assumed that the remaining 8.7 percent of Delaware’s 1999 throughput was dispensed at service stations which did not utilize Stage II controls.

##### 3.7	Hazardous Air Pollutant (HAP) Profiles

PES estimated HAP emissions from Stage I and Stage II operations based on average HAP contents for normal, reformulated and oxygenated gasoline.  The average HAP contents for these types of gasoline are listed in Table 6. The HAP profiles in Table 6 were obtained from the EPA document entitled “Documentation for the 1999 Base Year Nonpoint Source National Emission Inventory for Hazardous Air Pollutants,” dated September 28, 2001.  The HAP content is expressed as ratios by weight of HAP to total VOC.  PES multiplied the HAP ratios by the estimated VOC emissions to obtain the estimated HAP emitting from Stage I and Stage II operations.  The three types of gasoline are discussed below.

Normal Gasoline – As shown in Table 6, normal gasoline contains seven HAPs.  PES will use the HAP –to-VOC ratio for all Stage I and Stage II emissions generated in counties not using reformulated and/or oxygenated fuels.

Reformulated Gasoline - Section 211(k) of the 1990 Clean Air Act Amendments (CAAA) directs EPA to issue regulations that require some gasoline to be "reformulated" in order to burn cleaner and reduce smog (low-level ozone) and toxic air pollutants.  Reformulated gasoline (RFG) must have at least 2% oxygen by weight.  MTBE (methyl tertiary butyl ether), which is a HAP, and ethanol are the two most common oxygenates used in gasoline.  EPA has implemented the Reformulated Gasoline (RFG) program in two phases.  The Phase I program ran from 1995 through 1999, and the Phase II RFG program (similar to Phase I program) but is designed to further reduce emissions) began in 2000.  The CAAA requires RFG in the most severe ozone non-attainment areas of the country. However, other areas with ozone problems have voluntarily opted into the RFG program.  EPA’s Office of Transportation and Air Quality (OTAQ) provides a list of areas participating in the federal reformulated gasoline program on its Website (http://www.epa.gov/oms/regs/fuels/rfg/rfgarea.pdf).  Table D-1 lists 160 counties from 16 states and the District of Columbia which were modeled using the HAP-to-VOC ratios for RFG.

Oxygenated Gasoline – The 1990 CAAA also requires an oxygenated fuel program for areas exceeding the federal carbon monoxide (CO) air quality standards.  The oxygenated gasoline program requires that gasoline dispensed in CO nonattainment areas in the winter months meet a minimum oxygen content (2.7% by weight) to reduce the CO emissions from vehicles.  Table D-2 list 59 counties which were modeled using the HAP-to-VOC ratios for oxygenated gasoline as well as the MTBE and ethanol market shares for those counties and the winter months when oxygenated gasoline is used.

3.8	Calculations

3.8.1 	VOC Calculations.

A was developed using the procedures and assumptions in this document to calculate nationwide VOC emissions resulting from Stage I and Stage II service station operations at both the State and county level.  Twelve spreadsheets (one for each month) was developed using monthly temperature, RVP, and gasoline throughput data.  Each spreadsheet contains all VOC Stage I and Stage II emission estimates for all counties in the U.S. as well a summation of county emission by State.  An additional spreadsheet was added to calculate annual emission estimates by summing the monthly emission estimates.

3.8.2	HAP Calculations.

PES developed a Microsoft Access database to calculate HAP emissions.  The database contains a VOC emissions table which contains monthly VOC emissions estimates by county, month, and process as well as the MTBE share (developed from the Stage I VOC Microsoft Excel workbook (Section 3.8.1)).  The database also contains the HAP profiles for each of the fuel types (Table 6).  A query was developed which multiplies the VOC emissions by the appropriate HAP profile (i.e., normal, reformulated, or oxygenated gasoline).  Except for MTBE, all HAP calculations are simply a multiplication of the VOC emissions by the HAP weight percentage for each pollutant.  MTBE emissions are calculated by multiply VOC emissions by the MTBE weight percentage by the MTBE share.

3.9	Results Compared to NEI

Table E-1 provides a comparison of the Stage I and Stage II VOC emissions calculated by PES using the methodology described above and those emissions reported in the NEI.  In the NEI, many States reported emissions using a “Total” SCC (i.e., Stage I, Stage II, and All Processes).  Based on the regulatory analysis, PES has allocated its emissions to specific uncontrolled and controlled processes.  A county-level comparison of the Stage I and Stage II VOC emissions is provided in a spreadsheet located on the EFIG Website.

Note:  Several States emissions are reported under the "Service Station Total" SCC (250106000) which may include both Stage I and Stage II emissions. As a result, the emissions reported using SCC 250106000 are not listed in Table E-1.  The States reporting under 250106000 are Georgia (5,009 tons), Maryland (994 tons), Missouri (472 tons), Ohio (11,977 tons), and Washington (3,252 tons).

###### 4.0		CONCLUSIONS/RECOMMENDATIONS

Based on the analysis/QA of the Area and Point Source NEI for gasoline service stations, PES conclude2 the following:

- There are considerable data gaps in both the Area and Point Source NEI’s.  For example some States are not represented in the NEI.  Also many States are missing records containing throughput, emission factors, and emissions.

- For records that could be checked, it appears that throughput data may be in error for most of the states when compared to DOE estimates.  Also equal throughput should be attributed to Stage I and Stage II operations, as well as UST B&E; however, for most States that was not the case.  Also throughput units were sometimes inconsistent or wrong.

- For states that provided emission factors, many appear to be in error in the Point Source NEI because the range for a particular process varies by several orders of magnitude.

- The QA of the emission records was probably the most important because these are provided to other EPA groups for running various computer models.  PES determined that three states did not report emissions. Also, many states that reported emissions did not associate emissions with a meaningful SCC (e.g. a controlled or uncontrolled process).

After reviewing the 1999 NEI and identifying the data gaps and inconsistencies in the database, PES developed recommendations that it believes will simplify the NEI reporting process, promote consistency, and improve the overall data quality of the NEI records associated with gasoline service stations.  These recommendations are listed below and it is PES’ intent to implement these recommendations when developing its independent emissions estimate for Stage I and Stage II operations which it will compare to the reported NEI emission estimates.

- Use SCC’s that provide meaningful process information.  Avoid using SCC’s that combine several different processes (e.g., 2501060000, 2501060050, 2501060100, 2501060200, 40600305, 40600399, and 40600499).  This recommendation applies to records for gasoline throughput, emission factors, and emissions.

- Discontinue the use of either 40600401 (Stage II vapor loss without control-pumped) or 40600403 (Stage II vapor loss without control-transferred).  These SCC’s appear to cover the same process.

- Use DOE gasoline consumption data for reporting gasoline throughput for Stage I, Stage II, and UST B&E operations.

- Ensure that annual gasoline throughput is entered in appropriate units as thousands of gallons (e.g., E3GAL).

- Use emission factors that are consistent with EPA average factors.  Also enter the emission factors in units of “lbs/1000 gal.”

- Report emissions in “tons”.

- In the Point Source NEI, discontinue allocating gasoline throughput and emissions to SCC’s associated with “fleet operations.”  The amount of emissions allocated to fleet operations in the 1999 Point Source NEI is negligible (less than 0.1% of total emissions).

## Table 1.  List of SCCs Used in Analyzing

## the 1999 Point and Area Source Inventory for Service Stations

| Point Source SCC* | ### SCC Description |
| --- | --- |
| 40600301 | Gasoline Retail Ops - Stage I/Splash fill |  |  |
| 40600302 | Gasoline Retail Ops - Stage I/Submerged filling w/o controls |  |  |
| 40600305 | Gasoline Retail Ops - Stage I/Unloading ** |  |  |
| 40600306 | Gasoline Retail Ops - Stage I/Balanced Submerged Fill |  |  |
| 40600307 | Gasoline Retail Ops - Stage I/UST Breathing & Emptying |  |  |
| 40600399 | Gasoline Retail Ops - Stage I/Not Classified |  |  |
| 40600401 | Filling Vehicle Gas Tanks - Stage II/Vapor Loss w/o controls (pumped) |  |  |
| 40600402 | Filling Vehicle Gas Tanks - Stage II/Liquid Loss w/o controls |  |  |
| 40600403 | Filling Vehicle Gas Tanks - Stage II/Vapor Loss w/o controls (transferred) |  |  |
| 40600499 | Filling Vehicle Gas Tanks - Stage II/Not Classified ** |  |  |
| 40600601 | Consumer (Corporate) Fleet Refueling - Stage II/Vapor Loss w/o controls |  |  |
| 40600602 | Consumer (Corporate) Fleet Refueling - Stage II/Liquid Loss w/o controls |  |  |
| 40600603 | Consumer (Corporate) Fleet Refueling - Stage II/Vapor Loss w/ controls |  |  |
| 40600701 | Consumer (Corporate) Fleet Refueling - Stage I/Splash Filling |  |  |
| 40600702 | Consumer (Corporate) Fleet Refueling - Stage I/Submerged Filling w/o controls |  |  |
| 40600706 | Consumer (Corporate) Fleet Refueling - Stage I/Balanced Submerged fill |  |  |
| 40600707 | Consumer (Corporate) Fleet Refueling - Stage I/UST Breathing & Emptying |  |  |
| Area Source SCC | ### SCC Description |
| 2501060000 | Gasoline Service Stations/Total: All Gasoline/All Processes |  |  |
| 2501060050 | Gasoline Service Stations/Stage 1: Total |  |  |
| 2501060051 | Gasoline Service Stations/Stage 1: Submerged Filling |  |  |
| 2501060052 | Gasoline Service Stations/Stage 1: Splash Filling |  |  |
| 2501060053 | Gasoline Service Stations/'Stage 1: Balanced Submerged Filling |  |  |
| 2501060100 | Gasoline Service Stations/Stage 2: Total |  |  |
| 2501060101 | Gasoline Service Stations/Stage 2: Displacement Loss/Uncontrolled |  |  |
| 2501060102 | Gasoline Service Stations/Stage 2: Displacement Loss/Controlled |  |  |
| 2501060103 | Gasoline Service Stations/Stage 2: Spillage |  |  |
| 2501060200 | Gasoline Service Stations/Underground Tank: Total |  |  |
| 2501060201 | Gasoline Service Stations/Underground Tank: Breathing and Emptying |  |  |

## *	When analyzing the Point Source NEI, PES did not consider gasoline tanks associated with the following SCC’s (Non-Refinery): 40400401, 40400402, 40400403, 40400404, 40400405, and 40400406.

## Table 2.  1999 DOE Reported Gasoline Throughput and

## State-Reported Gasoline Throughputs (1,000 gallons)

|  |  | Area Source |  |  |  | Point Source |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| State | Gasoline Consumption 1 | All | Stage I | Stage II | UST | Stage I | Stage II | UST |
| Alabama | 2,273,658 | 0 | 0 | 0 | 2,395,486 | 0 | 0 | 0 |
| Alaska | 286,306 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Arizona | 2,063,528 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Arkansas | 1,374,006 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| California | 14,490,099 | 0 | 0 | 0 | 0 | 11,719 | 14,261 | 2,457 |
| Colorado | 1,988,739 | 0 | 0 | 0 | 0 | 4,715,151 | 3,126,539 | 1,565,801 |
| Connecticut | 1,394,264 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| District of Columbia | 124,903 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Delaware | 438,621 | 0 | 361,230 | 3,945,589 | 361,230 | 0 | 0 | 0 |
| Florida | 7,319,126 | 0 | 0 | 0 | 0 | 2,184 | 0 | 0 |
| Georgia | 4,643,567 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Hawaii | 354,963 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Iowa | 1,559,244 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Idaho | 568,743 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Illinois | 4,624,258 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Indiana | 3,137,175 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Kansas | 1,495,770 | 0 | 0 | 0 | 0 | 0 | 562,848 | 0 |
| Kentucky | 2,140,689 | 0 | 0 | 0 | 0 | 175,525 | 308,522 | 157,192 |
| Louisiana | 2,247,597 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Massachusetts | 2,478,031 | 0 | 2,738,289 | 0 | 2,738,289 | 0 | 0 | 0 |
| Maryland | 2,020,896 | 0 | 2,426,520 | 2,422,764 | 2,426,520 | 2,438 | 1,075 | 0 |
| Maine | 748,323 | 0 | 13,027,755 | 0 | 4,820,500 | 0 | 0 | 0 |
| Michigan | 4,840,594 | 0 | 0 | 0 | 0 | 173,915 | 8,175 | 0 |
| Minnesota | 2,406,299 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Missouri | 2,976,247 | 0 | 3,073,876 | 3,073,876 | 3,073,876 | 2,304 | 4,036 | 0 |
| Mississippi | 1,423,099 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Montana | 526,914 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| North Carolina | 4,218,707 | 0 | 0 | 0 | 0 | 904 | 0 | 0 |
| North Dakota | 379,381 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Nebraska | 841,727 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| New Hampshire | 550,493 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| New Jersey | 3,939,044 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| New Mexico | 914,398 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Nevada | 921,443 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| New York | 5,379,881 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Ohio | 5,238,079 | 0 | 0 | 0 | 0 | 5,710 | 12,576 | 2,869 |
| Oklahoma | 2,207,703 | 0 | 0 | 0 | 0 | 248,797 | 0 | 0 |
| Oregon | 1,558,404 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Pennsylvania | 5,116,315 | 0 | 0 | 0 | 0 | 388 | 1,574 | 0 |
| Rhode Island | 466,178 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| South Carolina | 2,337,095 | 0 | 3,432,154 | 4,954,923 | 2,134,835 | 0 | 0 | 0 |
| South Dakota | 421,794 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Tennessee | 3,616,457 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Texas | 11,629,192 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Utah | 1,086,897 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Virginia | 3,787,021 | 0 | 0 | 0 | 0 | 17,172 | 29,811 | 12,869 |
| Vermont | 297,804 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Washington | 2,837,072 | 1,436,000 | 0 | 0 | 0 | 1,382 | 21,318 | 1,382 |
| Wisconsin | 2,450,428 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| West Virginia | 615,171 | 0 | 0 | 0 | 0 | 271 | 301 | 48 |
| Wyoming | 309,119 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Total | 131,065,450 | 1,436,000 | 25,059,824 | 14,397,152 | 17,950,736 | 5,357,860 | 4,091,036 | 1,742,618 |

###### 1	Gasoline Consumption obtained from http://www.eia.doe.gov/emeu/states/_states.html

## Table 3.  VOC Emission Factors Listed in the 1999 Area and Point Source NEI for Gasoline Service Stations

|  | Area Source Inventorya |  |  |  |  | Point Source Inventoryb |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Process | SCC | VOC EF | VOC EF Unit | # of States | # of Counties | SCC | VOC EF | VOC EF Unit | # of States | # of Counties | Average EPA Factors (lbs/103 gal) |
| All Gasoline/ All Processes | 2501060000 | NR | NR | NR | NR | --- |  |  |  |  | --- |
| Stage I Total | 2501060050 | 0.37 | Lb/103 gal | 1 | 9 | 40600399 | 0.01–409.99 | lb/103 gal | 8 | 27 | --- |
|  |  | 0.54 | 103 gal/ton | 1 | 2 |  |  |  |  |  |  |
|  |  | 7.42 | 103 gal/ton | 1 | 27 |  |  |  |  |  |  |
|  |  | 11.37 | Lb/103 gal | 1 | 107 |  |  |  |  |  |  |
|  |  | 30 | Lb/103 gal | 1 | 45 |  |  |  |  |  |  |
| Stage I Unloading | --- |  |  |  |  | 40600305 | 0.01-239.52 | lb/103 gal | 6 | 11 | --- |
| Stage I: Submerged Fill w/o controls | 2501060051 | 7.3 | Lb/103 gal | 2 | 95 | 40600302 (Retail) | 0.01-900 | lb/103 gal | 9 | 84 | 7.3 |
|  |  | 12 | Lb/103 gal | 1 | 3 | 40600702 (Fleet) | 0.01-148 | lb/103 gal | 4 | 8 |  |
| Stage I: Splash Fill | 2501060052 | 11.5 | Lb/103 gal | 1 | 81 | 40600301 (Retail) | 0.01-2000 | lb/103 gal | 9 | 49 | 11.5 |
|  |  |  |  |  |  | 40600701 (Fleet) | 0.01-388 | lb/103 gal | 7 | 8 |  |
| Stage I: Balanced/ Submerged Fill | 2501060053 | 0.3 | Lb/103 gal | 1 | 51 | 40600306 (Retail) | 0.01-2000 | lb/103 gal | 9 | 106 | 0.3 |
|  |  |  |  |  |  | 40600706 (Fleet) | 0.01-833.33 | lb/103 gal | 7 | 17 |  |
| Stage II Total | 2501060100 | 1.1 | Lb/103 gal | 1 | 5 | 40600499 |  |  | 6 | 17 | --- |
|  |  | 6.76 | Lb/103 gal | 1 | 3 |  |  |  |  |  |  |
|  |  | 8.16 | Lb/103 gal | 1 | 29 |  |  |  |  |  |  |
|  |  | 11 | Lb/103 gal | 1 | 107 |  |  |  |  |  |  |
|  |  | 303.95 | G/gal | 1 | 4 |  |  |  |  |  |  |
|  |  | 304.07 | G/gal | 1 | 13 |  |  |  |  |  |  |
|  |  | 304.12 | G/gal | 1 | 4 |  |  |  |  |  |  |
|  |  | 304.14 | G/gal | 1 | 6 |  |  |  |  |  |  |
|  |  | 304.19 | G/gal | 1 | 14 |  |  |  |  |  |  |
|  |  | 304.22 | G/gal | 1 | 4 |  |  |  |  |  | --- |

Table 3.  VOC Emission Factors Listed in the 1999 Area and Point Source NEI for Gasoline Service Stations, continued

|  | Area Source Inventorya |  |  |  |  | Point Source Inventoryb |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Process | SCC | VOC EF | VOC EF Unit | # of States | # of Counties | SCC | VOC EF | VOC EF Unit | # of States | # of Counties | Average EPA Factors (lbs/103 gal) |
| Stage II: Displacement Loss – Uncontrolled | 2501060101 | 2 | G/gal | 1 | 1 | 40600401 (Retail) | 0.01-409.99 | lb/103 gal | 9 | 117 | 11.0 |
|  |  | 3 | G/gal | 1 | 1 | 40600601 (Fleet) | 0.01-900 | lb/103 gal | 0 | 0 |  |
| Stage II: Displacement Loss – Controlled | 2501060102 | NR | NR | NR | NR | 40600403 (Retail) | 0.01-2000 | lb/103 gal | 8 | 52 | 1.1 |
|  |  |  |  |  |  | 40600603 (Fleet) | 0.01-409.00 | lb/103 gal | 6 | 13 |  |
| Stage II: Spillage | 2501060103 | NR | NR | NR | NR | 40600402 (Retail) | 0.01-2000 | lb/103 gal | 9 | 111 | 0.7 |
|  |  |  |  |  |  | 40600602 (Fleet) | 0.07-382 | lb/103 gal | 4 | 5 |  |
| UST: Total | 2501060200 | NR | NR | NR | NR | --- | --- | --- | --- | --- | --- |
| UST: Breathing and Emptying | 2501060201 | 1 | lb/103 gal | 5 | 281 | 40600307 (Retail) | 0.01-2000 | lb/103 gal | 9 | 114 | 1.0 |
|  |  | 1.12 | 103 gal/ton | 1 | 29 | 40600707 (Fleet) | 0.01-379.5 | lb/103 gal | 3 | 5 |  |
|  |  | 301 | lb/103 gal | 1 | 44 |  |  |  |  |  |  |
|  |  | 3000 | Each/ton | 1 | 29 |  |  |  |  |  |  |

a – Seven States reported emission factors in the 1999 Area source NEI

b – Nine States reported emission factors in the 1999 Point Source NEI

UST – 	Underground Storage Tanks

NR – 	Not Reported

# Table 4.  Gasoline Throughput and Emissions Reported in the 1999 NEI’s

| State (FIPS) | 1999 Gasoline Throughput (1,000 gals) | Total VOC Emission Reported in 1999 NEI's (tons) | Percentage of Emissions in the Point Source NEI |
| --- | --- | --- | --- |
| Alabama (01) | 2,273,658 | 14,418 | 0.05% |
| Alaska (02) | 286,306 | NR | --- |
| Arizona (04) | 2,063,528 | 18,805 | 0.10% |
| Arkansas (05) | 1,374,006 | 14,001 | 0.00% |
| California (06) | 14,490,099 | 30,853 | 3.8% |
| Colorado (08) | 1,988,739 | 24,522 | 43.5% |
| Connecticut (09) | 1,394,264 | 2,708 | 0.00% |
| Delaware (10) | 438,621 | 1,166 | 0.00% |
| District of Columbia (11) | 124,903 | 227 | 0.00% |
| Florida (12) | 7,319,126 | 49,018 | 0.05% |
| Georgia (13) | 4,643,567 | 23,757 | 0.00% |
| Hawaii (15) | 354,963 | NR | --- |
| Idaho (16) | 568,743 | 4,976 | 0.00% |
| Illinois (17) | 4,624,258 | 42,649 | 0.20% |
| Indiana (18) | 3,137,175 | 25,591 | 0.02% |
| Iowa (19) | 1,559,244 | 12,807 | 0.00% |
| Kansas (20) | 1,495,770 | 12,603 | 0.14% |
| Kentucky (21) | 2,140,689 | 18,632 | 2.4% |
| Louisiana (22) | 2,247,597 | 16,719 | 0.51% |
| Maine (23) | 748,323 | 5,288 | 0.00% |
| Maryland (24) | 2,020,896 | 7,208 | 0.44% |
| Massachusetts (25) | 2,478,031 | 16,734 | 0.00% |
| Michigan (26) | 4,840,594 | 41,184 | 0.05% |
| Minnesota (27) | 2,406,299 | 19,548 | 0.00% |
| Mississippi (28) | 1,423,099 | 13,823 | 0.00% |
| Missouri (29) | 2,976,247 | 28,470 | 0.01% |
| Montana (30) | 526,914 | 4,588 | 0.00% |
| Nebraska (31) | 841,727 | 7,411 | 0.00% |
| Nevada (32) | 921,443 | 6,341 | 0.00% |
| New Hampshire (33) | 550,493 | 6,533 | 0.00% |
| New Jersey (34) | 3,939,044 | 4,854 | 4.49% |
| New Mexico (35) | 914,398 | 8,680 | 0.04% |
| New York (36) | 5,379,881 | 45,578 | 0.00% |
| North Carolina (37) | 4,218,707 | 28,607 | 0.01% |
| North Dakota (38) | 379,381 | 3,260 | 0.00% |
| Ohio (39) | 5,238,079 | 29,141 | 0.04% |
| Oklahoma (40) | 2,207,703 | 8,787 | 0.31% |
| Oregon | 1,558,404 | NR | --- |
| Pennsylvania (42) | 5,116,315 | 29,306 | 0.01% |
| Rhode Island (44) | 466,178 | 1,006 | 0.23% |
| South Carolina (45) | 2,337,095 | 23,926 | 0.00% |
| South Dakota (46) | 421,794 | 3,669 | 0.00% |
| Tennessee (47) | 3,616,457 | 34,359 | 0.00% |
| Texas (48) | 11,629,192 | 57,192 | 0.03% |
| Utah (49) | 1,086,897 | 5,025 | 0.13% |
| Vermont (50) | 297,804 | 4,994 | 2.0% |
| Virginia (51) | 3,787,021 | 25,912 | 0.00% |
| Washington (53) | 2,837,072 | 3,290 | 1.2% |
| West Virginia (54) | 615,171 | 9,928 | 3.9% |
| Wisconsin (55) | 2,450,428 | 13,614 | 0.001% |
| Wyoming (56) | 309,119 | 3,080 | 0.00% |
| Total | 131,065,450 | 814,791 | 1.65% |

Table 5.  Nationwide Service Station Consumption Distributiona.

| ## Facility Throughput Range (gallons/Month) | ## Percent Consumption |
| --- | --- |
| ## 0 – 5,999 | ## 4.7 |
| ## 6,000 – 9,999 | ## 4.1 |
| ## 10,000 – 24,999 | ## 17.8 |
| ## 25,000 – 49,999 | ## 27.5 |
| ## 50,000 – 99,999 | ## 27.2 |
| ## > 100,000 | ## 18.8 |
| ## TOTALb | ## 100 |

a	Source: Technical Guidance – Stage II Vapor Recovery Systems for Control of Vehicle Refueling Emissions at Gasoline Dispensing Facilities, Volume I: Chapters, Table 2-10, November 1991, EPA-450/3-91-022a.

b	Total may not sum due to rounding

## TABLE 6.  HAP Vapor Profile For

Various Gasoline Typesa

(Weight % of Total VOC)

|  |  |  | Reformulated |  |  | Winter-Oxygenated |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pollutant |  | Normal |  | W/MTBE | W/Ethanol |  | W/MTBE | W/Ethanol |
| 2,2,4-Trimethylpentane |  | 0.8 |  | 0.7 | 0.7 |  | 0.7 | 0.7 |
| Benzene |  | 0.9 |  | 0.4 | 0.4 |  | 0.7 | 0.7 |
| Ethyl benzene |  | 0.1 |  | 0.1 | 0.1 |  | 0.1 | 0.1 |
| Hexane |  | 1.6 |  | 1.4 | 1.4 |  | 1.4 | 1.4 |
| MTBE |  | 0 |  | 8.7 | 0 |  | 11.9 | 0 |
| POM as 16-PAH |  | 0.5 |  | 0.5 | 0.5 |  | 0.5 | 0.5 |
| Toluene |  | 1.3 |  | 1.1 | 1.1 |  | 1.1 | 1.1 |
| Xylene |  | 0.5 |  | 0.4 | 0.4 |  | 0.4 | 0.4 |

a	Documentation for the 1999 Base Year Nonpoint Source National Emission Inventory for Hazardous Air Pollutants,  U.S. EPA, Office of Air Quality Planning & Standards, Emissions, Monitoring and Analysis Division, Emission Factor and Inventory Group, Research Triangle Park, NC, September 28, 2001.

## APPENDIX A

## DETAILED DATA

## Table A-1.  Gasoline Throughput Reported in the Area Source NEI by State and SCC

(103 Gallons)

| State (FIPS) | 2501060000 (SS Total) | 2501060050 (S1 Total) | 2501060051 (S1 Submerged) | 2501060052 (S1 Splash) | 2501060053 (S1 B/S) | 2501060100 (S2 Total) | 2501060101 (S2 Unc.) | 2501060103 (S2 Spillage) | 2501060201 (UST B&E) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Alabama (01) |  |  |  |  |  |  |  |  | 2,395,486 |
| Delaware (10) |  |  | 361,230 |  |  |  | 3,945,589 | 3,945,589 | 361,230 |
| Maryland (24) |  |  |  |  | 2,426,520 | 2,422,764 |  |  | 2,426,520 |
| Massachusetts (25) |  |  | 2,738,289 |  |  |  |  |  | 2,738,289 |
| Michigan (26) |  |  | 4,820,500 | 4,820,500 | 3,386,755 |  |  |  | 4,820,500 |
| Missouri (29) |  | 3,073,876 |  |  |  | 3,073,876 |  |  | 3,073,876 |
| South Carolina (45) |  | 3,432,154 |  |  |  | 4,954,923 |  |  | 2,134,835 |
| Washington (53) | 1,436,000 |  |  |  |  |  |  |  |  |
| Total | 1,436,000 | 6,506,030 | 7,920,019 | 4,820,500 | 5,813,275 | 10,451,563 | 3,945,589 | 3,945,589 | 17,950,737 |

SS Total– Service Station/Total: All Gasoline-All Processes		S2 Total – Stage II Total

S1 Total – Service Station/Stage I: Total				S2 Unc. – Stage II Displacement Loss/Uncontrolled

S1 Submerged - Service Station/Stage I: Submerged			S2 Spillage – Stage II Spillage Losses (Liquid)

S1 Splash - Service Station/Stage I: Splash				UST B&E – Underground Tank Breathing & Emptying.

S1 B/S – Service Station/Stage I: Balanced/Submerged Fill

Note:   No throughput was reported for Stage II Displacement Loss/Controlled (2501060102) or Underground Tank Total (2501060200).

## Table A-2.  Gasoline Throughput Reported in the Point Source NEI by State and SCC

(1000 gal)

| State | 40600301 S1 Splash | 40600302 S1 Sub NC. | 40600305 S1 Unldg | 40600306 S1 B/S | 40600307 UST B&E | 40600399 S1 NC | 40600401 S2 Unc. | 40600402 S2 liquid | 40600403 S2 vapor | 40600499 S2 NC | 40600601 S2 Unc. | 40600602 Fleet S2 liquid | 40600603 Fleet S2 vapor | 40600701 Fleet S1 Splash | 40600702 Fleet S1 Sub NC | 40600706 Fleet S1 B/S | 40600707 Fleet UST B&E |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| California | 116 | 108 |  | 11,495 | 2,445 |  | 1,074 | 2,770 | 8,356 | 0 | 190 | 0 | 1,871 | 0 |  |  | 12 |
| Colorado | 8,934 | 74,797 | 771 | 4,618,829 | 1,565,801 | 11,443 | 1,546,931 | 1,562,814 | 15,234 | 120 |  |  | 1,440 |  | 95 | 282 |  |
| Florida |  | 2,184 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Kansas |  |  |  |  |  |  | 272,468 |  | 290,380 |  |  |  |  |  |  |  |  |
| Kentucky | 172,567 | 1,236 |  | 1,407 | 157,192 | 315 | 156,639 | 151,883 |  |  |  |  |  |  |  |  |  |
| Maryland |  |  | 2,438 |  |  |  |  |  | 1,075 |  |  |  |  |  |  |  |  |
| Michigan |  |  |  | 139,065 |  |  | 8,175 |  |  |  |  |  | 0 | 2,343 | 32,507 |  |  |
| Missouri |  |  | 2,304 |  |  |  | 9 | 274 | 3,649 |  |  |  | 104 |  |  |  |  |
| North Carolina |  | 904 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ohio |  | 221 |  | 5,486 | 2,869 | 3 | 7,771 | 4,805 |  |  |  |  |  |  |  |  |  |
| Oklahoma | 20 |  |  | 248,777 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Pennsylvania |  | 0 |  | 388 | 0 |  | 0 |  | 1,570 | 0 | 4 |  |  | 0 |  |  |  |
| Virginia | 375 | 765 |  | 16,032 | 12,869 |  | 12,493 | 7,396 | 9,922 |  |  |  |  |  |  |  |  |
| Washington |  |  |  | 1,382 | 1,382 |  | 1,501 | 1,382 |  | 18,435 |  |  |  |  |  |  |  |
| West Virginia |  | 271 |  |  | 48 |  | 301 |  |  |  |  |  |  |  |  |  |  |
| Total | 182,012 | 80,485 | 5,513 | 5,042,861 | 1,742,607 | 11,761 | 2,007,361 | 1,731,324 | 330,185 | 18,555 | 193 | 0 | 3,415 | 2,343 | 32,602 | 282 | 12 |

S1 Splash – Stage I: Splash Fill					Fleet S2 Liquid – Fleet Refueling Ops./Stage II: Liquid Loss without controls

S1 Sub NC – Stage I: Submerged filling w/o controls			Fleet S2 Vapor – Fleet Refueling Ops./Stage II: Vapor Loss with controls

S1 Unldg – Stage I: Unloading					Fleet S1 Splash – Fleet Refueling Ops./Stage I: Splash Filling

S1 B/S – Stage I: Balanced Submerged Fill				Fleet S1 Sub NC – Fleet Refueling Ops./Stage I: Submerged Filling without controls

UST B&E – Stage I: UST Breathing & Emptying			Fleet S1 B/S – Fleet Refueling Ops./Stage I: Balanced Submerged Fill

S1 NC – Stage I: Not Classified					Fleet UST B&E – Fleet Refueling Ops./Stage I: Breathing & Emptying

S2 Unc. – Stage II: Vapor Loss without controls (pumped)

S2 Liquid – Stage II: Liquid Loss without controls

S2 Vapor – Stage II: Vapor Loss with controls

S2 NC – Stage II: Not Classified

## Table A-3.  Emissions Reported in the Area Source NEI by State and SCC

(tons)

| State (FIPS) | 2501060000 (SS Total) | 2501060050 (S1 Total) | 2501060051 (S1 Submerged) | 2501060052 (S1 Splash) | 2501060053 (S1 Bal/Submerged) | 2501060100 (S2 Total) | 2501060101 (S2 Unc). | 2501060102 (S2 Controlled) | 2501060103 (S2 Spillage) | 2501060200 (UST Total) | 2501060201 (UST B&E) | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Alabama (01) |  | 2,005 |  |  | 10 | 11,163 |  |  |  | 36 | 1,197 | 14,411 |
| Alaska (02) | NR | NR | NR | NR | NR | NR | NR | NR | NR | NR | NR | NR |
| Arizona (04) |  | 8,659 |  |  |  | 9,490 |  |  |  |  | 636 | 18,786 |
| Arkansas (05) |  | 5,632 |  |  |  | 7,435 |  |  |  |  | 935 | 14,001 |
| California (06) |  |  |  |  |  |  | 7,134 | 5,245 | 12,861 |  | 4,446 | 29,685 |
| Colorado (08) |  | 4,299 |  |  |  | 8,487 |  |  |  |  | 1,062 | 13,847 |
| Connecticut (09) |  | 397 |  |  |  |  | 542 | 1,092 |  |  | 678 | 2,708 |
| Delaware (10) |  |  | 426 |  |  |  | 512 |  | 43 |  | 184 | 1,166 |
| District of Columbia (11) |  |  |  |  | 82 | 138 |  |  |  |  | 6 | 227 |
| Florida (12) |  | 11,129 | 5,900 |  |  | 29,469 |  |  | 93 |  | 2,402 | 48,992 |
| Georgia (13) | 5,009 | 6,965 |  |  | 470 | 9,921 |  |  |  | 100 | 1,292 | 23,757 |
| Hawaii (15) | NR | NR | NR | NR | NR | NR | NR | NR | NR | NR | NR | NR |
| Idaho (16) |  | 2,228 |  |  |  | 2,442 |  |  |  |  | 306 | 4,976 |
| Illinois (17) |  | 13,769 |  |  |  | 26,831 |  |  |  | 1,963 |  | 42,563 |
| Indiana (18) |  |  |  | 9,718 | 227 |  | 12,613 | 303 | 1,122 | 1,603 |  | 25,587 |
| Iowa (19) |  | 5,727 |  |  |  | 6,288 |  |  |  |  | 792 | 12,807 |
| Kansas (20) |  | 5,633 |  |  |  | 6,178 |  |  |  |  | 775 | 12,586 |
| Kentucky (21) |  | 3,208 | 3,156 | 522 | 359 | 9,884 |  |  |  |  | 1,050 | 18,180 |
| Louisiana (22) |  | 325 |  | 2,822 | 36 | 8,610 | 333 | 1,926 | 201 |  | 2,382 | 16,634 |
| Maine (23) |  |  | 524 | 1,576 | 63 | 2,734 | 61 |  |  |  | 330 | 5,288 |
| Maryland (24) | 994 |  |  |  | 1,955 | 2,303 |  |  |  |  | 1,924 | 7,177 |
| Massachusetts (25) |  |  | 3,032 | 16 | 2,278 | 10,039 |  |  |  |  | 1,369 | 16,734 |
| Michigan (26) |  | 439 | 9,842 | 2,732 | 247 | 24,299 |  |  |  | 1,187 | 2,414 | 41,161 |
| Minnesota (27) |  | 8,752 |  |  |  | 9,590 |  |  |  |  | 1,206 | 19,548 |
| Mississippi (28) |  | 6,187 |  |  |  | 6,782 |  |  |  |  | 854 | 13,823 |
| Missouri (29) | 472 | 10,547 |  |  |  | 15,911 |  |  |  |  | 1,536 | 28,466 |
| Montana (30) |  | 2,055 |  |  |  | 2,252 |  |  |  |  | 282 | 4,588 |
| Nebraska (31) |  | 3,316 |  |  |  | 3,635 |  |  |  |  | 460 | 7,411 |
| Nevada (32) |  | 2,242 |  |  |  | 3,924 |  |  |  |  | 175 | 6,341 |
| New Hampshire (33) |  | 2,924 |  |  |  | 3,426 |  |  | 65 |  | 117 | 6,533 |
| New Jersey (34) |  | 578 |  |  |  |  |  | 3,827 |  |  | 231 | 4,636 |

## Table A-3.  Emissions Reported in the Area Source NEI by State and SCC (Concluded)

(tons)

| State (FIPS) | 2501060000 (SS Total) | 2501060050 (S1 Total) | 2501060051 (S1 Submerged) | 2501060052 (S1 Splash) | 2501060053 (S1 B/S) | 2501060100 (S2 Total) | 2501060101 (S2 Unc). | 2501060102 (S2 Controlled) | 2501060103 (S2 Spillage) | 2501060200 (UST Total) | 2501060201 (UST B&E) | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| New Mexico (35) |  | 3,352 |  |  |  | 4,733 |  |  |  |  | 592 | 8,677 |
| New York (36) |  | 33,531 |  |  |  |  |  |  | 12,047 |  |  | 45,578 |
| North Carolina (37) |  | 22,556 |  |  |  |  | 4,425 |  | 282 |  | 1,342 | 28,604 |
| North Dakota (38) |  | 1,459 |  |  |  | 1,599 |  |  |  |  | 202 | 3,260 |
| Ohio (39) | 11,977 | 6,662 |  |  |  | 9,349 |  |  |  |  | 1,142 | 29,129 |
| Oklahoma (40) |  | 232 |  |  |  | 7,576 |  |  |  |  | 952 | 8,759 |
| Oregon (41) | NR | NR | NR | NR | NR | NR | NR | NR | NR | NR | NR | NR |
| Pennsylvania (42) |  |  | 13,251 |  | 4,046 |  | 9,796 |  |  |  | 2,211 | 29,303 |
| Rhode Island (44) |  |  | 116 | 385 | 38 |  | 440 |  |  |  | 25 | 1,004 |
| South Carolina (45) |  | 11,269 |  |  |  | 11,600 |  |  |  |  | 1,057 | 23,926 |
| South Dakota (46) |  | 1,644 |  |  |  | 1,798 |  |  |  |  | 227 | 3,669 |
| Tennessee (47) |  | 15,105 |  |  | 375 | 17,412 |  |  |  |  | 1,468 | 34,359 |
| Texas (48) |  |  |  |  | 27,364 | 22,548 |  |  | 2,241 |  | 5,023 | 57,176 |
| Utah (49) |  | 407 |  |  |  | 4,537 |  |  |  |  | 75 | 5,019 |
| Vermont (50) |  | 3,627 |  |  |  | 1,268 |  |  |  |  |  | 4,895 |
| Virginia (51) |  |  | 7,314 |  | 1,811 | 3,500 | 5,836 | 3,681 | 1,664 |  | 2,107 | 25,912 |
| Washington (53) | 3,252 |  |  |  |  |  |  |  |  |  |  | 3,252 |
| West Virginia (54) |  | 2,459 | 1,294 | 2,116 |  | 2,700 | 462 |  | 29 |  | 480 | 9,541 |
| Wisconsin (55) |  | 5,707 |  |  |  | 7,030 |  |  |  |  | 876 | 13,613 |
| Wyoming (56) |  | 1,379 |  |  |  | 1,512 |  |  |  |  | 189 | 3,080 |
| Total | 21,703 | 216,403 | 44,855 | 19,888 | 39,363 | 318,394 | 42,154 | 16,073 | 30,650 | 4,889 | 47,006 | 801,377 |

SS Total– Service Station/Total: All Gasoline-All Processes		S2 Total – Stage II Total

S1 Total – Service Station/Stage I: Total				S2 Unc. – Stage II Displacement Loss/Uncontrolled

S1 Submerged - Service Station/Stage I: Submerged			S2 Controlled – Stage II Displacement Loss/Controlled

S1 Splash - Service Station/Stage I: Splash				S2 Spillage – Stage II Spillage Losses (Liquid)

S1 B/S – Service Station/Stage I: Balanced/Submerged Fill 		UST Total – Underground Tank Total

UST B&E – Underground Tank Breathing & Emptying.

## Table A-4.  Emissions Reported in the Point Source NEI by State and SCC

(tons)

| State | 40600301 S1 Splash | 40600302 S1 Sub NC. | 40600305 S1 Unldg | 40600306 S1 B/S | 40600307 UST B&E | 40600399 S1 NC | 40600401 S2 Unc. | 40600402 S2 liquid | 40600403 S2 vapor | 40600499 S2 NC | 40600601 S2 Unc. | 40600602 S2 liquid | 40600603 S2 vapor | 40600701 S1 Splash | 40600702 S1 Sub NC | 40600706 S1 B/S | 40600707 UST B&E | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Retail | Retail | Retail | Retail | Retail | Retail | Retail | Retail | Retail | Retail | Fleet | Fleet | Fleet | Fleet | Fleet | Fleet | Fleet |  |
| Alabama |  | 3.00 |  |  |  | 3.82 |  |  |  |  | 0.20 |  |  |  |  |  |  | 7.02 |
| Arizona | 17.47 |  |  | 0.29 | 0.55 |  |  | 1.17 |  |  |  |  |  |  |  |  |  | 19.48 |
| California | 0.05 | 0.82 |  | 29.92 | 3.20 |  | 3.63 | 0.81 | 1,121.76 | 1.92 | 0.31 | 0.10 | 5.90 | 0.00 |  |  | 0.04 | 1,168.46 |
| Colorado | 63.64 | 307.62 | 0.38 | 453.44 | 785.77 | 45.55 | 8,489.60 | 518.61 | 7.22 | 0.00 |  |  | 0.79 |  | 0.38 | 1.65 |  | 10,674.65 |
| Florida |  | 25.88 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 25.88 |
| Illinois | 2.38 | 0.16 |  | 33.99 | 1.80 |  | 18.02 | 4.36 |  |  | 0.43 | 0.14 | 0.07 |  |  | 1.06 | 23.79 | 86.20 |
| Indiana |  |  |  |  |  |  | 4.39 |  |  |  |  |  |  |  |  |  |  | 4.39 |
| Kansas |  |  |  |  |  |  | 0.05 |  | 17.17 |  |  |  |  |  |  |  |  | 17.22 |
| Kentucky | 27.53 | 4.60 |  | 0.20 | 73.76 | 0.28 | 294.88 | 51.08 |  |  |  |  |  |  |  |  |  | 452.33 |
| Louisiana | 0.00 | 2.00 |  |  | 0.00 | 1.00 | 2.00 |  | 70.00 | 1.00 | 1.00 |  | 8.00 |  | 0.00 |  |  | 85.00 |
| Maryland |  |  | 19.15 |  |  |  |  |  | 12.39 |  |  |  |  |  |  |  |  | 31.54 |
| Michigan |  |  |  | 0.53 |  |  | 13.57 |  |  |  |  |  | 3.62 | 4.54 | 0.12 |  |  | 22.38 |
| Missouri |  |  | 1.15 |  |  |  | 0.52 | 0.09 | 1.25 |  | 0.13 |  | 0.25 |  |  |  |  | 3.39 |
| North Carolina |  | 3.01 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.01 |
| New Jersey |  |  |  |  |  |  | 200.53 |  | 16.98 |  |  |  |  |  |  |  |  | 217.51 |
| New Mexico |  |  |  |  |  |  |  |  |  |  | 3.09 |  |  |  |  |  |  | 3.09 |
| New York |  |  |  |  |  | 0.10 |  |  |  |  |  |  |  |  |  |  |  | 0.10 |
| Ohio |  | 0.79 |  | 0.96 | 1.93 | 0.31 | 6.43 | 1.64 |  |  |  |  |  |  |  |  |  | 12.06 |
| Oklahoma | 0.18 |  |  | 27.32 |  |  |  |  |  |  |  |  |  |  |  |  |  | 27.50 |
| Pennsylvania |  | 0.11 |  | 0.81 | 0.03 |  | 0.39 |  | 1.30 | 0.25 | 0.02 |  |  | 0.01 |  |  |  | 2.92 |
| Rhode Island |  |  |  |  |  |  |  |  |  | 2.32 |  |  |  |  |  |  |  | 2.32 |
| Texas | 0.79 |  |  | 0.14 | 0.85 |  | 4.97 |  | 7.75 |  |  |  |  | 1.75 |  |  |  | 16.25 |
| Utah | 6.47 | 0.09 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 6.56 |
| Virginia | 13.52 | 24.33 |  | 7.42 | 17.26 |  | 21.55 | 1.94 | 12.70 |  |  |  |  |  |  |  |  | 98.74 |
| Washington |  |  |  | 0.00 | 1.00 |  | 8.66 | 0.00 |  | 28.55 |  |  |  |  |  |  |  | 38.21 |
| Wisconsin |  |  |  |  |  |  | 0.10 |  | 0.00 |  |  |  |  |  |  |  |  | 0.11 |
| West Virginia |  | 0.02 |  |  | 386.45 |  | 0.25 |  |  |  |  |  |  |  |  |  |  | 386.73 |
| Total | 132.03 | 372.44 | 20.68 | 555.02 | 1,272.60 | 51.06 | 9,069.55 | 579.70 | 1,268.52 | 34.04 | 5.18 | 0.24 | 18.63 | 6.30 | 0.50 | 2.71 | 23.83 | 13,413.04 |

## Table B-1.  State Stage I and Stage II Control Levels

|  |  | Stage I |  | Stage II |
| --- | --- | --- | --- | --- |
| State | County | Submerged Loading | Vapor Balancing | Controlled |
| Alabama | ALL | Y | Y | N |
| Alaska | ALL | N | N | N |
| Arizona | Maricopa | Y | Y | Y |
| Arizona | ALL, except Maricopa | Y | N | N |
| Arkansas | Pulaski Co. | Y | N | N |
| California | ALL | Y | Y | Y |
| Colorado | Adams | Y | Y | N |
| Colorado | Arapohoe | Y | Y | N |
| Colorado | Boulder | Y | Y | N |
| Colorado | Denver | Y | Y | N |
| Colorado | Douglas | Y | Y | N |
| Colorado | Jefferson | Y | Y | N |
| Connecticut | ALL | Y | Y | Y |
| Delaware | ALL | Y | Y | Y |
| District of Columbia | District of Columbia | Y | Y | Y |
| Florida | Broword | Y | Y | Y |
| Florida | Dade | Y | Y | Y |
| Florida | Palm Beach | Y | Y | Y |
| Georgia | Cherokee | Y | Y | Y |
| Georgia | Clayton | Y | Y | Y |
| Georgia | Cobb | Y | Y | Y |
| Georgia | Coweta | Y | Y | Y |
| Georgia | De Kalb | Y | Y | Y |
| Georgia | Douglas | Y | Y | Y |
| Georgia | Fayette | Y | Y | Y |
| Georgia | Forsyth | Y | Y | Y |
| Georgia | Fulton | Y | Y | Y |
| Georgia | Gwinnett | Y | Y | Y |
| Georgia | Henry | Y | Y | Y |
| Georgia | Paulding | Y | Y | Y |
| Georgia | Rockdale | Y | Y | Y |
| Hawaii | ALL | Y | N | N |
| Idaho | ALL | N | N | N |
| Illinois | Cook (Chicago Area) | Y | Y | Y |
| Illinois | Du Page (Chicago Area) | Y | Y | Y |
| Illinois | Grundy (Chicago Area) | Y | Y | Y |
| Illinois | Kane (Chicago Area) | Y | Y | Y |
| Illinois | Kendall (Chicago Area) | Y | Y | Y |
| Illinois | Lake (Chicago Area) | Y | Y | Y |
| Illinois | Madison (Metro East) | Y | Y | N |
| Illinois | McHenry (Chicago Area) | Y | Y | Y |
| Illinois | Monroe (Metro East) | Y | Y | N |
| Illinois | St. Claire (Metro East) | Y | Y | N |
| Illinois | Will (Chicago Area) | Y | Y | Y |
| Illinois | All others except those in Chicago or Metro East Areas | Y | Y | N |
| Indiana | Clark | Y | Y | Y |
| Indiana | Elkhart | Y | Y | N |
| Indiana | Floyd | Y | Y | Y |
| Indiana | Hendricks | Y | Y | N |
| Indiana | Lake | Y | Y | Y |
| Indiana | Marion | Y | Y | N |
| Indiana | Porter | Y | Y | Y |
| Indiana | St. Joseph | Y | Y | N |
| Indiana | Boone | Y | Y | N |
| Indiana | Dearborn | Y | Y | N |
| Indiana | Hamilton | Y | Y | N |
| Indiana | Hancock | Y | Y | N |
| Indiana | Harrison | Y | Y | N |
| Indiana | Johnson | Y | Y | N |
| Indiana | Morgan | Y | Y | N |
| Indiana | Shelby | Y | Y | N |
| Indiana | Vanderburgh | Y | Y | N |
| Iowa | ALL | N | N | N |
| Kansas | ALL | N | N | N |
| Kentucky | Boone | Y | Y | Y |
| Kentucky | Bullit | Y | Y | Y |
| Kentucky | Campbell | Y | Y | Y |
| Kentucky | Jefferson | Y | Y | Y |
| Kentucky | Kenton | Y | Y | Y |
| Kentucky | Oldham | Y | Y | Y |
| Louisiana | Ascension | Y | Y | Y |
| Louisiana | Calcasieu | Y | Y | N |
| Louisiana | East Baton Rouge | Y | Y | Y |
| Louisiana | Iberville | Y | Y | Y |
| Louisiana | Livingston | Y | Y | Y |
| Louisiana | Pointe Coupee | Y | Y | Y |
| Louisiana | West Baton Rouge | Y | Y | Y |
| Louisiana | Beauregard | Y | Y | N |
| Louisiana | Bossier | Y | Y | N |
| Louisiana | Caddo | Y | Y | N |
| Louisiana | Grant | Y | Y | N |
| Louisiana | Jefferson | Y | Y | N |
| Louisiana | Lafayette | Y | Y | N |
| Louisiana | Lafourche | Y | Y | N |
| Louisiana | Orleans | Y | Y | N |
| Louisiana | St. Bernard | Y | Y | N |
| Louisiana | St. Charles | Y | Y | N |
| Louisiana | St. James | Y | Y | N |
| Louisiana | St. John the Baptist | Y | Y | N |
| Louisiana | St. Mary | Y | Y | N |
| Maine | Androscoggin | Y | Y | N |
| Maine | Aroostook | Y | Y | N |
| Maine | Cumberland | Y | Y | Y |
| Maine | Franklin | Y | Y | N |
| Maine | Hancock | Y | Y | N |
| Maine | Kennebec | Y | Y | N |
| Maine | Knox | Y | Y | N |
| Maine | Lincoln | Y | Y | N |
| Maine | Oxford | Y | Y | N |
| Maine | Penobscot | Y | Y | N |
| Maine | Piscataquis | Y | Y | N |
| Maine | Sagadahoc | Y | Y | Y |
| Maine | Somerset | Y | Y | N |
| Maine | Waldo | Y | Y | N |
| Maine | Washington | Y | Y | N |
| Maine | York | Y | Y | Y |
| Maryland | Allegany | Y | Y | N |
| Maryland | Anne Arundel | Y | Y | Y |
| Maryland | Baltimore | Y | Y | Y |
| Maryland | Calvert | Y | Y | Y |
| Maryland | Caroline | Y | Y | N |
| Maryland | Carroll | Y | Y | Y |
| Maryland | Cecil | Y | Y | Y |
| Maryland | Charles | Y | Y | Y |
| Maryland | Dorchester | Y | Y | N |
| Maryland | Frederick | Y | Y | Y |
| Maryland | Garrett | Y | Y | N |
| Maryland | Harford | Y | Y | Y |
| Maryland | Howard | Y | Y | Y |
| Maryland | Kent | Y | Y | N |
| Maryland | Montgomery | Y | Y | Y |
| Maryland | Prince George's | Y | Y | Y |
| Maryland | Queen Anne's | Y | Y | N |
| Maryland | St. Mary's | Y | Y | N |
| Maryland | Somerset | Y | Y | N |
| Maryland | Talbot | Y | Y | N |
| Maryland | Washington | Y | Y | N |
| Maryland | Wicomico | Y | Y | N |
| Maryland | Worcester | Y | Y | N |
| Massachusetts | ALL | Y | Y | Y |
| Michigan | Alcona | Y | N | N |
| Michigan | Alger | Y | N | N |
| Michigan | Allegan | Y | N | N |
| Michigan | Alpena | Y | N | N |
| Michigan | Antrim | Y | N | N |
| Michigan | Arenac | Y | N | N |
| Michigan | Baraga | Y | N | N |
| Michigan | Barry | Y | N | N |
| Michigan | Bay | Y | N | N |
| Michigan | Benzie | Y | N | N |
| Michigan | Berrien | Y | N | N |
| Michigan | Branch | Y | N | N |
| Michigan | Calhoun | Y | N | N |
| Michigan | Cass | Y | N | N |
| Michigan | Charlevoix | Y | N | N |
| Michigan | Cheboygan | Y | N | N |
| Michigan | Chippewa | Y | N | N |
| Michigan | Clare | Y | N | N |
| Michigan | Clinton | Y | Y | N |
| Michigan | Crawford | Y | N | N |
| Michigan | Delta | Y | N | N |
| Michigan | Dickinson | Y | N | N |
| Michigan | Eaton | Y | Y | N |
| Michigan | Emmet | Y | N | N |
| Michigan | Genesee | Y | Y | N |
| Michigan | Gladwin | Y | N | N |
| Michigan | Gogebic | Y | N | N |
| Michigan | Grand Traverse | Y | N | N |
| Michigan | Gratiot | Y | N | N |
| Michigan | Hillsdale | Y | N | N |
| Michigan | Houghton | Y | N | N |
| Michigan | Huron | Y | N | N |
| Michigan | Ingham | Y | Y | N |
| Michigan | Ionia | Y | N | N |
| Michigan | Iosco | Y | N | N |
| Michigan | Iron | Y | N | N |
| Michigan | Isabella | Y | N | N |
| Michigan | Jackson | Y | N | N |
| Michigan | Kalamazoo | Y | N | N |
| Michigan | Kalkaska | Y | N | N |
| Michigan | Kent | Y | Y | N |
| Michigan | Keweenaw | Y | N | N |
| Michigan | Lake | Y | N | N |
| Michigan | Lapeer | Y | N | N |
| Michigan | Leelanau | Y | N | N |
| Michigan | Lenawee | Y | N | N |
| Michigan | Livingston | Y | N | N |
| Michigan | Luce | Y | N | N |
| Michigan | Mackinac | Y | N | N |
| Michigan | Macomb | Y | Y | N |
| Michigan | Manistee | Y | N | N |
| Michigan | Marquette | Y | N | N |
| Michigan | Mason | Y | N | N |
| Michigan | Mecosta | Y | N | N |
| Michigan | Menominee | Y | N | N |
| Michigan | Midland | Y | N | N |
| Michigan | Missaukee | Y | N | N |
| Michigan | Monroe | Y | N | N |
| Michigan | Montcalm | Y | N | N |
| Michigan | Montmorency | Y | N | N |
| Michigan | Muskegon | Y | N | N |
| Michigan | Newaygo | Y | N | N |
| Michigan | Oakland | Y | Y | N |
| Michigan | Oceana | Y | N | N |
| Michigan | Ogemaw | Y | N | N |
| Michigan | Ontonagon | Y | N | N |
| Michigan | Osceola | Y | N | N |
| Michigan | Oscoda | Y | N | N |
| Michigan | Otsego | Y | N | N |
| Michigan | Ottawa | Y | Y | N |
| Michigan | Presque Isle | Y | N | N |
| Michigan | Roscommon | Y | N | N |
| Michigan | Saginaw | Y | N | N |
| Michigan | St. Clair | Y | N | N |
| Michigan | St. Joseph | Y | N | N |
| Michigan | Sanilac | Y | N | N |
| Michigan | Schoolcraft | Y | N | N |
| Michigan | Shiawassee | Y | N | N |
| Michigan | Tuscola | Y | N | N |
| Michigan | Van Buren | Y | N | N |
| Michigan | Washtenaw | Y | Y | N |
| Michigan | Wayne | Y | Y | N |
| Michigan | Wexford | Y | N | N |
| Minnesota | ALL | Y | N | N |
| Mississippi | ALL | N | N | N |
| Missouri | Clay | Y | Y | N |
| Missouri | Franklin | Y | Y | N |
| Missouri | St. Louis (city) | Y | Y | N |
| Missouri | Jackson | Y | Y | N |
| Missouri | Platte | Y | Y | N |
| Missouri | St. Charles | Y | Y | N |
| Missouri | St. Louis | Y | Y | N |
| Montana | ALL | Y | N | N |
| Nebraska | ALL | N | N | N |
| Nevada | ALL | Y | N | N |
| New Hampshire | Belknap | Y | Y | N |
| New Hampshire | Carroll | Y | Y | N |
| New Hampshire | Cheshire | Y | Y | N |
| New Hampshire | Coos | Y | Y | N |
| New Hampshire | Grafton | Y | Y | N |
| New Hampshire | Hillsborough | Y | Y | Y |
| New Hampshire | Merrimack | Y | Y | Y |
| New Hampshire | Rockingham | Y | Y | Y |
| New Hampshire | Strafford | Y | Y | Y |
| New Hampshire | Sullivan | Y | Y | N |
| New Jersey | ALL | Y | Y | Y |
| New Mexico | Bernalillo County | Y | Y | N |
| New Mexico | Alburquerqe | Y | Y | N |
| New York | Albany | Y | Y | N |
| New York | Allegany | Y | Y | N |
| New York | Bronx | Y | Y | Y |
| New York | Broome | Y | Y | N |
| New York | Cattaraugus | Y | Y | N |
| New York | Cayuga | Y | Y | N |
| New York | Chautauqua | Y | Y | N |
| New York | Chemung | Y | Y | N |
| New York | Chenango | Y | Y | N |
| New York | Clinton | Y | Y | N |
| New York | Columbia | Y | Y | N |
| New York | Cortland | Y | Y | N |
| New York | Delaware | Y | Y | N |
| New York | Dutchess | Y | Y | N |
| New York | Erie | Y | Y | N |
| New York | Essex | Y | Y | N |
| New York | Franklin | Y | Y | N |
| New York | Fulton | Y | Y | N |
| New York | Genesee | Y | Y | N |
| New York | Greene | Y | Y | N |
| New York | Hamilton | Y | Y | N |
| New York | Herkimer | Y | Y | N |
| New York | Jefferson | Y | Y | N |
| New York | Kings | Y | Y | Y |
| New York | Lewis | Y | Y | N |
| New York | Livingston | Y | Y | N |
| New York | Madison | Y | Y | N |
| New York | Monroe | Y | Y | N |
| New York | Montgomery | Y | Y | N |
| New York | Nassau | Y | Y | Y |
| New York | New York | Y | Y | Y |
| New York | Niagara | Y | Y | N |
| New York | Oneida | Y | Y | N |
| New York | Onondaga | Y | Y | N |
| New York | Ontario | Y | Y | N |
| New York | Orange | Y | Y | Y |
| New York | Orleans | Y | Y | N |
| New York | Oswego | Y | Y | N |
| New York | Otsego | Y | Y | N |
| New York | Putnam | Y | Y | N |
| New York | Queens | Y | Y | Y |
| New York | Rensselaer | Y | Y | N |
| New York | Richmond | Y | Y | Y |
| New York | Rockland | Y | Y | Y |
| New York | St. Lawrence | Y | Y | N |
| New York | Saratoga | Y | Y | N |
| New York | Schenectady | Y | Y | N |
| New York | Schoharie | Y | Y | N |
| New York | Schuyler | Y | Y | N |
| New York | Seneca | Y | Y | N |
| New York | Steuben | Y | Y | N |
| New York | Suffolk | Y | Y | Y |
| New York | Sullivan | Y | Y | N |
| New York | Tioga | Y | Y | N |
| New York | Tompkins | Y | Y | N |
| New York | Ulster | Y | Y | N |
| New York | Warren | Y | Y | N |
| New York | Washington | Y | Y | N |
| New York | Wayne | Y | Y | N |
| New York | Westchester | Y | Y | Y |
| New York | Wyoming | Y | Y | N |
| New York | Yates | Y | Y | N |
| North Carolina | ALL | Y | Y | N |
| North Dakota | ALL | Y | N | N |
| Ohio | Ashtbula | Y | Y | Y |
| Ohio | Butler | Y | Y | Y |
| Ohio | Clark | Y | Y | Y |
| Ohio | Clermont | Y | Y | Y |
| Ohio | Cuyahoga | Y | Y | Y |
| Ohio | Delaware | Y | Y | N |
| Ohio | Franklin | Y | Y | N |
| Ohio | Geauga | Y | Y | Y |
| Ohio | Greene | Y | Y | Y |
| Ohio | Hamilton | Y | Y | Y |
| Ohio | Lake | Y | Y | Y |
| Ohio | Licking | Y | Y | N |
| Ohio | Loraine | Y | Y | Y |
| Ohio | Lucas | Y | Y | N |
| Ohio | Mahoning | Y | Y | N |
| Ohio | Medina | Y | Y | Y |
| Ohio | Miami | Y | Y | Y |
| Ohio | Montgomery | Y | Y | Y |
| Ohio | Portage | Y | Y | Y |
| Ohio | Stark | Y | Y | N |
| Ohio | Summit | Y | Y | Y |
| Ohio | Trumbull | Y | Y | N |
| Ohio | Warren | Y | Y | Y |
| Ohio | Wood | Y | Y | N |
| Oklahoma | Tulsa | Y | Y | N |
| Oklahoma | Oklahoma | Y | Y | N |
| Oregon | Clackamus | Y | Y | Y |
| Oregon | Multnomah | Y | Y | Y |
| Oregon | Washington | Y | Y | Y |
| Oregon | Marion | Y | Y | N |
| Oregon | Polk | Y | Y | N |
| Oregon | Jackson) | Y | Y | N |
| Pennsylvania | Adams | Y | Y | N |
| Pennsylvania | Allegheny | Y | Y | Y |
| Pennsylvania | Armstrong | Y | Y | Y |
| Pennsylvania | Beaver | Y | Y | Y |
| Pennsylvania | Bedford | Y | Y | N |
| Pennsylvania | Berks | Y | Y | Y |
| Pennsylvania | Blair | Y | Y | N |
| Pennsylvania | Bradford | Y | Y | Y |
| Pennsylvania | Bucks | Y | Y | N |
| Pennsylvania | Butler | Y | Y | Y |
| Pennsylvania | Cambria | Y | Y | N |
| Pennsylvania | Cameron | Y | Y | N |
| Pennsylvania | Carbon | Y | Y | N |
| Pennsylvania | Centre | Y | Y | N |
| Pennsylvania | Chester | Y | Y | Y |
| Pennsylvania | Clarion | Y | Y | N |
| Pennsylvania | Clearfield | Y | Y | N |
| Pennsylvania | Clinton | Y | Y | N |
| Pennsylvania | Columbia | Y | Y | N |
| Pennsylvania | Crawford | Y | Y | N |
| Pennsylvania | Cumberland | Y | Y | N |
| Pennsylvania | Dauphin | Y | Y | N |
| Pennsylvania | Delaware | Y | Y | Y |
| Pennsylvania | Elk | Y | Y | N |
| Pennsylvania | Erie | Y | Y | N |
| Pennsylvania | Fayette | Y | Y | Y |
| Pennsylvania | Forest | Y | Y | N |
| Pennsylvania | Franklin | Y | Y | N |
| Pennsylvania | Fulton | Y | Y | N |
| Pennsylvania | Greene | Y | Y | N |
| Pennsylvania | Huntingdon | Y | Y | N |
| Pennsylvania | Indiana | Y | Y | N |
| Pennsylvania | Jefferson | Y | Y | N |
| Pennsylvania | Juniata | Y | Y | N |
| Pennsylvania | Lackawanna | Y | Y | N |
| Pennsylvania | Lancaster | Y | Y | N |
| Pennsylvania | Lawrence | Y | Y | N |
| Pennsylvania | Lebanon | Y | Y | N |
| Pennsylvania | Lehigh | Y | Y | N |
| Pennsylvania | Luzerne | Y | Y | N |
| Pennsylvania | Lycoming | Y | Y | N |
| Pennsylvania | McKean | Y | Y | N |
| Pennsylvania | Mercer | Y | Y | N |
| Pennsylvania | Mifflin | Y | Y | N |
| Pennsylvania | Monroe | Y | Y | N |
| Pennsylvania | Montgomery | Y | Y | Y |
| Pennsylvania | Montour | Y | Y | N |
| Pennsylvania | Northampton | Y | Y | N |
| Pennsylvania | Northumberland | Y | Y | N |
| Pennsylvania | Perry | Y | Y | N |
| Pennsylvania | Philadelphia | Y | Y | Y |
| Pennsylvania | Pike | Y | Y | N |
| Pennsylvania | Potter | Y | Y | N |
| Pennsylvania | Schuylkill | Y | Y | N |
| Pennsylvania | Snyder | Y | Y | N |
| Pennsylvania | Somerset | Y | Y | N |
| Pennsylvania | Sullivan | Y | Y | N |
| Pennsylvania | Susquehanna | Y | Y | N |
| Pennsylvania | Tioga | Y | Y | N |
| Pennsylvania | Union | Y | Y | N |
| Pennsylvania | Venango | Y | Y | N |
| Pennsylvania | Warren | Y | Y | N |
| Pennsylvania | Washington | Y | Y | Y |
| Pennsylvania | Wayne | Y | Y | N |
| Pennsylvania | Westmoreland | Y | Y | Y |
| Pennsylvania | Wyoming | Y | Y | N |
| Pennsylvania | York | Y | Y | N |
| Rhode Island | ALL | Y | Y | Y |
| South Carolina | None | N | N | N |
| South Dakota | None | N | N | N |
| Tennessee | Davidson | Y | Y | Y |
| Tennessee | Rutherford | Y | Y | Y |
| Tennessee | Shelby | Y | Y | N |
| Tennessee | Sumner | Y | Y | Y |
| Tennessee | Williamson | Y | Y | Y |
| Tennessee | Wilson | Y | Y | Y |
| Texas | Anderson | Y | Y | N |
| Texas | Angelina | Y | Y | N |
| Texas | Aransas | Y | Y | N |
| Texas | Atascosa | Y | Y | N |
| Texas | Austin | Y | Y | N |
| Texas | Bastrop | Y | Y | N |
| Texas | Bee | Y | Y | N |
| Texas | Bell | Y | Y | N |
| Texas | Bexar | Y | Y | N |
| Texas | Bosque | Y | Y | N |
| Texas | Bowie | Y | Y | N |
| Texas | Brazoria | Y | Y | Y |
| Texas | Brazos | Y | Y | N |
| Texas | Burleson | Y | Y | N |
| Texas | Caldwell | Y | Y | N |
| Texas | Calhoun | Y | Y | N |
| Texas | Camp | Y | Y | N |
| Texas | Cass | Y | Y | N |
| Texas | Chambers | Y | Y | Y |
| Texas | Cherokee | Y | Y | N |
| Texas | Collin | Y | Y | Y |
| Texas | Colorado | Y | Y | N |
| Texas | Comal | Y | Y | N |
| Texas | Cooke | Y | Y | N |
| Texas | Coryell | Y | Y | N |
| Texas | Dallas | Y | Y | Y |
| Texas | Delta | Y | Y | N |
| Texas | Denton | Y | Y | Y |
| Texas | DeWitt | Y | Y | N |
| Texas | Ellis | Y | Y | N |
| Texas | El Paso | Y | Y | Y |
| Texas | Falls | Y | Y | N |
| Texas | Fannin | Y | Y | N |
| Texas | Fayette | Y | Y | N |
| Texas | Fort Bend | Y | Y | Y |
| Texas | Franklin | Y | Y | N |
| Texas | Freestone | Y | Y | N |
| Texas | Galveston | Y | Y | Y |
| Texas | Goliad | Y | Y | N |
| Texas | Gonzales | Y | Y | N |
| Texas | Grayson | Y | Y | N |
| Texas | Gregg | Y | Y | N |
| Texas | Grimes | Y | Y | N |
| Texas | Guadalupe | Y | Y | N |
| Texas | Hardin | Y | Y | Y |
| Texas | Harris | Y | Y | Y |
| Texas | Harrison | Y | Y | N |
| Texas | Hays | Y | Y | N |
| Texas | Henderson | Y | Y | N |
| Texas | Hill | Y | Y | N |
| Texas | Hood | Y | Y | N |
| Texas | Hopkins | Y | Y | N |
| Texas | Houston | Y | Y | N |
| Texas | Hunt | Y | Y | N |
| Texas | Jackson | Y | Y | N |
| Texas | Jasper | Y | Y | N |
| Texas | Jefferson | Y | Y | Y |
| Texas | Johnson | Y | Y | N |
| Texas | Karnes | Y | Y | N |
| Texas | Kaufman | Y | Y | N |
| Texas | Lamar | Y | Y | N |
| Texas | Lavaca | Y | Y | N |
| Texas | Lee | Y | Y | N |
| Texas | Leon | Y | Y | N |
| Texas | Liberty | Y | Y | Y |
| Texas | Limestone | Y | Y | N |
| Texas | Live Oak | Y | Y | N |
| Texas | McLennan | Y | Y | N |
| Texas | Madison | Y | Y | N |
| Texas | Marion | Y | Y | N |
| Texas | Matagorda | Y | Y | N |
| Texas | Milam | Y | Y | N |
| Texas | Montgomery | Y | Y | Y |
| Texas | Morris | Y | Y | N |
| Texas | Nacogdoches | Y | Y | N |
| Texas | Navarro | Y | Y | N |
| Texas | Newton | Y | Y | N |
| Texas | Nueces | Y | Y | N |
| Texas | Orange | Y | Y | Y |
| Texas | Panola | Y | Y | N |
| Texas | Parker | Y | Y | N |
| Texas | Polk | Y | Y | N |
| Texas | Rains | Y | Y | N |
| Texas | Red River | Y | Y | N |
| Texas | Refugio | Y | Y | N |
| Texas | Robertson | Y | Y | N |
| Texas | Rockwall | Y | Y | N |
| Texas | Rusk | Y | Y | N |
| Texas | Sabine | Y | Y | N |
| Texas | San Augustine | Y | Y | N |
| Texas | San Jacinto | Y | Y | N |
| Texas | San Patricio | Y | Y | N |
| Texas | Shelby | Y | Y | N |
| Texas | Smith | Y | Y | N |
| Texas | Somervell | Y | Y | N |
| Texas | Tarrant | Y | Y | Y |
| Texas | Titus | Y | Y | N |
| Texas | Travis | Y | Y | N |
| Texas | Trinity | Y | Y | N |
| Texas | Tyler | Y | Y | N |
| Texas | Upshur | Y | Y | N |
| Texas | Van Zandt | Y | Y | N |
| Texas | Victoria | Y | Y | N |
| Texas | Walker | Y | Y | N |
| Texas | Waller | Y | Y | Y |
| Texas | Washington | Y | Y | N |
| Texas | Wharton | Y | Y | N |
| Texas | Williamson | Y | Y | N |
| Texas | Wilson | Y | Y | N |
| Texas | Wise | Y | Y | N |
| Texas | Wood | Y | Y | N |
| Utah | Davis | Y | Y | N |
| Utah | Salt Lake | Y | Y | N |
| Utah | Utah | Y | Y | N |
| Utah | Weber | Y | Y | N |
| Vermont | ALL | Y | Y | Y |
| Virginia | Accomack | Y | N | N |
| Virginia | Albemarle | Y | N | N |
| Virginia | Alleghany | Y | N | N |
| Virginia | Amelia | Y | N | N |
| Virginia | Amherst | Y | N | N |
| Virginia | Appomattox | Y | N | N |
| Virginia | Arlington | Y | Y | Y |
| Virginia | Augusta | Y | N | N |
| Virginia | Bath | Y | N | N |
| Virginia | Bedford | Y | N | N |
| Virginia | Bland | Y | N | N |
| Virginia | Botetourt | Y | N | N |
| Virginia | Brunswick | Y | N | N |
| Virginia | Buchanan | Y | N | N |
| Virginia | Buckingham | Y | N | N |
| Virginia | Campbell | Y | N | N |
| Virginia | Caroline | Y | N | N |
| Virginia | Carroll | Y | N | N |
| Virginia | Charles City | Y | Y | Y |
| Virginia | Charlotte | Y | N | N |
| Virginia | Chesterfield | Y | Y | Y |
| Virginia | Clarke | Y | N | N |
| Virginia | Craig | Y | N | N |
| Virginia | Culpeper | Y | N | N |
| Virginia | Cumberland | Y | N | N |
| Virginia | Dickenson | Y | N | N |
| Virginia | Dinwiddie | Y | N | N |
| Virginia | Essex | Y | N | N |
| Virginia | Fairfax | Y | Y | Y |
| Virginia | Fauquier | Y | N | N |
| Virginia | Floyd | Y | N | N |
| Virginia | Fluvanna | Y | N | N |
| Virginia | Franklin | Y | N | N |
| Virginia | Frederick | Y | N | N |
| Virginia | Giles | Y | N | N |
| Virginia | Gloucester | Y | N | N |
| Virginia | Goochland | Y | N | N |
| Virginia | Grayson | Y | N | N |
| Virginia | Greene | Y | N | N |
| Virginia | Greensville | Y | N | N |
| Virginia | Halifax | Y | N | N |
| Virginia | Hanover | Y | Y | Y |
| Virginia | Henrico | Y | Y | Y |
| Virginia | Henry | Y | N | N |
| Virginia | Highland | Y | N | N |
| Virginia | Isle of Wight | Y | N | N |
| Virginia | James City | Y | N | N |
| Virginia | King and Queen | Y | N | N |
| Virginia | King George | Y | N | N |
| Virginia | King William | Y | N | N |
| Virginia | Lancaster | Y | N | N |
| Virginia | Lee | Y | N | N |
| Virginia | Loudoun | Y | Y | Y |
| Virginia | Louisa | Y | N | N |
| Virginia | Lunenburg | Y | N | N |
| Virginia | Madison | Y | N | N |
| Virginia | Mathews | Y | N | N |
| Virginia | Mecklenburg | Y | N | N |
| Virginia | Middlesex | Y | N | N |
| Virginia | Montgomery | Y | N | N |
| Virginia | Nelson | Y | N | N |
| Virginia | New Kent | Y | N | N |
| Virginia | Northampton | Y | N | N |
| Virginia | Northumberland | Y | N | N |
| Virginia | Nottoway | Y | N | N |
| Virginia | Orange | Y | N | N |
| Virginia | Page | Y | N | N |
| Virginia | Patrick | Y | N | N |
| Virginia | Pittsylvania | Y | N | N |
| Virginia | Powhatan | Y | N | N |
| Virginia | Prince Edward | Y | N | N |
| Virginia | Prince George | Y | N | N |
| Virginia | Prince William | Y | Y | Y |
| Virginia | Pulaski | Y | N | N |
| Virginia | Rappahannock | Y | N | N |
| Virginia | Richmond | Y | Y | Y |
| Virginia | Roanoke | Y | N | N |
| Virginia | Rockbridge | Y | N | N |
| Virginia | Rockingham | Y | N | N |
| Virginia | Russell | Y | N | N |
| Virginia | Scott | Y | N | N |
| Virginia | Shenandoah | Y | N | N |
| Virginia | Smyth | Y | N | N |
| Virginia | Southampton | Y | N | N |
| Virginia | Spotsylvania | Y | N | N |
| Virginia | Stafford | Y | Y | Y |
| Virginia | Surry | Y | N | N |
| Virginia | Sussex | Y | N | N |
| Virginia | Tazewell | Y | N | N |
| Virginia | Warren | Y | N | N |
| Virginia | Washington | Y | N | N |
| Virginia | Westmoreland | Y | N | N |
| Virginia | Wise | Y | N | N |
| Virginia | Wythe | Y | N | N |
| Virginia | York | Y | N | N |
| Washington | Adams | Y | Y | N |
| Washington | Asotin | Y | Y | N |
| Washington | Benton | Y | Y | N |
| Washington | Chelan | Y | Y | N |
| Washington | Clallam | Y | Y | N |
| Washington | Clark | Y | Y | Y |
| Washington | Columbia | Y | Y | N |
| Washington | Cowlitz | Y | Y | Y |
| Washington | Douglas | Y | Y | N |
| Washington | Ferry | Y | Y | N |
| Washington | Franklin | Y | Y | N |
| Washington | Garfield | Y | Y | N |
| Washington | Grant | Y | Y | N |
| Washington | Grays Harbor | Y | Y | N |
| Washington | Island | Y | Y | N |
| Washington | Jefferson | Y | Y | N |
| Washington | King | Y | Y | Y |
| Washington | Kitsap | Y | Y | Y |
| Washington | Kittitas | Y | Y | N |
| Washington | Klickitat | Y | Y | N |
| Washington | Lewis | Y | Y | N |
| Washington | Lincoln | Y | Y | N |
| Washington | Mason | Y | Y | N |
| Washington | Okanogan | Y | Y | N |
| Washington | Pacific | Y | Y | N |
| Washington | Pend Oreille | Y | Y | N |
| Washington | Pierce | Y | Y | Y |
| Washington | San Juan | Y | Y | N |
| Washington | Skagit | Y | Y | N |
| Washington | Skamania | Y | Y | N |
| Washington | Snohomish | Y | Y | Y |
| Washington | Spokane | Y | Y | N |
| Washington | Stevens | Y | Y | N |
| Washington | Thurston | Y | Y | Y |
| Washington | Wahkiakum | Y | Y | N |
| Washington | Walla Walla | Y | Y | N |
| Washington | Whatcom | Y | Y | N |
| Washington | Whitman | Y | Y | N |
| Washington | Yakima | Y | Y | N |
| West Virginia | Putnam | Y | Y | N |
| West Virginia | Kanawha | Y | Y | N |
| West Virginia | Cabell | Y | Y | N |
| West Virginia | Wayne | Y | Y | N |
| West Virginia | Wood | Y | Y | N |
| Wisconsin | Brown | Y | Y | N |
| Wisconsin | Calumet | Y | Y | N |
| Wisconsin | Dane | Y | Y | N |
| Wisconsin | Dodge | Y | Y | N |
| Wisconsin | Door | Y | Y | N |
| Wisconsin | Fond du Lac | Y | Y | N |
| Wisconsin | Jefferson | Y | Y | N |
| Wisconsin | Kenosha | Y | Y | Y |
| Wisconsin | Kewaunee | Y | Y | Y |
| Wisconsin | Manitowoc | Y | Y | Y |
| Wisconsin | Milwaukee | Y | Y | Y |
| Wisconsin | Outagamie | Y | Y | N |
| Wisconsin | Ozaukee | Y | Y | Y |
| Wisconsin | Racine | Y | Y | Y |
| Wisconsin | Rock | Y | Y | N |
| Wisconsin | Sheboygan | Y | Y | Y |
| Wisconsin | Walworth | Y | Y | N |
| Wisconsin | Washington | Y | Y | Y |
| Wisconsin | Waukesha | Y | Y | Y |
| Wisconsin | Winnebago | Y | Y | N |
| Wyoming | ALL | N | N | N |

## TABLE C-1.  Average UST Temperatures Used for Stage I Calculations

|  | Jan | Feb | Mar | Apr | May | June | July | Aug | Sep | Oct | Nov | Dec |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Region 1 (NE) | 46 | 44 | 44 | 48 | 57 | 64 | 70 | 73 | 70 | 64 | 60 | 51 |
| Region 2 (SE) | 66 | 67 | 69 | 74 | 78 | 81 | 80 | 81 | 80 | 77 | 69 | 60 |
| Region 3 (SW) | 60 | 61 | 62 | 66 | 73 | 78 | 81 | 84 | 82 | 78 | 71 | 62 |
| Region 4 (MW) | 33 | 35 | 40 | 47 | 55 | 62 | 71 | 73 | 68 | 65 | 64 | 63 |
| Region 5 (W) | 50 | 52 | 62 | 66 | 73 | 76 | 80 | 83 | 86 | 84 | 73 | 60 |
| Region 6 (NW) | 49 | 50 | 50 | 52 | 57 | 62 | 67 | 72 | 68 | 60 | 49 | 42 |

Region 1: 	Alaska, Connecticut, Delaware, DC, Illinois, Indiana, Kentucky, Maine, Maryland, Massachusetts, Michigan, New Hampshire

New Jersey, New York, Ohio, Pennsylvania, Rhode Island, Vermont, Virginia, W. Virginia, Wisconsin

Region 2:	Alabama, Arkansas, Florida, Georgia, Hawaii, Louisiana, Mississippi, North Carolina, South Carolina, Tennessee

Region 3:	Arizona, New Mexico, Oklahoma, Texas

Region 4:	Colorado, Iowa, Kansas, Minnesota, Missouri, Montana, Nebraska, North Dakota, South Dakota, Wyoming

Region 5:	California, Nevada, Utah

Region 6:	Idaho, Oregon, Washington

Bold:	UST temperatures were not reported for some months for some Regions.  PES assumed an UST temperature.

Italics:	PES adjusted UST data as necessary

Table D-1.  Counties Modeled with Federal Reformulated Gasoline

| State | County |
| --- | --- |
| California | El Dorado |
| California | Los Angeles |
| California | Orange |
| California | Placer |
| California | Riverside |
| California | Sacramento |
| California | San Bernardino |
| California | San Diego |
| California | Solano |
| California | Sutter |
| California | Ventura |
| California | Yolo |
| Connecticut | ALL |
| Delaware | ALL |
| District of Columbia | District of Columbia |
| Illinois | Cook (Chicago Area) |
| Illinois | Du Page (Chicago Area) |
| Illinois | Grundy (Chicago Area) |
| Illinois | Kane (Chicago Area) |
| Illinois | Kendall (Chicago Area) |
| Illinois | Lake (Chicago Area) |
| Illinois | McHenry (Chicago Area) |
| Illinois | Will (Chicago Area) |
| Indiana | Lake |
| Indiana | Porter |
| Kentucky | Boone |
| Kentucky | Bullit (Portion) |
| Kentucky | Campbell |
| Kentucky | Jefferson |
| Kentucky | Kenton |
| Kentucky | Oldham (Portion) |
| Maryland | Baltimore |
| Maryland | Calvert |
| Maryland | Carroll |
| Maryland | Cecil |
| Maryland | Charles |
| Maryland | Frederick |
| Maryland | Harford |
| Maryland | Howard |
| Maryland | Kent |
| Maryland | Montgomery |
| Maryland | Prince George's |
| Maryland | Queen Anne's* |
| Massachusetts | ALL |
| New Hampshire | Hillsborough |
| New Hampshire | Merrimack |
| New Hampshire | Rockingham |
| New Hampshire | Strafford |
| New Jersey | ALL |
| New York | Bronx |
| New York | Dutchess |
| New York | Essex |
| New York | Kings |
| New York | Nassau |
| New York | New York |
| New York | Orange |
| New York | Putnam |
| New York | Queens |
| New York | Richmond |
| New York | Rockland |
| New York | Suffolk |
| New York | Westchester |
| Pennsylvania | Bucks |
| Pennsylvania | Chester |
| Pennsylvania | Delaware |
| Pennsylvania | Montgomery |
| Pennsylvania | Philadelphia |
| Rhode Island | ALL |
| Texas | Brazoria |
| Texas | Chambers |
| Texas | Collin |
| Texas | Dallas |
| Texas | Denton |
| Texas | Fort Bend |
| Texas | Galveston |
| Texas | Harris |
| Texas | Liberty |
| Texas | Montgomery |
| Texas | Tarrant |
| Texas | Waller |
| Virginia | Alexandria |
| Virginia | Arlington |
| Virginia | Charles City |
| Virginia | Chesapeake |
| Virginia | Chesterfield |
| Virginia | Colonial Heights |
| Virginia | Fairfax |
| Virginia | Fairfax County |
| Virginia | Falls Church |
| Virginia | Hampton |
| Virginia | Hanover |
| Virginia | Henrico |
| Virginia | Hopewell |
| Virginia | James City |
| Virginia | Loudoun |
| Virginia | Manassas |
| Virginia | Manassas Park |
| Virginia | New Port News |
| Virginia | Norfolk |
| Virginia | Portsmouth |
| Virginia | Powhatan |
| Virginia | Prince William |
| Virginia | Richmond |
| Virginia | Stafford |
| Virginia | Suffolk |
| Virginia | Virginia Beach |
| Virginia | Williamsburg |
| Virginia | York |
| Wisconsin | Kenosha |
| Wisconsin | Milwaukee |
| Wisconsin | Ozaukee |
| Wisconsin | Racine |
| Wisconsin | Washington |
| Wisconsin | Waukesha |

Table D-2.  Counties Modeled with Oxygenated Gasoline

(& Associated Fuel Modeling Parameters)

| State | County | MTBE Share % | Alcohol Blend Share % | Oxy Fuel Season |
| --- | --- | --- | --- | --- |
| Alaska | Anchorage | 0 | 100 | Nov – Feb |
| Arizona | Maricopa | 80 | 20 | Oct – Feb |
| Colorado | Adams | 75 | 25 | Nov - Feb |
| Colorado | Arapohoe | 75 | 25 | Nov - Feb |
| Colorado | Boulder | 75 | 25 | Nov - Feb |
| Colorado | Denver | 75 | 25 | Nov - Feb |
| Colorado | Douglas | 75 | 25 | Nov - Feb |
| Colorado | El Paso | 75 | 25 | Nov - Feb |
| Colorado | Jefferson | 75 | 25 | Nov - Feb |
| Colorado | Larimer | 75 | 25 | Nov - Feb |
| Connecticut | Fairfield | 90 | 10 | Nov - Feb |
| Minnesota | Anoka | 10 | 90 | Oct - Jan |
| Minnesota | Carver | 10 | 90 | Oct - Jan |
| Minnesota | Chisago | 10 | 90 | Oct - Jan |
| Minnesota | Dakota | 10 | 90 | Oct - Jan |
| Minnesota | Hennepin | 10 | 90 | Oct - Jan |
| Minnesota | Isanti | 10 | 90 | Oct - Jan |
| Minnesota | Ramsey | 10 | 90 | Oct - Jan |
| Minnesota | Scott | 10 | 90 | Oct - Jan |
| Minnesota | Washington | 10 | 90 | Oct - Jan |
| Minnesota | Wright | 10 | 90 | Oct - Jan |
| Montana | Missoula | 0 | 100 | Nov - Feb |
| Nevada | Clark | 0 | 100 | Oct - Mar |
| Nevada | Washoe | 95 | 5 | Oct- Jan |
| New Jersey | Bergen | 95 | 5 | Nov - Feb |
| New Jersey | Essex | 95 | 5 | Nov - Feb |
| New Jersey | Hudson | 95 | 5 | Nov - Feb |
| New Jersey | Hunterdon | 95 | 5 | Nov - Feb |
| New Jersey | Middlesex | 95 | 5 | Nov - Feb |
| New Jersey | Monmouth | 95 | 5 | Nov - Feb |
| New Jersey | Morris | 95 | 5 | Nov - Feb |
| New Jersey | Ocean | 95 | 5 | Nov - Feb |
| New Jersey | Passaic | 95 | 5 | Nov - Feb |
| New Jersey | Somerset | 95 | 5 | Nov - Feb |
| New Jersey | Sussex | 95 | 5 | Nov - Feb |
| New Jersey | Union | 95 | 5 | Nov - Feb |
| New York | Bronx | 95 | 5 | Nov - Feb |
| New York | Kings | 95 | 5 | Nov - Feb |
| New York | Nassau | 95 | 5 | Nov - Feb |
| New York | New York | 95 | 5 | Nov - Feb |
| New York | Orange | 95 | 5 | Nov - Feb |
| New York | Putnam | 95 | 5 | Nov - Feb |
| New York | Queens | 95 | 5 | Nov - Feb |
| New York | Richmond | 95 | 5 | Nov - Feb |
| New York | Rockland | 95 | 5 | Nov - Feb |
| New York | Suffolk | 95 | 5 | Nov - Feb |
| New York | Westchester | 95 | 5 | Nov - Feb |
| Oregon | Clackamus | 1 | 99 | Nov - Feb |
| Oregon | Jackson | 1 | 99 | Nov - Feb |
| Oregon | Josephine | 1 | 99 | Nov - Feb |
| Oregon | Klamath | 1 | 99 | Nov - Feb |
| Oregon | Multnomah | 1 | 99 | Nov - Feb |
| Oregon | Washington | 1 | 99 | Nov - Feb |
| Oregon | Yamhill | 1 | 99 | Nov - Feb |
| Texas | El Paso | 15 | 85 | Nov - Feb |
| Utah | Utah | 20 | 80 | Nov - Feb |
| Washington | Clark | 1 | 99 | Nov - Feb |
| Washington | Spokane | 1 | 99 | Sep - Feb |
| Wisconsin | St. Croix | 10 | 90 | Oct - Jan |

Source:	Documentation For the Draft 1999 National Emissions Inventory for Criteria Pollutant - Onroad Sources,

Table 8, E.H. Pechan & Associates, Inc., October 2001.

Table E-1.  State-Level Comparison of PES and NEI VOC Emission Estimates

|  | Stage I Total (tpy) |  |  | Splash Loading (tpy) |  |  | Submerged Loading (tpy) |  |  | Submerged Balanced (tpy) |  |  | UST B&E (tpy) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| State | PES | NEI | PES | NEI | PES | NEI | PES | NEI | PES | NEI |
| Alabama |  | 2,005 |  | 0 |  | 0 | 1,016 | 10 | 1,137 | 1,197 |
| Alaska |  | 0 | 1,443 | 0 |  | 0 |  |  | 143 | 0 |
| Arkansas |  | 5,632 | 8,344 | 0 | 1,935 | 0 | 435 | 0 | 1,032 | 935 |
| Arizona |  | 8,659 | 1,046 | 0 | 493 | 0 |  | 0 | 687 | 636 |
| California |  | 0 | 460 | 0 | 223 | 0 | 4,279 | 0 | 7,245 | 4,446 |
| Colorado |  | 4,299 | 4,606 | 0 |  | 0 | 400 | 0 | 994 | 1,062 |
| Connecticut |  | 397 | 642 | 0 |  | 0 | 465 | 0 | 697 | 678 |
| DC |  | 0 |  | 0 |  | 0 | 47 | 82 | 62 | 6 |
| Delaware |  | 0 |  | 0 |  | 426 | 174 | 0 | 219 | 184 |
| Florida |  | 11,129 | 43,140 | 0 |  | 5,900 | 533 | 0 | 3,660 | 2,402 |
| Georgia |  | 6,965 | 17,502 | 0 |  | 0 | 754 | 470 | 2,321 | 1,292 |
| Hawaii |  | 0 |  | 0 | 1,138 | 0 |  | 0 | 177 | 0 |
| Iowa |  | 5,727 | 8,777 | 0 |  | 0 |  | 0 | 780 | 792 |
| Idaho |  | 2,228 | 3,065 | 0 |  | 0 |  | 0 | 285 | 306 |
| Illinois |  | 13,769 |  | 0 |  | 0 | 1,805 | 0 | 2,312 | 0 |
| Indiana |  | 0 | 10,268 | 9,718 |  | 0 | 560 | 227 | 1,569 | 0 |
| Kansas |  | 5,633 | 8,506 | 0 |  | 0 | 0 | 0 | 748 | 775 |
| Kentucky |  | 3,208 | 10,387 | 522 |  | 3,156 | 179 | 359 | 1,070 | 1,050 |
| Louisiana |  | 325 | 8,252 | 2,822 |  | 0 | 498 | 36 | 1,124 | 2,382 |
| Massachusetts |  | 0 |  | 16 |  | 3,032 | 931 | 2,278 | 1,239 | 1,369 |
| Maryland |  | 0 |  | 0 |  | 0 | 769 | 1,955 | 1,010 | 1,924 |
| Maine |  | 0 |  | 1,576 |  | 524 | 261 | 63 | 374 | 330 |
| Michigan |  | 439 | 2,363 | 2,732 | 4,000 | 9,842 | 1,043 | 247 | 2,420 | 2,414 |
| Minnesota |  | 8,752 |  | 0 | 5,304 | 0 |  | 0 | 1,203 | 1,206 |
| Missouri |  | 10,547 | 9,346 | 0 | 1,268 | 0 | 342 | 0 | 1,488 | 1,536 |
| Mississippi |  | 6,187 | 10,233 | 0 |  | 0 |  | 0 | 711 | 854 |
| Montana |  | 2,055 |  | 0 | 1,154 | 0 |  | 0 | 263 | 282 |
| North Carolina |  | 22,556 |  | 0 |  | 0 | 1,760 | 0 | 2,109 | 1,342 |
| North Dakota |  | 1,459 |  | 0 | 797 | 0 |  | 0 | 190 | 202 |
| Nebraska |  | 3,316 | 4,659 | 0 |  | 0 |  | 0 | 421 | 460 |
| New Hampshire |  | 2,924 | 241 | 0 |  | 0 | 87 | 0 | 275 | 117 |
| New Jersey |  | 578 |  | 0 |  | 0 | 1,566 | 0 | 1,970 | 231 |
| New Mexico |  | 3,352 | 3,688 | 0 |  | 0 | 108 | 0 | 457 | 592 |
| Nevada |  | 2,242 |  | 0 | 2,346 | 0 |  | 0 | 461 | 175 |
| New York |  | 33,531 | 2,776 | 0 |  | 0 | 2,009 | 0 | 2,690 | 0 |
| Ohio |  | 6,662 | 10,936 | 0 |  | 0 | 1,325 | 0 | 2,620 | 1,142 |
| Oklahoma |  | 232 | 9,795 | 0 |  | 0 | 321 | 0 | 1,104 | 952 |
| Oregon |  | 0 | 3,529 | 0 |  | 0 | 315 | 0 | 779 | 0 |
| Pennsylvania |  | 0 |  | 0 |  | 13,251 | 2,064 | 4,046 | 2,558 | 2,211 |
| Rhode Island |  | 0 | 221 | 385 |  | 116 | 160 | 38 | 233 | 25 |
| South Carolina |  | 11,269 | 15,560 | 0 |  | 0 |  | 0 | 1,168 | 1,057 |
| South Dakota |  | 1,644 | 2,277 | 0 |  | 0 |  | 0 | 211 | 227 |
| Tennessee |  | 15,105 | 15,921 | 0 |  | 0 | 496 | 375 | 1,808 | 1,468 |
| Texas |  | 0 | 40,648 | 0 | 20 | 0 | 2,795 | 27,364 | 5,815 | 5,023 |
| Utah |  | 407 | 1,447 | 0 |  | 0 | 311 | 0 | 543 | 75 |
| Virginia |  | 0 | 1,829 | 0 | 3,316 | 7,314 | 771 | 1,811 | 1,894 | 2,107 |
| Vermont |  | 3,627 |  | 0 |  | 0 | 101 | 0 | 149 | 0 |
| Washington |  | 0 | 4,104 | 0 |  | 0 | 785 | 0 | 1,419 | 0 |
| Wisconsin |  | 5,707 | 4,352 | 0 |  | 0 | 60 | 0 | 308 | 876 |
| West Virginia |  | 2,459 | 2,780 | 2,116 |  | 1,294 | 600 | 0 | 1,225 | 480 |
| Wyoming |  | 1,379 | 1,524 | 0 |  | 0 |  | 0 | 155 | 189 |
| Grand Total |  | 216,403 | 274,667 | 19,888 | 21,994 | 44,855 | 30,125 | 39,363 | 65,532 | 47,006 |