Overview of this module: The list below summarizes the content and teaching methods for this module. This is for your reference. Do not cover this with the participants.

| Module 3: Review of Setup Practices 20 Minutes |  |
| --- | --- |
| - 3-1: Module 3: Review of Setup Practices - 3-2: Keep Dust Within the Containment - 3-3: Vertical Containment - 3-4: Overview of Interior Setup Steps - 3-5: Overview of Exterior Containment Steps | Key message: Keep the dust in the work area and make it easier to cleanup. |

Slide 3-1: Module 3: Review of Setup Practices

- Module objective: The purpose of this module is to review the simple setup techniques that will stop the spread of lead-contaminated dust to non-work areas during both interior and exterior work.

- This module covers interior and exterior containment requirements and methods. Discuss the definition of, and need for, containment of the work area as required in the RRP Rule.

Slide 3-2: Keep Dust Within the Containment

Review the information on the slide.

- Ask the participants for examples of dusty jobs. Ask them about the reasoning behind their examples – why one particular job creates and/or spreads more dust than another.  Jobs that generate more dust will require larger areas of floor or ground to be covered during work. Discussion about the amount of dust generated by jobs in this module should lead into thinking about how best to design containment to guard against the spread of that dust (and debris) from the work area to other areas of a building or property.

- Emphasize to the participants that the work practices and equipment used on a job, as well as the size of the job are all factors that affect the amount of dust generated and, therefore, the size and placement of containment. For example, vigorous hand sanding can create enough dust to migrate farther than 6 feet from the work surface.

- Emphasize that minimizing dust generation through work practice modification and containment planning is helpful in controlling dust produced by high-dust renovation activities.

Slide 3-3: Vertical Containment

- Review the information on the slide and in the student notes.

Slide 3-4: Overview of Interior Containment Setup Steps

- Use this slide to review the five setup steps for establishing interior containment. Do not go into detail about the steps here. This is meant to provide reinforcement of the five steps to interior containment so that participants will have a structure to organize the information.

Slide 3-5: Overview of Exterior Containment Steps

- Use this slide to highlight the three setup practices. Do not go into detail about the steps here.

- Note that vertical containment set up within 10 feet of the renovation area provides the option to reduce the ground containment size, to allow flexibility for designing effective containment strategies.