July 14, 2023

MR JEFFREY ROBINSON

BRANCH CHIEF AIR PERMITS AND MONITORING

US ENVIRONMENTAL PROTECTION AGENCY REGION 6

1201 ELM ST STE 500

DALLAS TX  75270

# Re:	Executive Director’s Response to EPA Objection

Renewal

Permit Number:  O3785

Intercontinental Terminals Company LLC

ITC Pasadena Terminal

Pasadena, Harris County

Regulated Entity Number:  RN106119175

Customer Reference Number:  CN601470222

Dear Mr. Robinson:

On June 30, 2022, the U.S. Environmental Protection Agency (EPA) Region 6 Office signed a letter identifying objections to the issuance of the proposed federal operating permit for the above referenced site. In accordance with Title 30 Texas Administrative Code § 122.350 (30 TAC § 122.350), the Texas Commission on Environmental Quality (TCEQ) may not issue the permit until the objections are resolved.

The TCEQ has completed the technical review of your objections and offers the enclosed responses to facilitate resolution of the objections. In addition, the attached responses to the objections describe the changes, if applicable, that have been made to the revised proposed permit and supporting statement of basis (SOB).  The revised proposed permit and SOB are attached for your review.

Consistent with 30 TAC §122.350, please provide an indication of your acceptance or assessment of the responses and resolutions to the objections as soon as possible.  After receipt of your acceptance to the responses and resolutions to the objections, TCEQ will issue the proposed permit.  Thank you for your cooperation in this matter.  Please contact Mr. Mark Meyer at (512) 239-0445 if you have any questions concerning this matter.

Sincerely,

Samuel Short, Deputy Director

Air Permits Division

Office of Air

Texas Commission on Environmental Quality

cc:	Mr. Michael Gaudet, Environmental Compliance Manager, Intercontinental Terminals Company LLC, La Porte

Chief Health Inspector, Health Department, City of Pasadena, Pasadena

Director, Harris County, Pollution Control Services, Pasadena

Air Section Manager, Region 12 - Houston

Air Permit Section Chief, U.S. Environmental Protection Agency, Region 6-Dallas (Electronic copy)

Enclosure:	TCEQ Executive Director’s Response to EPA Objection

Proposed Permit

Statement of Basis

Project Number:  31086

# Executive Director’s Response to EPA Objection

Permit Number O3785

The Texas Commission on Environmental Quality (TCEQ) Executive Director (ED) provides this Response to EPA’s Objection to the renewal of the Federal Operating Permit (FOP) for Intercontinental Terminals Company LLC, ITC Pasadena Terminal, Permit No. O3785, Harris County, Texas.

Background

Procedural Background

The Texas Operating Permit Program requires that owners and operators of sites subject to 30 Tex. Admin. Code (TAC) Chapter 122 obtain a FOP that contains all applicable requirements to facilitate compliance and improve enforcement.  The FOP does not authorize construction or modifications to facilities, and it does not authorize emission increases.  To construct or modify a facility, the responsible party must have the appropriate new source review authorization.  If the site is subject to 30 TAC Chapter 122, the owner or operator must submit a timely FOP application for the site and ultimately must obtain the FOP to operate.  Intercontinental Terminals Company LLC applied to the TCEQ for a renewal of the FOP for the ITC Pasadena Terminal located in Pasadena, Harris County on August 26, 2020, and notice was published on July 28, 2021, in The Pasadena Citizen.  A public hearing was held on December 9, 2021, and the public comment period ended on December 9, 2021.  In response to the public comments, a response to comments (RTC) letter and a proposed permit was sent to EPA on May 13, 2022. During the EPA review period starting May 17, 2022, TCEQ received an objection to the permit from EPA on June 30, 2022.

In accordance with state and federal rules, the permit renewal may not be issued until TCEQ resolves EPA’s objections.

Description of Site

Intercontinental Terminals Company LLC owns and operates the ITC Pasadena Terminal, located at 1030 Ethyl Road in Pasadena, Harris Texas 77503. ITC’s Pasadena Terminal is a for-hire bulk liquid storage facility and marine terminal. The terminal handles fuels, commodity chemicals, crude oil and petroleum products which are stored in storage tanks and transferred in and out of the terminal tankage by pipeline, marine vessel, railcar, and tank truck.

The following responses follow the references used in EPA’s objection letter.

EPA Objection 1: Failure to Include Adequate Monitoring Requirements to Assure Compliance

with Permitted Emission Limits.

The title V permit incorporates by reference minor NSR permit 95754 which was amended on April 16, 2021. Although this NSR permit received numerous comments during the public comment period which ran from December 18, 2020 to January 29, 2021, TCEQ made no changes to the NSR permit based on comments that were received. TCEQ received many comments on the proposed title V permit that make similar claims on the inadequacy of monitoring in the NSR permit. TCEQ failed to address these comments any further and directed the public to the response to comments document for the NSR permit for more detailed responses. TCEQ made no changes to the title V permit and did not add any supplemental monitoring to the title V permit. EPA has reviewed both response to comments documents and finds that the NSR permit does not contain adequate monitoring to assure compliance with the terms and conditions of the permit and provides specific examples below where additional monitoring is needed.

  - NSR permit 95754 includes Special Condition (SC) 2 which establishes VOC emission caps for three groups of units. These groups are based on three separate projects that were reviewed by TCEQ retrospectively. The groups each contain the three vapor combustors (EPNs VC-001, VC-002, and VC-003) and the flare (EPN FL-001). There is no explanation on how compliance with each of these emission caps is to be determined across these EPNs. To make this even more confusing, the MAERT includes additional emission caps that apply to the vapor combustors and flares. Can TCEQ please explain why these emission units have multiple emission caps and how compliance is demonstrated to include the calculation methodology for each one?

  - The NSR permit refers to SC 46 for determining compliance with the emission limits in SC 2. These conditions are inadequate for determining compliance with the stated VOC emission caps. SC 46 states “Monthly compliance calculation for limits on VOC potential to emit”. Permit emission limit compliance should be based upon actual emissions and not on calculated potential to emit (PTE) levels. The permit must include adequate monitoring to assure compliance with the permitted limits and be monitored/calculated on measurable parameters for accurate calculation of emissions. TCEQ needs to confirm if the permit condition intended to state “potential to emit” as a permit limit and if so, to explain for the record why PTE would be used for compliance purposes and how this limit can be practically enforced.

  - SC 11 states “Storage tank emissions shall be estimated consistent with the methodology employed in the permit amendment application (PI-1 dated December 27, 2018).” EPA looked on the TCEQ Central File Room Online and found an application dated December 2018 with a PI-1 that appeared to have the date of December 21, 2018. The referenced document was difficult to locate and was found under content ID 5684402. EPA suggests providing this information or clarifying where the information can be found. The application states “ITC proposes to establish emission caps for the proposed facilities rather than individual throughput limits due to the varying nature of products and customer markets at this terminal. Specifically, rather than limiting product throughputs, ITC proposes to manage the facilities included in this application such that the permitted emission limits are not exceeded. Managing to emissions caps allows ITC the operational flexibility to respond to market changes and customer demands”. However, there is no explanation on how ITC manages the facilities to meet the emission caps. Can TCEQ please provide more detail on what monitoring and recordkeeping are required to ensure compliance with these emission caps at all times and how the facility also ensure compliance with the lb/hr limits in the MAERT for each tank? If the company is relying on the limits in Attachment 1 of the permit for these purposes, have they provided data to show that it is physically impossible to exceed the emission limit caps and lb/hr emission limits for each tank? What data has been provided to ensure that the operational limits in Attachment 1 are sufficient to ensure compliance with both the emission caps and the lb/hr limits?

  - SC 17.C. This condition refers to a maximum permitted loading rate. Does this condition refer to the operational limits in Attachment 1? Can TCEQ please identify where this maximum permitted loading rate can be found?

  - SC 19. This SC refers back to a “test conducted per Condition 17”. Should this condition be referring to Condition 18? Can TCEQ please confirm?

  - SC 20. This SC states the dates of ship collection efficiency tests. All of these collection efficiency tests were performed in 2015 and 2016. The SC goes on to state that “…additional ship collection efficiency testing is not required”. What basis is TCEQ using to support no additional periodic testing requirements? Can TCEQ or the applicant provide data to show that there is no decrease in the collection efficiency since the three tests were performed?

  - SC 26. States “The loading rate shall be recorded after loading any marine vessel, truck, or railcar with liquid if the loading rate is restricted under Attachment 1”. Can TCEQ explain for the record why the loading rate should not be recorded in all cases? Can TCEQ also explain how this condition may be related to SC 17 which referred to a maximum permitted loading rate?

  - SC 46.A. This SC refers to AP-42, Chapter 5 for calculating emissions from loading activities. There is no indication which EPNs are associated with loading activities or if one of the three groups rom SC2 is specifically subject to this requirement. The condition at (2) refers to equation 1, however, no equation is shown or provided to be used for calculating emissions. Can TCEQ clarify this permit condition to identify the equation used for calculating emissions and to identify the EPNs that are subject to this emission calculation methodology?

  - SC 46.A.3 and 4. These conditions refer to the emissions associated with the transfer of a product, but the equation for calculation is not provided nor is it indicated for what EPNs this requirement is applicable to. Can TCEQ please include the equation that appears to be missing and provide information on what EPNs should have emissions calculated using this methodology?

  - SC 46.E. This special condition appears to have combined two unrelated permit conditions, one for wastewater emissions and one for vapor combustion units. The second part of the condition states “Monthly compliance calculation for limits on pollutants for annual control device potential to emit (EPNs VC-001, VC-002, VC-003, and FL-001)”. Should this condition be a stand-alone condition? Also, can TCEQ explain why compliance would be based on a potential to emit and not the VOC emission limit prescribed in the table of Special Condition 2 of NSR 95754?

- SC 46.G. This condition relates to flare emissions. Is this flare equipped with continuous gas flow and composition monitors? How is the waste gas component contribution and the heat content of the waste gas constituents determined? Can TCEQ please clarify for the record what monitoring equipment is available for the waste gas flow to the flare and how any such data is used to calculate emissions.

TCEQ Response to EPA Objections 1a, 1b, 1c, 1d, 1e, 1f, 1g, 1h, 1i, 1j and 1k: The ED notes NSR Permit No. 95754, renewal and amendment Project 345739 issued on June 8, 2023 addresses the above referenced EPA objections. Proposed Permit incorporates by reference NSR Permit No. 95754, issued June 8, 2023 in the New Source Review authorizations table listed on page 140.

Detailed description of the changes made in Project 345739 is documented in the technical review document (WCC content ID 6557967). It includes adequate monitoring requirements to assure compliance with permitted emission limits. This document also contains a technical analysis of the nonattainment and PSD review applicability, including a retrospective review. A detailed response to each of the EPA Objection 1 is listed below.

TCEQ Response to EPA Objection 1a: In regard to NSR Permit No. 95754, Special Condition (SC) 2 which establishes VOC emission caps for three groups of units, the permit holder maintains records of product specific movements, individual MSS activities, and the associated product physical properties as required by NSR Permit No. 95754, Special Condition Nos. 11, 27, 34, and 43 which is utilized to calculate monthly emissions to demonstrate compliance with the applicable emission limit(s). The calculated emissions are then summed and compared to the appropriate MAERT limits per the applicable requirements of NSR Permit No. 95754, Special Condition Nos. 2, 11, 27, 34, 44, and 46.

Regarding EPNs VC-001, VC-002, VC-003, and FL-001, TCEQ provides the following explanation on how compliance with each of these emission caps is to be determined across these EPNs: Each VCU and flare (EPNs VC-001, VC-002, VC-003, and FL-001) has individual short-term emissions limit applicable to Groups A, B, and C based worst-case emission calculations.

In addition, the VCUs and Flare have an annual emission cap for Groups A and B (VOC limits are separate for the two groups) and an individual annual cap for Group C. The annual emission cap is the summation of the following activities that route to control: Controlled Loading, Hose Venting, Wastewater System, Roof Landing (Routine and MSS) for each Group Facility. Each activity contributing to the emissions represented under EPNs VC-001, VC-002, VC-003, and FL-001 are bound the by the emission representations made in the application and Special Conditions related to each activity. Emission calculations conservatively assumes that all waste streams can be routed to any VCU, and the control shall achieve a 99.9% control efficiency (as per Special Condition 46) and an appropriate capture efficiency for loading type.

Please also see ITC Response-to-Comments (Comment #6) for NSR Project 294773 (WCC Doc. 5596361), Technical Review for NSR Project 219916 (WCC Doc. 4945304) and Technical Review for NSR Project 235891 (WCC Doc. 4888296).

Non-Aggregation of Group Facility B

A project aggregation review was conducted for TCEQ NSR Projects 211610 and 219916 (Issued January 16, 2015) for possible nonattainment circumvents due to requested authorization of Group Facility ID B as a distinct project from Group Facility ID A. In the review, a 2009 EPA action on Project Aggregation was used as a source in combination with information supplied by the Applicant – specifically, construction of Group A Facilities had commenced during the time Group B Facilities were proposed, the nature of customer contracts for for-hire bulk marine terminals, the overall project scope, and it was concluded that project aggregation was unwarranted.

Specifically, the determination was based on the EPA’s interpretation in the 2009 action as follows. “When activities are undertaken three or more years apart, there is less of a basis that they have a substantial technical or economic relationship because the activities are typically part of entirely different planning and capital funding cycles. The fact that the earlier activities were constructed and operated independently for such a long period of time tends to support a determination that the latter activities are technically and economically unrelated and independent from the other earlier constructed activities. Even if activities are related, once three years have passed, it is difficult to argue that they are substantially related and constitute a single project. We note that the selection for a 3-year timeframe is long enough to ensure a reasonable likelihood that the presumption if independence will be valid but is short enough to maintain a useful separation between relevant construction cycles, consistent with industry practice”.

Non-Aggregation of Group Facility C

A Project Aggregation review was conducted for TCEQ NSR Project 243313 (Issued July 11, 2017) due to possible nonattainment circumvention in regard to project increases of VOC emissions from the proposed amendment. In Project 243313, the Applicant proposed to authorize an expansion of storage capacity, including new storage tanks and associated piping and fugitive emissions (Known as Group Facility C).

A case-by-case aggregation review was conducted due to possible nonattainment circumvention concerns. The TCEQ used information provided by the Applicant to determine technical and economical relatedness of the projects.

TCEQ reviewed contemporaneous documents provided by ITC, on the two projects (Group Facility B and C), which was generated during the 2013-2016 time period, including: agreements with terminal customers, engineering firms and insurance underwriters, transactions of ITC’s board of directors; construction progress and start-up notifications provided to the TCEQ; and ITC’s prior statements to TCEQ concerning its plans for operation. Considering the overall timespan of a construction project (from initial “open season” discussions with customers through start of operations of new facilities), there was no indication that ITC had improperly avoided major NSR review through non-aggregation. All Group B facilities have been authorized in January 2015 will have started operations before any actual construction work began on the Group C facilities.

Emission summary for plantwide emissions and for each of the three facility groups (Group IDs A, B and C) authorized by this permit is described on pages 1-3 of the technical review document (WCC content ID 6557967). There is an emissions cap for each Tank Group (A, B, C) and Controlled Routine/MSS Activities. As each Group Facility ID contains different EPNs, each Group is bounded by the emission caps found on the MAERT along with Special Condition No. 2.

For Groups A and B facilities, the emissions cap located in Special Condition No. 2 is the total of the Storage Tank Emissions Cap along with the respective Fugitive, Uncontrolled Loading, Uncontrolled Roof Landing, Hose Venting, Hose Draining, Uncontrolled MSS, Controlled Routine Activities (Loading, Hose Venting, Roof Landing) and Controlled MSS Cap. This cap was included to ensure that each Group would not be above major source threshold and to prevent circumvention of nonattainment review applicability.

For Group C facilities, the emissions cap located in Special Condition No. 2 is the total of the Storage Tank Emissions Cap along with the respective Fugitive, Uncontrolled Loading, Uncontrolled Roof Landing, Hose Venting, Hose Draining, Controlled (Loading, Hose Venting, Roof Landing, MSS) Activities Cap, and Uncontrolled MSS Cap.

As all Storage Tanks are Internal Floating Roof (IFR) tanks, all tank EPNs represent controlled emissions. For Group Facilities A and B, EPN VC-001, VC-002, VC-003, and FL-001 represent the controlled emissions cap for loading, hose venting, wastewater system, and MSS activities. There is also a Controlled MSS Emission Cap (EPNs MSS-CONT-A and MSS-CONT-B). For Group Facility C, EPN LOAD-CONT-C and MSS-CONT-C represent emissions routed to control devices.

Monitoring requirements for the VCUs are described on page 6 of the technical review document (WCC content ID 6557967). The Vapor Combustors (VCUs) are used to control a variety of routine and MSS related sources. This includes emissions from the IFR Tanks (working/breathing emissions, landing events), marine and land-based loading, hose venting and draining, Wastewater Collection, and MSS activities, as applicable. The monthly emissions from VCUs are the summation of the calculated emissions from these controlled activities utilizing the appropriate control efficiency as part of Special Condition No. 46.A(5).

TCEQ Response to EPA Objection 1b: In regard to NSR Permit No. 95754, SC 46 for determining compliance with the emission limits in SC 2, the ED notes SC 46 establishes the emission calculation methodologies upon which the permit holder is required to demonstrate compliance with the Maximum Allowable Emission Rate Table (MAERT). The phrase “potential to emit” is not intended to refer to anything other than the MAERT and as defined in accordance with Title 30 Texas Administrative Code Chapter 122.10(20).

TCEQ Response to EPA Objection 1c: In regard to locating information related to storage tank emissions referenced in SC 11, the ED notes all publicly accessible files associated with the April 16, 2021, issuance of NSR Permit No. 95754 are available TCEQ Central File Room Online. These records may be accessed by selecting NSR permit records database and entering 95754 as primary ID (same as NSR permit number) and 294773 as secondary ID (same as NSR project number).

Regarding EPA’s request to provide more detail on  monitoring and recordkeeping requirement to ensure compliance with emission caps, please refer to NSR Permit 95754, SC 11 (for Tanks), SC 46 (for Facility wide) emissions and TCEQ’s Source Analysis & Technical Review documents for each NSR project (e.g., WCC content ID 5596193), and the enforceable representations in the applicant’s permit applications (e.g., WCC content ID 5665298). More specifically, the permit holder maintains records of product specific movements, individual MSS activities, and the associated product physical properties as required by NSR Permit No. 95754, Special Condition Nos. 11, 27, 34, and 43 which is utilized to calculate monthly emissions to demonstrate compliance with the applicable emission limit(s). The calculated emissions are then summed and compared to the appropriate MAERT limits per the applicable requirements of NSR Permit No. 95754, Special Condition Nos. 2, 11, 27, 34, 44, and 46.

Regarding ensuring compliance with both the tank emission caps and the lb/hr limits imposed on each individual tank, TCEQ notes each storage tank is limited by both individual short- and long-term emission rates. The storage tank calculations were based on AP-42 Chapter 7 (Liquid Storage Tanks, November 2006). The calculations assume that the highest emitting product that will be stored for each tank. ITC shall comply with the individual tank emission limits, including maximum throughput listed for each storage tank emission calculation provided in Table A-1A (WCC Document 5684402).

In addition, Special Condition 11 requires the permit holder to maintain an emissions record that includes calculated emissions of VOC from all storage tanks and loading operations within the previous calendar month and the past consecutive 12-month period. The records include tank capacity gallons of liquid transferred, name of the material stored/loaded, VOC molecular weight, average temperature, vapor pressure, and throughput for the previous month and year-to-date. These parameters are utilized in the same storage tank emission calculations as referenced.

Due to the operational nature of for-hire bulk terminals and for further assurance that emissions from individual storage tanks will not exceed permitted emission limits, an annual Tank Cap for each facility group (A, B, and C) was authorized, which is the summation of the individual storage tank annual emissions. Storage tanks shall comply with both the individual and group emission limits and recordkeeping is required as part of Special Condition 11.

TCEQ Response to EPA Objection 1d: In regard to NSR Permit 95754, SC 17.C, this condition refers to a maximum permitted loading rate and the operational limits in Attachment 1. The maximum permitted loading rates are included in Table 2 of Attachment 1 of the NSR permit.

TCEQ Response to EPA Objection 1e: In regard to NSR Permit 95754, SC 19, TCEQ confirms this condition should refer back to SC 18. This change is being made in pending Amendment/Renewal for NSR permit 95754 (Project 345739) and upon issuance applicant intends to submit a Title V revision project to incorporate by reference the revised NSR Permit 95754 (Project 345739) into FOP O3785.

TCEQ Response to EPA Objection 1f: In regard to NSR Permit 95754, SC 20 related to ship collection efficiency tests, TCEQ has revised its previous guidance regarding marine loading collection efficiencies for ocean-going marine vessels [see TCEQ Marine Loading Collection Efficiency Guidance (texas.gov)]. SC 18 reflects current TCEQ requirements based on receipt, review, and approval of ship collection efficiency data provided by the regulated community (60+ tests from 2015-2018). NSR Permit No. 95754, SC 17 establishes the applicable requirements developed by TCEQ based on the above noted ship collection efficiency testing document for ongoing compliance demonstration purposes.

TCEQ Response to EPA Objection 1g: In regard to NSR Permit 95754, SC 26 related to loading rate, the ED notes loading rates are physically limited and the additional restrictions for various products are identified to ensure there are no adverse off-property impacts. Attachment 1 limits the loading of all authorized compounds. SC 17 refers to loading requirements onto ocean-going, inerted marine vessels. SC 17.C refers to maximum permitted loading rates. The maximum permitted loading rates are included in Table 2 of Attachment 1 of the permit for marine, truck, and rail loading operations.

TCEQ Response to EPA Objection 1h: In regard to NSR Permit 95754, SC 46.A related to calculating emissions from loading activities, the ED notes loading emission points and source names are contained in the MAERT of the permit, by Group A, B, or C (EPNs: DOCK-1, DOCK-2, DOCK-3, DOCK-4, RACK-1, RACK-3, RACK-5, LOAD-UNC-A, LOAD-UNC-B, LOAD-CONT-C, and LOAD-UNC-C). The referenced equation is defined by AP-42, Section 5.2 (July 2008 edition). The equation is included as SC 46.A(2) in NSR permit 95754 (Project 345739), issued June 8, 2023.

TCEQ Response to EPA Objection 1i: In regard to NSR Permit 95754, SC 46.A.3 and 4 related to emissions associated with the transfer of a product, the ED notes the equations were inadvertently left out of the NSR permit are listed below. They are included as SC 46.A(3) and (4) in NSR permit 95754 (Project 345739), issued June 8, 2023.

The following EPNs are applicable to the emission calculation methodology required in Special Condition No 46.A(3) and 46.A(4)

SC 46.A(3): EPNs LOADUNC-A, LOADUNC-B, LOADUNC-B

SC 46.A(4): EPNs VC-001, VC-002, VC-003, FL-001, LOAD-CONT-C.

TCEQ Response to EPA Objection 1j: In regard to NSR Permit 95754, SC 46.E related to an apparent combination of two unrelated permit conditions, the ED agrees that the noted text in SC 46.E reflects a formatting error which requires updating the special condition to reflect a new SC with current SC 46.F – H being applicable requirements associated with the VCUs and flare. The revised SC 46.F-H has been included in pending Amendment/Renewal for NSR permit 95754 (Project 345739) and upon issuance applicant intends to submit a Title V revision project to incorporate by reference the revised NSR Permit 95754 (Project 345739) into FOP O3785.

SC 46 establishes the emission calculation methodologies upon which the permit holder is required to demonstrate compliance with the Maximum Allowable Emission Rate Table (MAERT). The phrase “potential to emit” is not intended to refer to anything other than the MAERT, which is defined in accordance with Title 30 Texas Administrative Code Chapter 122.10(20).

TCEQ Response to EPA Objection 1k: In regard to NSR Permit 95754, SC 46.G related to flare emissions, the ED notes that SC 31.D requires the permit holder to install continuous flow monitors on flares that provide a record of vent stream flow to the flare. SC 46.G specifies how to calculate the emissions. Please refer to TCEQ Response to EPA Objection 1a for additional information.

As part of Special Condition 31, the Backup Flare (EPN FL-001) is required to meet the requirements listed in 40 CFR §60.18, including requirements for the minimum heating value (using calculations found in 60.18 (f)(3)) and maximum tip velocity (60.18 (f)(6)). The waste gas constituents and contribution from the stream is determined using oxygen analyzers (TCEQ NSR Project 219916) to assure that the heat content of the vapors is above the minimum heating value. This ensures that the flare is properly combusting and meeting the required destruction efficiency as part of Special Condition 46.A(5) and considered best available control during the permit review.

In regard to calculation of flare emissions, flare is equipped with a waste gas flow meter to demonstrate compliance with the specifications of 40 CFR §60.18 as part of Special Condition No. 31. ITC utilizes oxygen analyzers to determine hydrocarbon concentrations. This information is used to calculate the flare emissions with the provided control efficiency as Part of Special Condition No. 46. Per SC 46.G, the emissions will first be determined by calculating the waste gas component contribution using the highest heat content of the waste gas constituent(s). Then the natural gas component will be calculated based on the feed rate required by Special Condition No. 31.D. Total flare emissions will be calculated by summing the waste gas component and the natural gas component for each flare firing period.

EPA Objection 2: Lack of Assurance to Comply with Emission Limits and Operating

Requirements.

Commenters identified several areas of concern regarding the monitoring requirements in NSR permit 95754 which is incorporated by reference into the title V permit. In responding to comments, TCEQ explained that Texas has a two-permit system and only NSR permits authorize air emissions under 30 TAC Chapter 116 and/or under 30 TAC Chapter 106 for PBRs. RTC Response 5, 6, 9, 10, and 11. TCEQ stated in their response to some of the public comments, “This is a Title V permitting action and therefore challenges related to emission limits or changes in emission limits in NSR permits are beyond the scope of this Title V review”. Many of these comments were related to how emissions are calculated, the monitoring established by the NSR permit, or how compliance with emission limits is determined. TCEQ does note in response to these comments that similar comments were made during the public comment period for the NSR permit and provides the content ID to obtain the more detailed response that was provided in that response to comments. TCEQ should consider reevaluating comments as they are received on title V permits even if similar comments were addressed under review of the underlying NSR permit. The EPA does view monitoring, recordkeeping, and reporting adequacy to be part of the title V permitting process and will therefore continue to review whether a title V permit contains adequate monitoring, recordkeeping, and reporting provisions sufficient to assure compliance with the terms and conditions established in the preconstruction permit. EPA has identified conditions in the NSR permit in the comment above where we question if the permit does contain adequate monitoring requirements. The statutory obligations to ensure that each title V permit contains “enforceable emission limitations and standards” supported by “monitoring . . . requirements to assure compliance with the permit terms and conditions,” 42 U.S.C. § 7661c(a), (c), apply independently from and in addition to the underlying regulations and permit actions that give rise to the emission limits and standards that are included in a title V permit.” See South Louisiana Methanol Order at 10; Yuhuang II Order at 7-8; PacifiCorp-Hunter Order at 16, 17, 18, 18 n.33, 19; Big River Steel Order at 17, 17 n.30, 19 n.32, 20. Therefore, regardless of the monitoring, recordkeeping, and reporting initially associated with a minor NSR permit or PBR, TCEQ has a statutory obligation independent of the process of issuing those permits to evaluate monitoring, recordkeeping, and reporting in the title V permitting process to ensure that these terms are sufficient to assure compliance with all applicable requirements and title V permit terms. Sierra Club v. EPA, 536 F.3d 673 (D.C. Cir. 2008); see Motiva Order at 25-26.

In responding to this objection, TCEQ should amend the permit and permit record as necessary to specify monitoring, recordkeeping, and reporting requirements that assure compliance with the emission limits and operational requirements in NSR permit 95754. If the title V permit, the underlying NSR permit, or the enforceable representations in the application already contain adequate terms to assure compliance with the NSR permit, then TCEQ should amend the permit and/or permit record to identify such terms and explain how these requirements assure compliance with these emission limits and operational requirements for an individual emission unit, process area, or site-wide where such permit applies site-wide.

TCEQ Response to EPA Objection 2: Proposed permit O3785 does contain adequate monitoring, recordkeeping, and reporting provisions sufficient to assure compliance with the terms and conditions established in the NSR permit 95754. Please refer to pages 118 through 129 of the proposed permit to review the unit specific periodic monitoring (PM) and compliance assurance monitoring (CAM) required under Title V and 30 TAC Chapter 122.

In addition, please refer to TCEQ’s Source Analysis & Technical Review (e.g., WCC content ID 5596193) document page 7 for periodic monitoring and compliance assurance monitoring required under Title V and 30 TAC Chapter 122 that cross references special conditions in NSR Permit 95754.

In regard to monitoring recordkeeping, and reporting requirements under NSR Permit 95754 and demonstration of compliance with applicable emission limits, please refer to TCEQ’s Response to EPA Objections 1a, 1b, 1c, 1d, 1e, 1f, 1g, 1h, 1i, 1j and 1k for additional details. Enforceable representations in the applicant’s permit applications regarding emission calculations, emission limits, etc. may be found in WCC content ID 5665298 document.

EPA Objection 3: Failure to Include Specific Enforceable Terms and Conditions for Applicable

NESHAP Requirements.

Under title V of the CAA, the EPA’s part 70 regulations, and TCEQ’s EPA-approved title V program rules, every title V permit must include all applicable requirements that apply to a source, as well as any permit terms necessary to assure compliance with these requirements. E.g., 42 U.S.C. § 7661c(a). The CAA requirement to include all applicable requirements (including NESHAP regulations) in a title V permit can be satisfied using IBR in certain circumstances. The CAA§ 504 requirement to include all applicable requirements in a title V permit can be satisfied using IBR in certain circumstances. See, e.g., White Paper 2 at 40 (explaining how IBR can satisfy the requirements of CAA § 504). In all cases where IBR is employed, the title V permit must contain references that are "detailed enough that the manner in which the referenced material applies to the facility is clear and is not reasonably subject to misinterpretation." White Paper 2 at 37. Moreover, "Where only a portion of the referenced document applies, . .. permits must specify the relevant section of the document." Id.

Requirements of a NESHAP that apply to emission units at a facility are "applicable requirements." 40 C.F.R. § 70.2; 30 TAC§ 122.10(2)(I)(ii). The EPA has previously addressed the manner by which NESHAP (or NSPS) requirements may be incorporated by reference into title V permits. In 1999, the EPA rejected suggestions that states have the discretion to include high-level citations to an entire NESHAP subpart, stating: "The permit needs to cite to whatever level is necessary to identify the applicable requirements that apply to each emissions unit or group of emission units (if generic grouping is used), and to identify how those units will comply with the requirements. " The EPA has also objected to title V permits that have attempted to IBR NESHAP (or NSPS) requirements without providing sufficient detail to determine the specific requirements that apply to emission units at the source. Specifically, in the Tesoro Order, the EPA found that references to sections of a NESHAP that were not associated with specific emission units created ambiguity and applicability questions that "render[ ed] the Permit unenforceable as a practical matter and incapable of meeting the Part 70 standard that it assure compliance with all applicable requirements." In the Matter of Tesoro Refining, Order on Petition No. IX-2004-06 at 8 (March 15, 2005). Additionally, in the ETC Waha Order, the EPA found that high-level references to an entire NSPS subpart, which did not identify the specific requirements within the subpart that applied to each emissions unit, similarly failed to comply with the CAA. In the Matter of ETC Texas Pipeline, Ltd. Waha Gas Plant, Order on Petition No. VI-2020-3 at 17-19 (January 28, 2022)

Text from TCEQ’s EPA-approved title V regulations is arguably more specific than language found in 40 CFR 70.6(a)(1); however, the underlying principle is the same and explicitly requires citation to the appropriate applicable requirements. 30 Tex. Admin. Code § 122.142(b)(2)(B), requires Title V permits to include “the specific regulatory citations in each applicable requirement … identifying the emission limitations and standards; and … the monitoring, recordkeeping, reporting, and testing requirements associated with the emission limitations and standards … sufficient to ensure compliance with the permit.”

The permits Applicable Requirements Summary Table cites to 40 C.F.R. §§ 63.11089, 63.11088(a), or 63.11087(a) under the “Emission Limitation, Standard or Equipment Specification Citation” column and indicates: “The permit holder shall comply with the applicable limitation, standard, and/or equipment specification requirements of 40 CFR Part 63, Subpart BBBBBB” (or a similar variation of this text with respect to testing, monitoring, recordkeeping, or reporting requirements). The requirements under the citations provided might be the only applicable part of the NESHAP that is applicable, but that is not clear from the permit. The entirety of 40 C.F.R. § 63.11089 is indicated as applicable to some sources by the [G] indicated next to the citation. Looking at this entire citation, it appears to contain monthly monitoring (leak inspection under §§ 63.111000 is cited), recordkeeping (maintaining a logbook under §63.11089(b), (c), and (d) which also refers to §§ 63.11094 and 63.11095. It also specified reporting under § 63.11089(f) which refers to § 63.11093 and §63.11089(g) which is also for recordkeeping. The title V permit Special Terms and Conditions 14 and 15 identify a list of the citations in 40 CFR Part 63 Subpart BBBBBB that the permit holder must comply with, but this list doesn’t inform what emission units are subject to these requirements.

TCEQ must revise the Permit to ensure that it is unambiguous as to which requirements of NESHAP subpart BBBBBB are applicable to emission units at ITC. Specifically, TCEQ must revise the ITC Title V permit to include the specific emission limitations and standards applicable to each emission unit subject to 40 CFR Part 63 Subpart BBBBBB, as well as the specific monitoring and testing, recordkeeping, and reporting requirements applicable to each emission unit subject to 40 CFR Part 63 Subpart BBBBBB.

TCEQ Response to EPA Objection 3: TCEQ notes that the permit holder has submitted an OP-REQ3 form identifying sources of emissions at ITC’s Pasadena site subject to 40 CFR Part 63, Subpart BBBBBB for inclusion in the proposed Title V permit. The unit-specific detailed applicable requirements have been included in the Title V permit. Since the flare, identified by Unit ID FL-001, is being used as a control device to comply with this subpart, the flare requirements in § 63.11 have also been added to the Title V permit. The permit holder has also provided a completed Form OP-UA7 identifying the attributes of the flare under 40 CFR Part 63, Subpart A for applicability determinations. The unit-specific applicable requirements have been identified and included in the proposed Title V permit.

EPA Objection 4: Failure to Include all Applicable Requirements.

The proposed title V permit fails to meet the requirements of CAA § 504(a) for “(e)ach permit issued under this subchapter shall include enforceable emission limitations and standards, . . . and such other conditions as are necessary to assure compliance with applicable requirements of this chapter, including the requirements of the applicable implementation plan.” TCEQ’s definition of “applicable requirement” (found at 30 TAC § 122.10(2)) includes an extensive list of federal and state provisions. Minor NSR permits and Permits by Rule (PBRs) are included in TCEQ’s definition of applicable requirement and are applicable requirements as defined under 40 CFR § 70.2. The application for the title V permit includes the May 2020 version of the PBR Supplemental Tables and on Table B and D, the applicant shows that EGEN-4, EGEN-5, EGEN-6, and EGEN-10 are authorized by PBR 106.511. However, the title V permit does not include this PBR on the New Source Review Authorization References Table on page 134 nor on the following pages on the New Source Review Authorization References by Emissions Unit table. TCEQ should verify if the PBR needs to be added to the title V permit or if the PBR has been consolidated by incorporation into the NSR permit and update the title V permit as necessary.

TCEQ Response to EPA Objection 4: NSR Permit 95754 was amended on April 16, 2021, to authorize EGEN-4, EGEN-5, EGEN-6, and EGEN-10. As a result of the amendments, the applicant submitted an updated OP-PBRSUP form dated June 17, 2022, to remove PBR 106.511 because the amendments to NSR Permit 95754 incorporates permit by rule (PBR) 106.511. Therefore, EGEN-4, EGEN-5, EGEN-6, and EGEN-10 are now authorized by NSR 95754 instead of PBR 106.511. The Title V permit document correctly lists that these engines are authorized by NSR Permit 95754 and no changes are needed to the New Source Review Authorization References by Emission Unit table in the permit. The Special Terms and Conditions (ST & C) of the Title V permit have been updated to include this change (ST & C 19) for clarity.