February 23, 2024

MR JEFFREY ROBINSON

BRANCH CHIEF AIR PERMITS AND MONITORING

US ENVIRONMENTAL PROTECTION AGENCY REGION 6

1201 ELM ST STE 500

DALLAS TX  75270

# Re:	Executive Director’s Response to EPA Objection

Renewal

Permit Number:  O1668

Shell Chemical LP

Chemical Plant

Deer Park, Harris County

Regulated Entity Number:  RN100211879

Customer Reference Number:  CN601405145

Dear Mr. Robinson:

On November 2, 2022, the U.S. Environmental Protection Agency (EPA) Region 6 Office signed a letter identifying objections to the issuance of the proposed federal operating permit for the above referenced site. In accordance with Title 30 Texas Administrative Code § 122.350 (30 TAC § 122.350), the Texas Commission on Environmental Quality (TCEQ) may not issue the permit until the objections are resolved.  In addition, the letter identifies certain additional concerns.  The TCEQ understands that the additional concerns are provided for information only, and do not need to be resolved in order to issue the permit.

The TCEQ has completed the technical review of your objections and offers the enclosed responses to facilitate resolution of the objections. In addition, the attached responses to the objections describe the changes, if applicable, that have been made to the revised proposed permit and supporting statement of basis (SOB).  The revised proposed permit and SOB are attached for your review.

Consistent with 30 TAC §122.350, please provide an indication of your acceptance or assessment of the responses and resolutions to the objections as soon as possible.  After receipt of your acceptance to the responses and resolutions to the objections, TCEQ will issue the proposed permit.  Thank you for your cooperation in this matter.  Please contact Ms. Camilla Widenhofer at (512) 239-1028 if you have any questions concerning this matter.

Sincerely,

Samuel Short, Deputy Director

Air Permits Division

Office of Air

Texas Commission on Environmental Quality

cc:	Ms. Nicole Roper, Environmental Team Lead, Shell Chemical LP, Deer Park

Director, Harris County, Pollution Control Services, Pasadena

Air Section Manager, Region 12 - Houston

Air Permit Section Chief, U.S. Environmental Protection Agency, Region 6-Dallas (Electronic copy)

Enclosure:		TCEQ Executive Director’s Response to EPA Objection

Proposed Permit

Statement of Basis

Project Number:  28179

# Executive Director’s Response to EPA Objection

Permit Number O1668

The Texas Commission on Environmental Quality (TCEQ) Executive Director (ED) provides this Response to EPA’s Objection to the renewal of the Federal Operating Permit (FOP) for Shell Chemical LP, Chemical Plant, Permit No. O1668, Harris County, Texas.

Background

Procedural Background

The Texas Operating Permit Program requires that owners and operators of sites subject to 30 Tex. Admin. Code (TAC) Chapter 122 obtain a FOP that contains all applicable requirements to facilitate compliance and improve enforcement.  The FOP does not authorize construction or modifications to facilities, and it does not authorize emission increases.  To construct or modify a facility, the responsible party must have the appropriate new source review authorization.  If the site is subject to 30 TAC Chapter 122, the owner or operator must submit a timely FOP application for the site and ultimately must obtain the FOP to operate.  Shell Chemical LP applied to the TCEQ for a renewal of the FOP for the Chemical Plant located in Deer Park, Harris County on October 1, 2018, and notice was published on June 20, 2021, date in Pasadena Citizen.  The public comment period ended on July 20, 2021.  Public comments were received by TCEQ on July 21, 2021.  TCEQ submitted a response to comments and Proposed Permit for EPA review on September 20, 2022.  TCEQ received an objection to the proposed permit from EPA on November 2, 2022.

In accordance with state and federal rules, the permit renewal may not be issued until TCEQ resolves EPA’s objections.

Description of Site

Shell Chemical LP owns and operates the Chemical Plant, located at 5900 Highway 225 in Deer Park, Harris Texas 77536-2434.

Shell Chemical LP’s Deer Park Chemical Plant is primarily engaged in the production of olefins, heavy olefins, aromatics, phenol, and acetone.  These base chemicals or raw material chemicals are typically sold to other chemical companies that transform them into thousands of consumer products ranging from plastics to building materials.  These products are transferred via pipeline, marine loading, and rail and tank truck loading.

The following responses follow the references used in EPA’s objection letter.

EPA Objection 1: Objection for Failure to Identify Specific Applicable Requirements under 40 CFR Part 61 and Part 63, National Emission Standards for Hazardous Air Pollutants (NESHAP). Underlying NSR permits 3219/PSDTX974, 3179, and 3219 indicate that the facilities shall comply with 40 CFR Part 61 Subparts J, V, and FF. The Applicable Requirements Summary table shows that GRP-MON- FE as subject to Part 61 Subpart J for Benzene, yet the table identifies “none” for monitoring and testing requirements, recordkeeping requirements, and for reporting requirements. The textual description states “Each owner or operator subject to this subpart shall comply with the requirements of 40 CFR 61, Subpart V – National Emission Standard for Equipment Leaks (Fugitive Emission Sources)”. The permit shield on page 459 indicates that for GRP-MON-FE, that a permit shield exists for Part 61 Subpart V and the basis of the determination is stated as “Equipment subject to the requirements of 40 CFR 63, Subpart FFFF which is also subject to the provisions of 40 CFR 61 are required to comply with the provisions of 40 CFR 63, Subpart FFFF”. The applicable requirements summary table does include applicable requirements from 40 CFR Part 63 Subpart FFFF for GRP- MON-FE for HAPs.

The Title V permit should contain references that are detailed enough that the manner in which the referenced requirements apply to the facility is clear and is not reasonably subject to misinterpretation. See In the Matter of Tesoro Refining, Order on Petition No. IX-2004-06 (March 15, 2005) at 8; see also White Paper Number 2 for Improved Implementation of The Part 70 Operating Permits Program, 34-38 (March 5, 1996). In the case of the Shell Chemical Plant permit, the permit does not contain enough information or detailed enough citations to determine how the specific requirements of the NESHAP requirements apply to the facility. Can TCEQ confirm that no additional requirements for Benzene are applicable for GRP-MON-FE and clarify the VOC requirements for the facility?

TCEQ Response to Objection 1: As listed in the Applicable Requirements Summary table (page 247) of the proposed permit, GRP-MON-FE unit is subject to 40 CFR Part 61, Subpart J. However, Subpart J does not have specific monitoring, recordkeeping and reporting requirements because it requires the permit holder to comply with 40 CFR Part 61, Subpart V requirements. GRP-MON-FE unit is therefore subject to both 40 CFR Part 61, Subpart V and 40 CFR Part 63, Subpart FFFF fugitive requirements. Citation 63.2535(k) in 40 CFR Part 63, Subpart FFFF states that if the source is subject to both 40 CFR Part 61, Subpart V and 40 CFR Part 63, Subpart FFFF, the owner or operator may elect to apply 40 CFR Part 63, Subpart FFFF to all such equipment. Accordingly, applicant has elected to comply with 40 CFR Part 63, Subpart FFFF requirements. Therefore, citation 63.2535(k) was manually added as a related standard for GRP-MON-FE unit in the ARS table for 40 CFR Part 61, Subpart J (index 61JALL) to clarify that applicant is complying with the requirements in 40 CFR Part 63, Subpart FFFF instead of 40 CFR Part 61, Subpart V in order to comply with 40 CFR Part 61, Subpart J. In addition, a permit shield for 40 CFR Part 61, Subpart J for GRP-MON-FE unit documents that applicant is electing to comply with 40 CFR Part 63, Subpart FFFF instead of 40 CFR Part 61, Subpart V.

EPA Objection 2: Inadequate Monitoring for Flares. As you are aware from petition orders recently published for other TCEQ title V permits, EPA has discovered that meeting the requirements of 40 CFR § 60.18 does not always account for certain problems that can reduce combustion efficiency, such as those caused by excess steam or air assistance to the flare. Steam- and air-assisted flares for certain waste gas streams are susceptible to performance problems that may reduce VOC destruction efficiency below 98%. EPA seeks clarification on a few items in an effort to determine if the draft permit requires sufficient monitoring to ensure compliance with the stated 98% / 99% destruction efficiencies and the related lb/hr and TPY emission rates.

Special Condition 9.A. through 9.E. of NSR permit PSDTX974/3219 imposes permit conditions on flares covered by the NSR permit (OP-2 Elevated Flare, OP-3 Elevated Flare, and OP-3 Ground Flare). Condition 9.E. states, “In lieu of the monitoring requirements in Paragraph D of this condition, the flare may comply with the monitoring and testing requirements of 30 TAC §§ 115.725(d), effective December 23, 2004, as specified in 30 TAC Chapter 115 Subchapter H, Division 1”. None of the conditions in Special Condition 9 contain any required monitoring for the amount of assist steam or air that is sent to the flares. Nor does the permit identify the minimum destruction efficiency required of the flares. Special Condition 6 refers to applications dated November 20, 2017, and July 19, 2018, for documentation on how the emission limits were established. The Application dated November 20, 2017, identifies a destruction/removal efficiency (DRE) of 98%/99% for the ground flare. Is the 99% DRE for compounds containing no more than 3 carbons and the 98% DRE for all other compounds per TCEQ Tier 1 BACT? Neither application reference contained a destruction efficiency for the elevated flares. Can TCEQ provide additional details that identify what the destruction efficiency is for each flare and show the conditions in permit PSDTX974 that ensure compliance with the specified destruction efficiency of each flare?

Special Conditions 14 and 15 of NSR permit 3179 contains requirements for the HIPA Flare (EPN A1333). This flare is steam assisted according to condition 14.C. The permit does not specify a destruction efficiency for the flare. Can TCEQ provide additional details that identify what the destruction efficiency is for the flare and show the conditions in permit 3179 that ensure compliance with the specified destruction efficiency of the flare? It is important that the NSR permits contain adequate monitoring to ensure compliance with any VOC emission limits specified by TCEQ in the permit on both a short term and long-term basis to be practically enforceable. Where the NSR permit does not contain adequate monitoring, it is important that the title V permit fill the gap by requiring any needed monitoring, recordkeeping, and reporting to assure compliance,

TCEQ Response to Objection 2: NSR permit 3219 (project 365078) issued 02/07/2024 and NSR permit 3179 (project 365077) issued 02/07/2024, contain revised special conditions 9 and 14 respectively to include sufficient monitoring requirements for flare units to demonstrate compliance with the applicable requirements, including compliance with the specified destruction efficiency. NSR permits 3219 and 3179 are incorporated by reference in the proposed permit (New Source Review Authorization References table at page 475).

EPA Objection 3: Objection for Failure to Include all Applicable Requirements. EPA reviewed the Permit By Rule (PBR) Supplemental Table that was included with the permit application. The New Source Review (NSR) Authorization References by Emissions Unit table including registration numbers next to emission units authorized by registered PBRs conforms with TCEQ's EPA-approved regulations, 30 TAC § 122.142(2)(B)(i), as well as the agreements underpinning the EPA's approval of the Incorporation By Reference (IBR) of PBRs - namely that "PBRs will be cited to the lowest level of citation necessary to make clear what requirements apply to the facility." However, upon review of all registered PBRs associated with the Shell Chemical Plant, several effective PBRs appear to be omitted from title V permit O1668 (identified below by PBR rule number and registration number) and there are PBRs listed in the PBR Supplemental Table that are shown to be Cancelled on the TCEQ website and are not included in the title V permit:

  - Shown to be cancelled:

- 98473

- 109247

- 146255

  - Missing from title V permit

- 164009 for 106.478 for K306R1

- 167075 for TOL910 (106.261, 106.478) and OP2FUG (106.261)

- 168056 for K307R1 for 106.478

- 168460 for TUT604 and TUT605 for 16.261

- 168727 for HT2FUG, HT3FUG, and OP2FUG for 106.261 and 106.262

Because the title V permit omits direct reference to certain source-specific requirements (e.g., certified emission limits) derived from registered PBRs, it is not clear that the Shell Chemical Plant permit currently includes or incorporates all applicable requirements for the facility, as required by the CAA, the EPA's regulations, TCEQ's regulations, the agreements underlying the EPA’s approval of IBR in Texas, and the EPA’s longstanding position concerning IBR. Pursuant to 40 CFR § 70.8(c)(1), EPA objects to the issuance of the Shell Chemical Plant title V permit since it is not in compliance with the requirements of CAA § 504(a) and 40 CFR § 70.6(a)(1) & (3).

In response to this objection, TCEQ should review the listed PBR registrations to determine if they are applicable to the Shell Chemical Plant authorized by title V permit O1668. If so, the title V permit and/or PBR Supplemental Table should be revised to include all registered PBRs. EPA understands that the Shell Refinery and Shell Chemical Plant previously operated under a common Regulated Entity Reference Number (RN100211879), but the refinery now operates under a different RN number since the company and TCEQ have worked to separate their operating permits. To the extent that any of the aforementioned PBR registrations apply solely to units authorized by the Shell Refinery under title V permit O1669, please ensure that those PBR registrations are included in the O1669 permit or associated PBR Supplemental Table for that title V permit.

TCEQ Response to Objection 3: As noted by EPA, the Shell Refinery and Shell Chemical Plant previously operated under a common Regulated Entity Reference Number (RN100211879), but the refinery now operates under a different RN number since the company and TCEQ have worked to separate their operating permits. Applicant had submitted a minor revision in May 2022 to incorporate updated preconstruction authorizations resulting from the divestment from Deer Park Refinery (Permit O1669).  That minor revision project is now incorporated in this renewal project, and all preconstruction authorizations have been updated in the proposed permit (New Source Review Authorization References table at page 475).

Updated preconstruction authorizations resulting from the divestment also included a revised PBR Supplemental Table (OP-PBRSUP) dated January 31, 2023, which is included in the application file for FOP O1668/project 28179. Missing PBR registration numbers 164009, 167075, 168056, 168460, and 168727 have been added to the OP-PBRSUP table dated January 31, 2023, and the proposed permit (New Source Review Authorization References by Emissions Unit at pages 477 through 494).

EPA Objection 4: Objection for Failure to Include Adequate Monitoring for PBRs. EPA finds that the draft Response to Comments (RTC) and permit could benefit from more detailed monitoring information in the PBR Supplemental Table. The goal of this supplemental table is to explicitly incorporate monitoring/recordkeeping requirements and reduce ambiguity with respect to how emissions are calculated and how compliance is determined. When TCEQ relies on Table D to incorporate additional monitoring requirements, the monitoring and recordkeeping terms must be sufficient to assure compliance with emission limitations and operational requirements. When records are identified as being maintained, it would be practical to include a frequency for the recordkeeping. See below for a few relevant examples of units that need more detailed information - this list of examples is not comprehensive:

  - Unit V392 – This unit is subject to 40 CFR Part 63 Subparts F, G, and H according to the PBR registration application. The monitoring listed in the PBR supplemental table is not very descriptive and the Applicable Requirements Summary table in the title V permit only shows applicable requirements for 30 TAC Chapter 115 and 40 CFR Part 63 Subpart G.

  - Unit ACNLOAD – This unit shows it is subject to 40 CFR Part 63 Subpart G according to the PBR registration application. The PBR supplemental table only states “loading dates, volumes will be monitored”. The PBR registration contained more information on how the emissions were to be calculated which included that the vapors are collected and sent to a flare for control and that emissions were estimated using chapter 5 of AP-42 for VOC emissions and the NOx and CO were calculated using AP-42 emission factors for steam assisted flares, In addition, the title V permit contains additional monitoring requirements under 30 TAC Chapter 115.

  - Various tanks subject to 106.472, 106.473, 106.476, 106.478, 16.261, and 106.262 – Many tanks listed “Monitor tank throughput and material stored”. It is preferable that it be stated that this information will be recorded on a monthly basis and that calculation of a rolling 12- month total for emissions is calculated using whatever calculation methodology has been chosen by the company (AP-42 or other reference material)

TCEQ Response to Objection 4: The Title V permit and Statement of Basis were updated to include identifiable monitoring, recordkeeping, and reporting necessary to assure compliance with the emission limits and standards in the PBRs as follows:

- The PBR Supplemental Table (OP-PBRSUP) dated January 31, 2023, which is included in a Title V application becomes part of the permit record. A reference to the PBR Supplemental Table was added in the Special Term and Condition 25 in the proposed permit to include the text “(including the terms and conditions which include monitoring, recordkeeping, and reporting identified in registered PBRs and permits by rule identified in the PBR Supplemental Tables dated January 31, 2023, in the application for project 28179)”.

- New Source Review Authorization References by Emissions Unit table in the proposed permit (pages 477-494) has been updated to include the emission units listed in the OP-PBRSUP table.

- New Source Review Authorization table (New Source Review Authorization References table at page 475) was updated to list NSR Permit Numbers 3179 and 3219, effective 02/07/2024.

- The statement of basis (SOB) was updated to revise the insignificant activities list and add a reference to the PBR Supplemental Table, and the Special Term and Condition 25 listed in the FOP.

- The SOB was updated to identify insignificant activities and emission units not required to be addressed in the operating permit application.

- Regarding V392 being subject to 40 CFR Part 63 Subparts F, G, and H according to the PBR registration application, whereas the Applicable Requirements Summary (ARS) table in the title V permit only showing applicable requirements for 30 TAC Chapter 115 and 40 CFR Part 63 Subpart G, TCEQ notes that HON MACT standards have overlapping subparts. Based on OP-UA15 form submitted by the applicant TCEQ determined that “Owners or operator is required to comply only with the requirements of 40 CFR Part 63, Subpart G” which is listed in the ARS. Subsequently, in response to EPA’s objection, applicant has reconfirmed that this unit is only subject to MACT G as listed in the ARS and MACT F and H regulations do not apply as was stated in the PBR representation.

- Regarding ACNLOAD unit, TCEQ notes that although PBR 50423 was not recently revised, OP-PBRSUP was revised to contain the following PBR monitoring requirements “Keep records of volume of spent ACN loaded into tank trucks each time loading occurs. During loading vapors are collected and sent to a flare (OP2ELFLA, OP3ELFLA, and OP3GRFLA). The flares are monitored according to the requirements in Special Condition Nos. 9 and 18 of NSR Permit No. 3219 issued on 03/12/2021 found in Appendix B of the Title V permit to demonstrate compliance with the limits in 106.4. Loading and the flare control device are also monitored according to the requirements found in Applicable Requirements Summary table in the Title V permit.”

  - Regarding applicability determination for ACNLOAD unit being subject to MACT G, applicant has stated “although the current operations of Unit ID ACNLOAD are not subject to the requirements of 40 CFR Part 63, Subpart G, we request that operation as a HON Group 2 transfer rack be included in the permit. The attached Form OP-UA04, Table 7a has been completed to reflect this operating scenario.”

  - Per applicant’s request, the proposed permit has been revised to add applicable MACT G citations for ACNLOAD unit in the ARS (page 96).

EPA Objection 5: Objection to Improperly Incorporating Confidential Emission Calculations. In reviewing the PBR Supplemental Table, EPA found it necessary to look at the application for some PBR registrations to review the monitoring requirements. When doing so, it was found that at least one PBR that was incorporated by reference into the title V permit contained confidential information in the application. PBR registration 112344 referred to emission calculations contained in the confidential file.

The CAA limits the types of information that may be treated as confidential under title V, and therefore withheld from the public. As a general matter, some information may be protected as a trade secret under section 114(c) of the CAA. 42 U.S.C. § 7414(c). However, the CAA specifically limits this protection: “The contents of a [title V] permit shall not be entitled to [confidential] protection under section [114(c)].” 42 U.S.C. § 7661b(e). Regarding the contents of a title V permit, the CAA further requires that “Each permit issued under this subchapter shall include enforceable emission limitations and standards, … and such other conditions as are necessary to assure compliance with applicable requirements ….” 42 U.S.C. § 7661c(a). EPA regulations further require that the contents of a title V permit include “emissions limitations and standards, including those operational requirements and limitations that assure compliance with all applicable requirements at the time of permit issuance.” 40 C.F.R. §70.6(a)(1). Further, “terms and conditions in a part 70 permit… are enforceable by the Administrator and citizens under the Act.” 40 C.F.R. §70.6(b)(1). Additionally, information which is considered emission data, as well as standards or limitations, are also not entitled to confidential treatment. See CAA § 114(c) (“other than emissions data”); 40 C.F.R. §2.301(f). It is not clear in the current permit record whether the title V permit contains all the necessary emissions limitations and standards, including those emission methodologies and inputs, operational requirements and limitations that assure compliance with all applicable requirements, or if some of that information may be inappropriately treated as confidential resulting in the title V permit not complying with the CAA.

In responding to this objection, TCEQ should review the PBR registration 112344 in order to determine if the relevant emissions data, standards, or limitations and emissions calculation methodologies and inputs required to determine compliance needs to remain in the confidential portion of the permit record. TCEQ should conduct a reevaluation to ensure that this information is neither part of the title V permit, establishing binding, enforceable permit terms, nor considered emissions data for purposes of CAA 503(e) and 40 C.F.R. § 2.301(a)(2)(i)(B). If TCEQ can establish that this confidential information is not part of the title V permit operational limit, emissions data, or contains emissions calculation methodologies and inputs to determine compliance, TCEQ will still need to establish the basis or details in the permit record for why it is not necessary to enforce these as a term or condition of the title V permit. Otherwise, TCEQ should take appropriate action to ensure the emission calculations (including calculation methodologies and inputs required) and/or throughput and operation limits are transferred into non-confidential portion of the permitting record.

TCEQ Response to Objection 5:  As noted in the technical summary document (WCC content ID 6464534) for PBR registration number 112344, project number 353371, applicant submitted a revision application to make the emission calculations previously contained in the confidential file to be non-confidential and publicly accessible. Therefore, emission calculations included in revised PBR 112344 may now be accessed by the public via CFR Online.