Surface Water Scenario Development for the Pesticide Root Zone Model (PRZM) for Modeling Pesticide Exposure in Outdoor Nurseries: Supplement to Metadata.

GSA Contract No. GS-00F-0019L

Order No. EP06H000149

Technical Direction #4

Submitted to:

U.S. Environmental Protection Agency

Office of Pesticide Programs, Environmental Fate and Effects Division

Ariel Rios Bldg.; 1200 Pennsylvania Ave. NW
Washington, DC 20460

ATTN:  Milwina Locklear (7507C), Project Officer

(703) 305-7327

Locklear.Milwina@epamail.epa.gov

Submitted by:

Syracuse Research Corporation

6225 Running Ridge Road

North Syracuse, NY 13212

Submitted November 13, 2006

SRC TR-06-081

Table of Contents

1.	PURPOSE	4

2.	TECHNICAL APPROACH	4

2.1.	Selection of Scenario Locations	4

2.2.	Scenario Development Guidance	5

2.3.	Land Use Data	5

2.4.	Soils Data	5

2.5.	Curve Numbers	6

2.6.	RUSLE Parameters	6

2.7.	Weather Station Selection	7

2.8.	Cropping Dates	8

3.	IMPORTANT UNCERTAINTIES, LIMITATIONS, AND DATA GAPS	8

4.	CONTACTS	10

5.	REFERENCES	11

Appendix A. Soil Maps and Major Land Resource Areas	12

Appendix B. Geospatial Analysis Completed by SERA	18

Appendix C. Aerial Photography of Ornamental Nurseries.	23

List of Acronyms

AOI 	Area of Interest

EFED	Environmental Fate and Effects Division

LULC 	Land Use / Land Cover

NASS	National Agricultural Statistics Service

NOAA	National Oceanic and Atmospheric Administration

OPP	Office of Pesticide Programs

OSD	Official Soil Series Description

PRZM	Pesticide Root Zone Model

RUSLE	Revised Universal Soil Loss Equation

SOW 	Scope of Work

SRC	Syracuse Research Corporation

TD 	Technical Direction

TR-55	Technical Release Number 55

USDA	United State Department of Agriculture

USEPA	United States Environmental Protection Agency

USGS	United States Geological Survey

# PURPOSE

This report is submitted by Syracuse Research Corporation (SRC) in conjunction with six PRZM scenarios developed in response to a revised draft of Technical Direction No. 4 (TD4) received from Milwina Locklear on 15 September 2006 under GSA Contract No. GS-00F-0019L, Order No. EP06H000149.  The purpose of this report is to summarize the scope of the technical direction, document the overall technical approach used to generate the ornamental outdoor nursery scenarios, document any issues, and provide supplemental information that was important for scenario development, but too detailed to document in the metadata submitted with each scenario.  This document is not intended to be a reproduction of the metadata and all information here is superseded by metadata.  Interested individuals should refer to the metadata for all important information, sources, limitations, and parameter values related to each scenario.

The purpose of TD4 is to develop PRZM scenarios specific for the assessment of risk to aquatic organisms from the application of pesticides to ornamental/outdoor nursery sites.  The scenarios will be used for exposure assessment being conducted by the United State Environmental Protection Agency (USEPA) Office of Pesticide Programs (OPP) Environmental Fate and Effects Division (EFED).

SRC received a list of six scenarios listed in Attachment A of the statement of work (SOW).  The list was based on a spatial analysis of the top outdoor nursery production areas in the conterminous United States.

# TECHNICAL APPROACH

SRC utilized a wide range of guidance documents, technical reports, data bases, and local experts to research the need for the six ornamental outdoor nursery scenarios. If necessary based on background research, SRC developed a parameter file and metadata for the particular use.  Pertinent details regarding scenario parameterization have been included in the metadata with supplemental information provided in later sections of this report.  None of the scenarios listed below were identified as not being required, therefore no justification for excluding scenarios from development was necessary.

## Selection of Scenario Locations

Scenarios for development were identified based on a spatial analysis provided to SRC in attachment B of the Technical Directive.  The analysis identified the top 10 states and top counties within those states with the largest acreage of outdoor ornamental nurseries.  Rankings were computed based on 2002 USDA census of agriculture data for the following outdoor ornamental nursery categories:

- Floriculture crops - bedding/garden plants, cut flowers and cut  florist greens, foliage plants, and potted flowering plants, (total);

- Nursery stock; and

- Other nursery and greenhouse crops

Based on the spatial analysis provided by EPA, six regions were identified for scenario development:

- Oregon/Washington (review the existing Oregon Christmas tree scenario and revise as needed)

- Southern California/Arizona

- South Florida

- Eastern Tennessee/Western North Carolina

- Eastern Pennsylvania/New Jersey

- Northeast Illinois/Southern Michigan

- After geographic regions were identified, the location of the scenarios was refined to specific counties within each of the regions in order to determine whether the use pattern is likely to occur in the areas of interest, and then to select weather, crop, landscape, and soil parameters for the exposure scenarios.  Within each of the broad areas, SRC selected the county identified in EPA’s spatial analysis as having the largest extent of outdoor ornamental nursery acreage in the given region.  Upon preliminarily selecting a representative county for the scenario, SRC proceeded to locate and contact local experts in the field of ornamental horticulture (see Section 4) to determine first if there are commercial and/or wholesale nurseries in the area of interest.  Second, SRC determined if nursery/ornamental operations are predominantly commercial versus wholesale in nature.  The significance of this fact is that commercial nurseries are likely to be small and widely spaced while wholesale nurseries are presumed to be larger in scale and more likely to dominate a given watershed.  Overall, SRC researched these facts, determined the level of significance of the use pattern to the area of interest, and developed the scenario if needed.  In this analysis, all proposed scenarios listed above were deemed necessary, therefore documentation for why a scenario was not needed (including citation of sources used in this determination) was unnecessary.

## Scenario Development Guidance

Ornamental nursery scenarios are atypical scenarios in that they do not represent a homogenous crop as most of EFED’s standard and OP scenarios.  Nurseries are highly diverse in the spatial layout of crop fields as well as the variety of crops grown.  To the extent practical and applicable, SRC followed current USEPA guidance for developing field and orchard crop PRZM scenarios (USEPA 2004) to parameterize the outdoor ornamental nursery scenarios. Default values were used when appropriate and in cases where scenario-specific data could not be obtained. Conceptual models regarding the type of nursery represented by each scenario is provided in the metadata introduction for each scenario.  In cases where it was necessary to model irrigation, current irrigation guidance was utilized (USEPA 2005) noting that irrigation involves many uncertainties; some nurseries have onsite professionals that are devoted to irrigation while others irrigate based on visual inspection of the plant and generalizations regarding irrigation rates are conjecture at best.

## Land Use Data

No land use data were located, at least not at a scale useful for identifying the spatial locations and geographic extent of nurseries in a given region.  Very few land use data sets include classifications that include “nurseries”.  The USGS GIRAS Landuse/Landcover data for the Conterminous United States includes a category for “Orchards, groves, vineyards, nurseries, and ornamental horticultural”, however the data is based on imagery from the 1970’s and 1980’s and based on a preliminary analysis, nurseries didn’t even show up in for the top outdoor nursery county (by acreage) in the US (Warren County, Tennessee).  High quality nursery land use data could have been used to refine soil selections for each scenario.  However, despite the lack of spatial data, representative soils were still able to be selected based on analysis of all soils in the counties and discussion and/or data from local experts in ornamental horticulture.

## Soils Data

As note in Section 2.3 soils were selected without knowing the exact geographic extent of nurseries in each of the given regions due to spatial data quality limitations.  However, as noted above, despite the lack of spatial data representative soils were still able to be selected based on analysis of all soils in the counties and based on discussion and/or data from local experts in ornamental horticulture.

Data for each of the scenario counties (see metadata) were downloaded from Soil Data Mart (USDA 2006).  As requested by EFED, emphasis was placed on evaluating the types of soil expected to be present where outdoor nurseries are present.  The emphasis was placed on identifying the predominant soil type while simultaneously focusing on those soils expected to yield high runoff volumes.  Typically, this consisted of hydrologic groups C or D soils.  When evaluating commercial nursery operations, care was given to determining the relevance on “native” soils as opposed to engineered sites.  Anecdotal evidence suggested that many nursery operations reside on sites where the underlying soil has either been “re-worked” to be more highly compact, or the site has been graded and covered with a less permeable material.  However, discussions with local horticulture experts in the selected nursery counties indicated that most of the nurseries were located on native soils but that the surfaces were sometimes compacted or covered in gravel.  Soils were parameterized based on representative soils that will yield high-end runoff and sediment values.  Specifically, this focused on Hydrological Group C and D soils with high erodibility and slope.  Quantitative descriptions of the soil selection process are provided in the metadata for each scenario with additional detail provided in later sections of this report.  Official soil series descriptions (OSD) of the selected soils were used to characterize the soils of interest for the scenarios (Soil Survey Staff 2006a, b).  Soil parameters were obtained from USDA Soil Data Mart (USDA 2006).

- Two factors guided the selection of a soil series for scenario development.  First, it was determined if the crop and/or use site is likely to occur within the relevant geographic area as described in Attachment A of the SOW.  Second, is the soil type relevant to the use pattern?  Typically, this second question was based on the type of analysis provided in Attachment B of the SOW and Table 5 of the scenario metadata, as well as specific conversations with local soil and agricultural experts as described in the guidance for scenario development provided in previous technical directions.  For the non-agricultural scenarios preferential consideration was given to “benchmark soils” when selecting a representative soil series during this step.  The objective is for the scenario developed to be representative of a high-end exposure location with a target of being a 90th percentile site.  Ranking of the relevant site and soil selection parameters was as quantitative as possible and described fully in the relevant metadata file.  Analysis followed the recent example of this approach for a new scenario provided in Attachment C of the SOW.

- When possible, particular attention was given to developing parameters for the curve number and USLE factors which mimic runoff and erosion losses expected from the use site.  EFED requested that, where possible, the estimates of runoff and sediment yield derived from PRZM for the various scenarios developed be compared against available data from actual runoff studies with a goal of insuring that the PRZM predictions represent a high end sediment and runoff yield from the range of yield typical for the cover type.  For example, well-maintained turf is expected to yield significant volume of runoff water but sediment yields are expected to be low.  Conversations with local experts did not yield any runoff and erosion data for ornamental nurseries, therefore attempts to identify typical ranges of sediment and runoff yields from the relevant use site was impractical.

## Curve Numbers

Due to the generally compacted soil surface conditions and heterogeneity of land cover typical of nurseries, including roads, buildings, grassed areas, and nursery plots, SRC consulted the USDA TR-55 manual for selecting the appropriate curve number for the scenario.

## RUSLE Parameters

For determination of USLEC and Manning’s N values, the RUSLE EPA Pesticide project (2000) was used. Recently, internal investigation by EFED determined an inconsistency in how the currently approved version of PRZM (3.12 beta) is incorporating the RUSLE C Factor into the scenario.  All currently approved scenarios have the RUSLE C Factors tied chronologically to the first day of the year.  However, the correct methodology is to tie the initial C Factor to the date of emergence (Record 11).  SRC used the new guidance for selecting and correlating RUSLE C Factor chronologically with the scenario. To do this, SRC identified the emergence date in the revised scenario after completion of Task 1 above.  SRC then identified the RUSLE date in the scenario that was closest to the emergence date and moved this to the first value in Record 9B.  All subsequent RUSLE dates and C Factors in the were moved to follow the new while maintaining the original chronological order (i.e. RUSLE C factors were still tied to the same date).  Finally, if the emergence date and the new, chronologically first RUSLE date were the same, no additional changes were made.  However, in the case where the new chronologically first RUSLE date and emergence date were not the same, the emergence date was changed to correspond to the new RUSLE date.

## Weather Station Selection

Weather stations were selected based on guidance provided by EFED in previous technical directions.  Most of EFED’s standard and OP scenarios were developed with an associated MLRA. The MLRA was assigned with an eye towards identifying a weather station to link to the scenario.  However, the WQTT agreed that this approach no longer makes sense and that the WQTT should develop criteria by which the contractor, or an internal scenario developer, revises our current scenarios. The criteria states EFED's preference for selecting a weather station.

"The scenario should represent the area most vulnerable to runoff (for surface water scenarios) and/or leaching (for groundwater scenarios) depending on the scenario need for a given crop/geography combination. In general, when a contractor is developing a scenario, this effort at locating a crop scenario will already be completed and the scenario location shall typically be focused on a finite (county level) area. Once the scenario location is selected, the developer shall associate the weather station based on proximity and relevance to the site. Criteria to be used in associating a weather station with a scenario are as follows.

1. Data Availability – Stations were chosen based on a list of stations that recorded all the elements necessary for the current version of EFED's model. In the case of PRZM, these elements include the following daily values:

- date

- precipitation (cm/day)

- pan evaporation (cm/day)

- mean temperature (C)

- wind speed @10 meter (cm/second)

- solar radiation (langleys/day)

- FAO shortgrass Eto (mm/day)

- Daylight Station Pressure (kiloPascal)

- Daylight Relative Humidity (percent)

- Daylight Opaque Sky Cover (tenths of sky covered)

- Daylight Temperature degrees Centigrade

- Daylight Broadband Aerosol (optical depth)

- Daylight Prevailing Wind Speed @10 meter (meter/second)

- Daylight Prevailing wind direction (degrees (N=0, E=90, ...))

All available weather stations compatible with the current EFED model

(PRZM/EXAMS) can be found at http://www.epa.gov/ceampubl/tools/metdata/us_met.htm

2.  Proximity - The weather station selected was generally the closest station to the identified location of the scenario, unless it was determined that a scenario located farther from the scenario had more relevant data (see the discussion below on relevancy).  In the case of the ornamental nursery scenarios the closest station to the scenario county was selected.

3 - Relevance – Relevance is based on the nature, timing and intensity of rainfall events relative to the chosen cropping site. Examples of important factors to consider when evaluating relevancy are similar geography, topography, and landcover, as well as timing, intensity, and seasonality of rainfall & temperature. Selection of an alternative weather station based on the relevance of the data contained within a station farther from the scenario shall be noted within the scenario comments and shall be justified and thoroughly documented within the Metadata file. Other factors such as temperature, evapotranspiration, and wind speed can be used to justify an alternative selection but must be documented. In the case of the ornamental nursery scenarios the closest station to the scenario county was selected.

## Cropping Dates

Cropping dates were selected based on guidance provided by EFED in previous technical directions.  Nurseries are diverse in the number, age, and variety of plants grown.  Therefore, there is considerable uncertainty in the temporally sensitive parameters such as emergence, maturity, and harvest dates.  For the purposes of scenario development, SRC attempted to select cropping dates based on the most predominant nursery crop grown.  Important factors considered were evergreen versus deciduous crops.  EFED provided the following additional guidance for setting cropping dates in the scenarios.  For a crop which is considered “evergreen” or with year round canopy coverage (e.g. citrus) the emergence, maturity and harvest dates should be selected to ensure full year canopy coverage.  For example for citrus (see the CAcitrus scenario) the emergence date should be selected for January 1 while the maturity date can be January 2 and the harvest date December 31 (however see Jan 1 section below).  This ensures year round full canopy coverage for the “evergreen” crop.  For deciduous tree and orchard crops, the emergence, harvest and planting dates should be selected to coincide with the growth cycle of the leaves and/or fruit of the crop.  For example, MIcherry scenario has an emergence date tied to the leaf bloom, while the emergence date is tied to the maturity of the leaf, and the harvest date tied to the harvest of the fruit.

Note on January 1 Emergence Day

Initially SRC was instructed to set the emergence day to January 2.  SRC indicated that setting the emergence date to January 2 would produce a minor conflict with previous guidance for aligning RUSLE C factors and dates with emergence dates.  The previous guidance was to shift the RUSLE date that is closest to the emergence date to the first position in record 9B. The first RUSLE date for the evergreen crops will be 0101. The next step in the guidance is to move the emergence date to the first RUSLE date (i.e., January 1).  EFED stated that January 2 was selected over January 1, because in previous versions of PRZM, setting the emergence date to January 1 resulted in an issue with the file transfer and array in the model.  It’s unclear whether this is still an issue in PRZM.  SRC conducted several PRZM/EXAMS runs, but could not generate an error using Jan 1.  EFED indicated to set the emergence date to Jan 1, as done for PR coffee.  The Agency will revisit this issue during the review of the deliverable.

# IMPORTANT UNCERTAINTIES, LIMITATIONS, AND DATA GAPS

Curve Number: The curve number is the most sensitive parameter in PRZM scenarios.  The curve numbers for these scenarios were based on TR-55 (Table 2-2c).  Unlike homogenous crop fields, nurseries are often heterogeneous and large areas of the nursery field can be covered in roads, wooded areas, field borders, and areas to steep to grow.  Therefore the land use type closest to these scenarios was the value for farmsteads, buildings, lanes, driveways, and surrounding lots.  Runoff and erosion may be under or over-estimated for alternative nursery operations and model validation has high value in this regard.  Based on preliminary investigation, no runoff data are available for validating runoff from outdoor nursery operations in these geographic areas.

Crop Parameters: Nurseries are diverse in the number, age, and variety of plants grown.  Therefore, there is considerable uncertainty in the temporally sensitive parameters such as emergence, maturity, and harvest dates, as well as spatial parameters such as canopy cover and canopy interception.  For the purposes of scenario development, SRC attempted to select cropping parameters based on the most predominant nursery crop grown.

RUSLE factors: The RUSLE Project does not include data for ornamental nursery crops.  Therefore, USLE C Factor and Manning’s N values were generally selected from orchards with representative cover, due to the similarities between conditions.  In some cases, Halcomb (2002) suggests that up to 40% of nursery fields can be covered in roads, wooded areas, field borders, and areas to steep to grow, leaving 60% available for production. Roads, travel lanes, waterways, etc. should be covered in fescue (Halcomb, 2002) or other permanent ground cover/cover crop adhering to Nursery Crop Best Management Practices for field nurseries.

# CONTACTS

Adkins, Craig

Area Specialized Agent , North Carolina Cooperative Extension Service

120 Hospital Avenue, NE

Lenoir, NC  28645

Phone:  828-757-1290

Fax: 828-757-1251

Email: Craig_Adkins@ncsu.edu

Bethke, Jame

UC Cooperative Extension, Farm Advisor-Nurseries and Floriculture

334 Via Vera Cruz, Suite 152

San Marcos, CA 92078

Phone: 760-752-4715

Fax: 760-752-4725

Email:  bethke@ucr.edu

Thomas, Dudek

Cooperative Extension

Horticulture and Marketing Agent

333 Clinton Street

Grand Haven, MI  49417

Phone: 616-846-8250

Email: dudek@msu.edu

Guiser, Scott

Senior Extension Educator (horticulture)
Penn State Coop. Ext.

1282 Almshouse Rd

Doylestown, PA 18901

Phone: 215-345-3283

Fax: 215-345-1653

Email sxg6@psu.edu

Johnson, James

Rutgers Cooperative Research and Extension, County Agricultural Agent (Ornamental Horticulture)

291 Morton Avenue

Millville, NJ 08332

Phone: 856-451-2800

Fax: 856-451-4206

Email:  jjohnson@rcre.rutgers.edu

Mikel, Terry

Natural Res. Agent (horticulture)
Arizona Coop. Ext.

4341 E. Broadway Rd

Phoenix, AZ 85040

Phone: 602-470-8086

Fax: 602-470-8086

Email: tmikel@cals.arizona.edu

Regan, Richard

District Extension Agent, Oregon State University,

15210 NE Miley Road

Aurora, OR  97002

Phone:  (503) 678-1264  x122

Fax:   (503) 678-5986

Email: rich.regan@oregonstate.edu

Ed Weskerna

SWCD District Manager

1648 S. Eastwood Dr.

Woodstock, IL 60098

Phone: 815-338-0099 x3

Fax: 815-338-7731

Email: ed.weskerna@mchenryswcd.org

Wilen, Cheryl PhD

UC Cooperative Extension
IPM Advisor for Ornamentals & Nurseries

5555 Overland Ave Bldg 4

San Diego, CA  92123

Phone: 858-694-2846

# REFERENCES

Halcomb, M. 2002.  Nursery Field Production. University of Tennessee Agricultural extension Service.  Online at: www.utextension.utk.edu/hbin/Field Prod rev w equip.pdf

NOAA. 2006.  National Weather Service Forecast Office.  Climatological data for Austin, Texas. data. http://www.weather.gov/climate/xmacis.php?wfo=ewx

USDA Soil Conservation Service. 1986. Urban Hydrology for Small Watersheds. Technical Release 55, 2nd edition, June 1986.

USDA.  1997.  1997 Census of Agriculture. U.S. Department of Agriculture, National Agricultural Statistics Service.  Accessed March 2006.  Online at: http://www.nass.usda.gov/index.asp.

USDA.  2002.  2002 Census of Agriculture. U.S. Department of Agriculture, National Agricultural Statistics Service.  Accessed March 2006.  Online at: http://www.nass.usda.gov/index.asp.

USDA. 2000.  Knisel, W.G., and Davis, F.M., 2000, GLEAMS: Groundwater Loading Effects of Agricultural Management Systems, Version 3.0: Agricultural Research Service, U.S. Department of Agriculture, Publication No. SEWRL-WGK/FMD-050199, 191 p.

USEPA. 1998.   Carsel, R.F., J.C. Imhoff, P.R. Hummel, J.M. Cheplick, and A.S. Donigian, Jr.  PRZM-3, A Model for Predicting Pesticide and Nitrogen Fate in the Crop Root and Unsaturated Soil Zones: Users Manual for Release 3.0.  National Exposure Research Laboratory, Office of Research and Development, U.S. Environmental Protection Agency, Athens, GA.

USEPA. 2004.  Pesticide Root Zone Model (PRZM) Field and Orchard Crop Scenarios: Guidance for Selecting Field Crop and Orchard Scenario Input Parameters.  November 15, 2001; Revisions July 2004.

USEPA. 2005. Irrigation Guidance for developing PRZM Scenario. June 15, 2005.

# Appendix A. Soil Maps and Major Land Resource Areas

Soil hydrologic groups Cumberland County New Jersey and MLRAs.

Soil hydrologic groups Clackamas County Oregon and MLRAs.

Soil hydrologic groups Miami-Dade County Florida and MLRAs.

Soil hydrologic groups Ottawa County Michigan and MLRAs.

Soil hydrologic groups San Diego County California and MLRAs.

Soil hydrologic groups Warren County Tennessee and MLRAs.

# Appendix B. Geospatial Analysis Completed by SERA

PRZM Scenario Selection

This document is intended to provide a summary of analysis performed to facilitate PRZM scenario selection for outdoor nursery operations.  The overall approach involved obtaining NASS data from USDA to determine the most likely potential outdoor nursery areas in the US and determining the location and representativeness of PRZM scenarios and meteorological stations with respect to high potential pesticide usage areas.

To determine where the most likely application of pesticides to outdoor nurseries may occur, an analysis of nursery outdoor acreage was performed.  Total 2002 nursery acreage in the open was obtained from the USDA National Agricultural Statistics Survey, 2002 Census of Agriculture (Table 34).  State and county level data for outside nursery acreage were extracted from Table 34 which includes: Nursery, Greenhouse, Floriculture, Mushrooms, Sod, and Vegetable Seeds Grown for Sale in 2002 and 1997.  Three subcategories in this group were summarized to generate total outdoor acreage:

- Floriculture crops - bedding/garden plants, cut flowers and cut  florist greens, foliage plants, and potted flowering plants, (total);

- Nursery stock; and

- Other nursery and greenhouse crops

State level data were summarized across the three categories listed above to obtain total acreage of outdoor nurseries.  States were then ranked by total outdoor acreage to determine the top 10 nursery states.  Figure 1 shows the state level distribution of nurseries in the conterminous US.  County level data was then summarized in a similar manner for the top 10 states to determine the top nursery county in each of the top 10 states.  Figure 2 displays county level totals in addition to the top nursery county (by outside acreage) within each of the top 10 nursery states.

The Oregon Christmas tree scenario has been recommended by EFED as a representative scenario for Tier II aquatic exposure modeling on nursery site uses.  To characterize the potential national scale variability in pesticide exposure in high potential use areas (Figure 2), EFED may wish to use alternate PRZM meteorological stations in conjunction with the Christmas tree scenario.  Locations of the geographically closest met station to the top nursery counties in the top 10 states have also been identified and are displayed in Figure 2.  It should be noted that this approach has some limitations and/or uncertainties.  For example, the Oregon Christmas tree scenario is based on the Pilchuck soil series, which is unlikely to be located through the identified nursery areas (Figure 2).  However, the soil series has been selected to be “conservative” and may be sufficient for screen purposes.  An alternative approach may be to utilize soils in the existing standard and organophosphate PRZM scenarios or to use alternative, yet similar scenarios altogether (e.g., orchard scenarios).  The approximate locations of each PRZM scenario in relation to top outdoor nursery states are provided in Figure 3.

- Development of these scenarios should focus on curve numbers identified in Table 2-2a through d from the USDA Publication Urban Hydrology for Small Watersheds, Technical Release 55 (TR-55) dated  June 1986.  Unlike other cover types there is no clear category from the TR-55 for ornamental/nursery settings.  The contractor shall evaluate all the settings and provide a rationale for selection.  The contractor shall research (e.g. published data, trade association information, conversations with local county extension agents) whether commercial and/or wholesale nurseries are likely present in the areas defined in Attachment A.  If it is determined that nurseries are present but are dominated by commercial activities, the contractor shall determine if local regulations preclude runoff from these use sites (via NPDES permits or some other regulatory enforcement).  If so, the contractor may conclude that these uses are not likely to influence exposure and no scenario is needed.  However, if the contractor is unable to definitively determine these facts, then a scenario shall be developed using the best approximation of typical nursery practices in this area with particular attention paid to the type of ornamental products being developed in the area of interest.

- EFED’s preliminary analysis suggests that  for ornamental/nurseries that a cover type such as woods-grass combination (Table 2-2c) or herbaceous (Table 2-2d) may be considered, but does not preclude other types or hybrids.  EFED geospatial analysis indicates that five to six distinct geographic areas my need individual scenarios.  As with all scenarios, these ornamental/outdoor nursery scenarios should focus on the most vulnerable sites typically with an emphasis on Hydrologic soil group C and D soils.  If information suggests that most commercial outdoor nurseries in these areas are defined by an artificial surface which is more impermeable than native soils, then this fact should be accounted for when parameterizing the scenarios.  EFED currently relies on the existing ORxmastree scenario as a surrogate for ornamental/nursery operations.  These scenarios should be developed similarly to the ORxmastree scenario, but the contractor should research nursery operations for a typical cover type that best approximates this use site.

- Conceptually, this scenario assumes that the entire 10 hectare treated area will drain into the receiving water body.

Figure 1. Outdoor Nursery Acreage in the Conterminous US.

Figure 2. County Level Outdoor Nursery Acreage, Top 10 Nursery States, and Top Nursery Counties, including nearest PRZM Meteorological File.

Figure 3. Outdoor Nursery Acreage and Available Standard and Organophosphate PRZM Scenarios.

# Appendix C. Aerial Photography of Ornamental Nurseries.

Example of a Wholesale Nursery In Southern Florida.

Example of a Wholesale Nursery in Southern Florida.

Examples of  wholesale nurseries in San Diego County, southern California.

Example of a wholesale nursery in southern New Jersey

Example of a wholesale nursery in southern New Jersey