DELMARVA PENINSULA (SWEET CORN) GROUNDWATER

12/23/13:

- Updated Table 3b to reflect correct maximum water capacity (half way between Porosity and Field Capacity) and revised table title.

- Updated Table 2 to reflect crop height of 300 cm to be consistent with standard PRZM surface water scenarios.

- Updated allowable depletion of available water (fraction) from 0.33 to 0.84 in Table 2.

Metadata

Sweet corn is grown throughout the Delmarva Peninsula. It is planted in small blocks, usually less than 5 acres, in successive plantings to provide a continuous supply of corn. In contrast, the processing acreage is planted in larger rotated fields, usually greater than 30 acres and ranging up to 120 acres.  In the southern areas, sweet corn is often double cropped with crops such as soybeans or cucumbers. The highest cucumber production area in Maryland is along the Eastern Shore (Delmarva Peninsula), with the highest production in Wicomico County.

Sweet corn grows in many types of soils; however light, sandy soils with a pH level of 6.5 are optimal. Cucumbers grow best on light-textured (sandy), well-drained soils high in organic matter with a pH between 6 and 6.5.

Sweet corn requires a continuous supply of water. Irrigation during silking, tasseling, and ear development is used by nearly all Delmarva processors and some fresh market growers.  Per the USDA crop profile, fields should be irrigated if rain does not occur for more than 2 weeks during the early stages of growth.  Irrigation becomes critical as corn begins to tassel. About 1 inch of water per week is needed from tasseling through harvest.

The USDA crop profile for cucumbers indicate that 50% of the cucumber crop in Maryland is grown on black plastic with drip irrigation, with the remainder grown on bare ground.

Sweet corn is generally seeded one inch deep with row spacing of 30 inches and plants separated by 8 to 12 inches within rows. Cover crops, such as rye or wheat, are typically used prior to seeding sweet corn, to reduce soil erosion and improve soil tilth and fertility. No-till is used on less than 20% of fresh market sweet corn acres and about 10% of processed corn. Sweet corn may be planted as early as the last week of March in the southern portions of Delmarva, with successive plantings into early July in the remaining regions. Fresh market sweet corn is harvested 8-21 days after silking, whereas processing corn is harvested 5-7 days later when the ears are more mature (http://www.ipmcenters.org/cropprofiles/docs/DEsweetcorn.pdf).

The pH of the shallow aquifer at 3 to 20 feet below surface is typically in the range of 5 to 6 (Blaier and Baxter, 2000).

| Table 1. PRZM 3.12 Climate and Time Parameters for the DELMARVA Peninsula |  |  |
| --- | --- | --- |
| Parameter | Value | Source |
| Starting Date | January 1, 1961 | Meteorological File – Wilmington, DE (13781) |
| Ending Date | December 31, 1990 | Meteorological File – Wilmington, DE (13781) |
| Pan Evaporation Factor (PFAC) | 0.77 | PRZM 3 Manual, Figure 5.9 (Suarez, 2006) |
| Snowmelt Factor (SFAC) | 0.36 | PRZM 3 Manual, Table 5.1 (Suarez, 2006) |
| Minimum Depth of Evaporation (ANETD) | 17 | PRZM Manual Figure 5.2 (Suarez, 2006). Use the mid-point of the range of values based on location of the crop scenario. If a crop region crosses one or more boundaries, select the average of the midpoints. |

| Table 2. PRZM 3.12 Crop Parameters for the DELMARVA – Sweet Corn |  |  |
| --- | --- | --- |
| Parameter | Value | Source |
| Initial Crop (INICRP) | 1 | The simulation date starts before the emergence date of the crop. PRZM is currently untested for other conditions. |
| Initial Surface Condition (ISCOND) | 1 | The effect of this parameter is nearly irrelevant in EFED standard scenarios. This parameter specifies the curve number in place before the main crop is planted. |
| Number of Different Crops (NDC) | 1 |  |
| Maximum interception of Storage of Crop (CINTCP) | 0.27 | PRZM 3 Manual (Suarez, 2006) Table 5.4 |
| Maximum Rooting Depth (AMXDR) | 100 | Selection of root depth by professional judgment will be a compromise between a need to accurately describe evapotranspiration and accurately describing irrigation needs. Use USDA crop profiles. |
| Maximum Aerial Canopy Coverage (COVMAX) | 98 | -- |
| Surface Condition of Crop after Harvest | 1 | PRZM 3 manual (Suarez, 2006) |
| Curve Number (CN) | 10 | Typically for groundwater scenarios, curve numbers will be low and have little effect on simulated results (e.g., from A or B soils). |
| Max Dry Weight of Crop at Full Canopy (WFMAX) | 0 kg/m2 | Not Used in GW scenarios – place holder |
| Canopy Height (HTMAX) | 300 | Consistent with standard corn surface water scenarios. |
| Number of Cropping Periods (NCPDS) | 30 | Set to weather data. |
| Date of Crop Emergence (EMD/EMM/IYREM) | 10/04 | http://www.ipmcenters.org/CropProfiles/docs/DEsweetcorn.pdf |
| Date of Crop Maturation (MAD/MAM/IYRMAT) | 30/05 | http://www.ipmcenters.org/CropProfiles/docs/DEsweetcorn.pdf |
| Date of Crop Harvest (HAD/HAM/IYRHAR) | 15/06 | http://www.ipmcenters.org/CropProfiles/docs/DEsweetcorn.pdf |
| Crop Number Associated with NDC | 1 | Only one crop modeled |
| IPSCND | 1 | Conservatively assume bulk of material is incorporated into ground |
| Extra Water for Leaching | 0.1 | Typical |
| Available Depletion | 0.84 | Value calculated based on updated maximum water. Calculations are included in the attached excel file “Scenario Update Calculations Dec 2013”). Previous value determined by back calculation of ET demand based on typical practices. |
| Max Rate of Water Supplied | 0.3 | Set high enough such that demand would be met on a single day. |

| Table 3a. PRZM 3.12 Evesboro Loamy Sand Soil Parameters for the DELMARVA Peninsula, Delaware |  |  |
| --- | --- | --- |
| Parameter | Value | Source |
| Soil Property Title (STITLE) | Evesboro loamy sand | USDA Soil Data Mart |
| Total Soil Depth (CORED) | 600 cm | Standard GW Scenarios are for 10-meter soil profiles, with the last meter simulated as an aquifer. |
| Number of Horizons (NHORIZ) | 8* | For the upper Horizons use USDA Soil Data Mart (Soil Survey Staff, 2008). Resolution need not be less than 1 cm in the top portion of the profile and not less than 20 cm in the remaining profile. The top profile is resolved into 1 cm increments in order to allow for accurate applications of pesticides into the soil surface. Below 10 cm, discretization is increased to 20 cm in order to simulate realistic dispersion. |

* Using 8 horizons greatly simplifies the procedure for entering declining degradation rates.  The first 2 horizons are 10 cm, and the next four are 20 cm thickness; these 6 horizons represent the aerobic degradation zone. The 7th horizon is variable depending on the depth to the simulated aquifer and represents the depths between the aerobic degradation zone and the water table. The 8th horizon represents the groundwater which starts at a variable depth depending on the parameterization, with the profile extending 100 cm below the water table.   Note that the pore water degradation rate should never be less than the hydrolysis rate.

| Table 3b. PRZM 3.12.2 Evesboro Loamy Sand Soil Horizon Input and Field Parameters for the DELMARVA Peninsula, Delaware |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Horizon # | Horizon Thickness (THKNS) (cm) | Bulk Density (BD) (g/cm3) | Initial Soil Water Content (THETO)* | Soil Drainage Parameter (AD) | Pesticide Hydrodynamic Solute Dispersion Coefficient (DISP) | Lateral Soil Drainage Parameter (ADL) | Thickness of Compartments in Horizon (DPN) (cm) | Field Capacity (THEFC) (cm3/cm3)* | Maximum Water Capacity1 | Wilting Point (THEWP) (cm3/cm3) | Organic Carbon (OC) (%) | Sand (%) | Clay (%) |
| 1 | 10 | 1.56 | 0.095 | 0 | 0 | 0 | 1 | 0.095 | 0.25 | 0.022 | 0.52 | 92.3 | 2.4 |
| 2 | 10 | 1.56 | 0.095 | 0 | 0 | 0 | 5 | 0.095 | 0.25 | 0.022 | 0.52 | 92.3 | 2.4 |
| 3 | 20 | 1.561 | 0.095 | 0 | 0 | 0 | 20 | 0.095 | 0.25 | 0.017 | 0.20 | 91.5 | 2.7 |
| 4 | 20 | 1.62 | 0.058 | 0 | 0 | 0 | 20 | 0.058 | 0.22 | 0.042 | 0.18 | 89.7 | 5.9 |
| 5 | 20 | 1.66 | 0.080 | 0 | 0 | 0 | 20 | 0.080 | 0.23 | 0.038 | 0.17 | 89.6 | 5.2 |
| 6 | 20 | 1.68 | 0.060 | 0 | 0 | 0 | 20 | 0.060 | 0.21 | 0.024 | 0.13 | 95.0 | 3.3 |
| 7 | 400 | 1.63 | 0.059 | 0 | 0 | 0 | 50 | 0.059 | 0.22 | 0.018 | 0.13 | 96.0 | 2.3 |
| 8 | 100 | 1.71 | -- | 0 | 0 | 0 | 50 | -- | -- | -- | 0.14 | 86.5 | 9.5 |
| NRCS, National Soils Characterization Database (NRCS, 2001) http://soils.usda.gov/survey/nscd/ USDA Soil Data Mart Draft Model and Scenario Development for Groundwater Estimates Using PRZM 1 Assumed equal to value above. *THETO set to mid-point between field capacity and saturation. 1. 1-[bulk density/2.65 (i.e., grain density)] = porosity; average field capacity and porosity |  |  |  |  |  |  |  |  |  |  |  |  |  |

GW temperature: 11ºC (PRZM Manual Figure 5.4 Suarez, 2006, also: http://0.tqn.com/d/homerepair/1/0/o/3/-/-/climate_temps.jpg)

Albedo: 0.2

REFERENCES:

Blaier, S.C., and Baxter, S.J. 2000. The occurrence and distribution of several agricultural pesticides in Delaware’s shallow ground water: Delaware Geological Survey Report of Investigations No. 61, 23 p.

Suarez, L.A., 2006. PRZM-3, A Model for Predicting Pesticide and Nitrogen Fate in the Crop Root and Unsaturated Soil Zones: Users Manual for Release 3.12.2.   EPA/600/R-05/111 September 2006, revision a.